# TRIBHUVAN UNIVERSITY INSTITUTE OF ENGINEERING PASHCHIMANCHAL CAMPUS DEPARTMENT OF GEOMATICS ENGINEERING LAMACHAUR-16, POKHARA # A REPORT ON SURVEY CAMP 2080 Submitted by:- Submitted to:- Abishek Adhikari (078BGE004) Department of Geomatics Engineering Group ‘3’. Bharat Kumar Yadav (078BGE010) Kiran Guragai (078BGE020) Nishan Gautam (078BGE029) Sandesh Gaire (078BGE039) Saugat Thapa (078BGE042)

ABSTRACT

Surveying  is the process of determining the relative position of point on, above or under the surface of the earth, and the most important part of Geomatics Engineering.The result of surveys are used to map the earth, prepare navigational charts,establish property boundaries and many more.Further survey maintains highways, railroads, building, bridge, tunnels, canals, dam and many more. Thus, as a geomatics engineers field survey is the most important aspect.

The main objective of the survey camp for the geomatics engineering student is to provide an opportunity to consolidate and update their practical and theoretical knowledge of surveying in the actual field and to build up confidence to conduct engineering survey on required accuracy. In the survey camp we have to prepare topographical map of given area by using Total Station(T.S),topographical map of road alignment and bridge site  including plan,L-section and X-section. In this regards  we were required to carryout the necessary field works in our sub-groups so that we will get ample opportunity to the  decision on planning and execution of  the field works.

The summary of the conduction of whole report is pressed below:

Project Title: Survey Camp 2080

Location:  Pashchimanchal Campus premise and Kali Khola area

Duration: Chaitra 20th to 29th (10 days)

Working Time: 07:00 am to 5:00 pm

Surveyed By:   GROUP 3

(078BGE[004,010,020,029,039,042])

Working Schedule:

| S.N | Day | Survey Field Work |
| --- | --- | --- |
| 1 | 20th Chaitra | Selection of an alignment for the road and detail survey of it. |
| 2 | 21st | Level transfer on IP,BC,MC,EC of the road |
| 3 | 22nd | Level transfer on IP,BC,MC,EC of the road |
| 4 | 23rd | Detail survey of selected alignment and Bridge site survey and reciprocal leveling . |
| 5 | 24th | Bridge site survey and road alignment survey completion |
| 6 | 25th | Reconnaissance,station fixing, distance and angular measurement and bearing observation for major traverse. |
| 7 | 26th | Station Fixing ,distance and angular measurement and bearing observation for major and minor traverse.Two peg test,fly leveling and Rl transfer to the stations. |
| 8 | 27th | Traverse Calculation,Marking grid and computation Detailing |
| 9 | 28th | Detailing by Total Station and theodolite |
| 10 | 29th | Completion of Remaining Task and Completion of survey Camp 2080 |

ACKNOWLEDGEMENT

First and foremost, we would like to express our gratitude to Department of Geomatics Engineering , Pashchimanchal Campus, Pokhara for providing an opportunity to consolidate our theoretical and practical knowledge in engineering surveying. And we are deeply grateful to management committee of our Campus for providing us with all the required equipment for our Camp.

We would like to express our sincere gratitude to Camp coordinator       Er. Pardip Aryal  . For his helpful suggestion and valuable feedback thorough out the field work without which it was very difficult to do the work in the field and produce a report. We are equally indebted to our respected teachers camp instructor, Er. Netra Bahadur Katuwal, Dr.Niraj K.C,  Er.Saurav Gautam and Er.Umesh Bhurtyal   for constant guidance and continual encouragement and also providing with the prompt comment and rectification necessary before producing the final report.

At last I would like to express my gratitude to all those who walked with us throughout our Camp journey – your support was our strength, and this achievement is as much yours as it is our.This initiative would not have been a success without the contributions of each and every individual. We were always there to cheer each other on, and that is what kept us together until the end.

GROUP NO:3

Abishek Adhikari (078BGE004)

Bharat Kumar Yadav (078BGE010)

Kiran Guragai (078BGE020)

Nishan Gautam (078BGE029)

Sandesh Gaire (078BGE039)

Saugat Thapa (078BGE042)

ABBREVIATION

B.C      :  Beginning Of Curves

BL       :  Bed Level

B.M    :   BenchMark

B.S       :  Backsight

C.P    :  Control Points

E.C       :  End of Curves

F.S       :   Foresight

H.A       :  Horizontal Angle

H.C.R       :  Horizontal Circle Reading

HFL        :  High Flood Level

H.I       :  Height of Instruments

I.P       :   Intersection Point

M.O      : Mid Ordinates

NF       :  Normal Flow

R.L      :  Reduce Level

T.B.M      :  Temporary BenchMark

T.P     :  Turning Point

V.C.R    :  Vertical Circle Reading

Detaling Co-ordiante

LB : Library

TR : Tree

R   : Road

CT : Canteen

PF  : Parafit

Table of contents

ABSTRACT

ACKNOWLEDGEMENT

ABBREVIATION

Salient Feature Of The Survey Camp	1

INTRODUCTION	4

Background	4

Primary division of surveying:	4

Classification of surveying:	5

Principle of surveying:	6

Accuracy and Errors:	7

Sources of error:	7

TOPOGRAPHICAL SURVEY	9

Introduction	9

Objectives:	9

Brief description of the area:	9

Norms (Technical specifications):	11

Equipment used:	12

Methodology:	12

Reconnaissance(recci):	13

Traversing:	14

Detailing:	18

Levelling:	19

Computations and plotting:	22

Result and calculation	23

Comments and Conclusions:	27

ROAD ALIGNMENT	28

Introduction:	28

Objectives of road alignment survey:	29

Brief Description of the project area:	30

Norms (Technical Specifications):	30

Equipment & Accessories:	31

Methodology:	31

Reconnaissance:	31

Horizontal Alignment:	32

Vertical Alignment:	33

Leveling:	33

L-section & Cross Section:	33

Curve Setting:	34

Result and calculation	36

Comments and Conclusion:	37

BRIDGE SITE SURVEY	38

Introduction	38

Objectives: 	38

Brief description of the site:	38

Technical Specification(Norms)	39

Equipment & Accessories:	40

Methodology:	40

Site Selection:	40

Tacheometry:	41

Topographic survey:	41

L-Section & Cross Section:	41

Leveling:	42

Detailing:	43

Computation & Plotting:	43

Result and calculation	44

Comments and Conclusions:	45

BIBLIOGRAPHY:	46

ANNEX-A	47

Field Book and Calculation	47

TOPOGRAPHICAL SURVEY	48

ROAD SURVEY	76

BRIDGE SITE SURVEY	86

LIST OF PLOTTING AND FIGURE

- Cross -section of Bridge

- Cross -section of Road

- Topographic map of WRC in color print

- Topographic map of WRC in chart paper

- Topographic map of WRC in tracing paper

- Profile of Road

- Proposed Plan of Road

Salient Feature Of The Survey Camp

Name of the project:

Detail Survey,Design and complete Report of Survey Camp 2080.

Location

1.Region: Lamachaur, Pokhara (metropolitan city)

2.Zone: Gandaki

3.District: Kaski

Description

Surveying is an art of determining the relative positions of point on above or beneath the surface of the earth by means of angular and linear measurements. The result of surveys are used to map the earth, prepare navigational charts,establish property boundaries and many more.Further survey maintains highways, railroads, building, bridge, tunnels, canals, dam and many more.

The B.G.E. Survey Camp 2080, organized by the Geomatics Department , I.O.E, Pashchimanchal Campus is a part of the four-year Bachelor's degree in Geomatics Engineering course, second year second semester, carrying a total of 100 marks. The total duration of the survey camp was 10 days, from 20th of Chaitra to 29th Chaitra, 2080.

This is a detailed report of the works performed by Group 3, during the camp period. It briefly explains the working procedures and technique along with the observations, calculations, and methods of adjustment of error. In addition, it also contain the main problem faced during work and their solution, results of all calculations.

The work done during the camp duration can be categorized into:

1.Topographical survey

2. Bridge site survey

3. Road alignment survey

Sites:

1.For topographic survey of area-Pashchimanchal Campus

2.Bridge and Road alignment-Kali Stream(Kali Khola)

Objectives of survey camp:

- The main objectives of the survey camp are as follows: 

- To Become familiar with the Surveying problems that are arise during the field work.

- Linear segment survey practice through Road Alignment Survey.

- To become familiar with the spirit and importance of teamwork , as surveying is not a single person work.

- Horizontal control and vertical control survey practices with respect to National grid system and produces topographic map in coordinate system.

- To tackle the mistakes and incomplete data from the field work in office work.

Geographical features

I.Terrain: Hilly

ii.Climate: Mild Temperature

iii.Geology: Project area follows geomorphic form of Mahabharata range of Kaski district of Western Nepal

Description of work:

1.Topographic Survey (Traversing + Detailing):

Traversing:

i.No.of major traverse stations:7

ii.No. of minor traverse stations: 5

Detailing:

All the area enclosed by the traverse was  detailed. It included both natural and man-made features like buildings, parks, trees, spot heights, etc.

2. Road Alignment:

i.Starting point of the road: a point (say IPo) near the given PBM.

ii.Length of the road: 1003.87m

iii.Cross-section: 6m left and 6m right on both sides from the center line

3. Bridge Site Survey:

i.Bridge span:34.048 m

ii.Surveyed area for topography: 150m upstream and 100m downstream

iii.Cross-section: up to 150m upstream and 100m downstream

Camping schedule:

Survey Camp is scheduled for 10 days starting from orientation on 2080/12/20 and ending on 2080/12/29. Students are requested to reach the site on time early in the morning at 6:30 AM and daily field work starts at 7:00 AM and ends at 5:00 PM.

INTRODUCTION

Background

Surveying is the process of determining the relative position of natural and man-made features on or under the earth's surface, the presentation of this information either graphically in the form of plans or numerically in the form of tables and the setting out of measurements on the earth's surface. It usually involves measurements, calculations, the production of plans, and the determination of specific locations.

Surveyors work with elements of geometry, trigonometry, regression analysis, physics,engineering, metrology, programming languages, and the law. They use equipment, such as total stations, robotic total stations, GPS receivers, retro reflectors, 3D scanners, radios, hand held tablets,digital levels, subsurface locators, drones, GIS, and surveying software.

Surveying has been an element in the development of the human environment since the beginning of recorded history. The planning and execution of most forms of construction require it. It is also used in transport, communications, mapping, and the definition of legal boundaries for landownership.It is an important tool for research in many other scientific disciplines .

Primary division of surveying:

As to whether the surveyor must regard the earth surface as curved or may regard its as plane depends upon the character and magnitude of the survey, and upon the precision required. Primarily,surveying can be divided into plane and geodetic. In plane surveying, mean surface of the earth is considered as a plane and spheroidal shape is neglected,all triangle formed are considered as plane triangles, level line is considered as straight and plumb line are considered parallel. It is reasonable for the area involving less than 250 sq. km. Since length of an arc 12km. long lying on the earth surface is only 1 cm greater than the subs tended chord.And the difference between the sum of angles in a plane triangle and sum of those in spherical triangles only 1 second for a triangle at the earth surface having area of 195 sq. km.

In geodetic surveying, the shape of the earth is taken into account. All lines are curved line, all triangle are spherical triangle so it involves spherical trigonometry. The object of geodetic survey is to determine the precise position on the surface of the earth, of a system of widely distant point which forms the control station to which survey of less precision may be referred.

Keeping in view the above fact in our survey camp we conduct plane surveying since the area to be surveyed is small and precision required is within the limit as that obtained by plane surveying.

Classification of surveying:

Survey may be classified on the different heading depending upon the uses or purposes of resulting map.

Based on nature of field

- Land Survey: includes topographical, cadastral and city survey.

- Hydrographic Survey

- Astronomical Survey

Based on object of survey

- Engineering Survey

- Military Survey

- Mine Survey

- Geological Survey

- Archaeological Survey

Based on instruments used

- Chain survey

- Theodolite survey

- Traverse survey

- Triangulation survey

- Tachometric survey

- Plane Table Survey

- Photogrammetric survey

- Aerial Survey

In our survey camp, the type of survey that we performed is engineering survey which includes the preparation of topographic map, in which both horizontal and vertical controls are necessary.As per instrument used we perform theodolite traverse survey for fixing control points,tachometric survey for detailing with the alliance of total station and triangulation survey for establishing control points in bridge site survey.

Principle of surveying:

The fundamental principles of plane surveying are:

- Working from whole to part:

It is very essential to establish first a system of control points with higher precision.Minor control pointscan then be established by less precise method and details can then be located using minorcontrol points by running minor traverse. This principle is applied to prevent the accumulationof error and to control and localize minor error.

- Location of point by measurement from two points of reference:

The relative position of points to be surveyed should be located by measurement from at leasttwo (preferably three) points of reference, the position of which have already been fixed.

- Consistency in work:

The survey work should be performed by keeping consistency in method,instrument,observer etc. to get desired level of accuracy.

- Independent Check:

An independent check should be applied on data when possible. For e.g. measuring all threeangles of triangle, even though third angle measurement is redundant.

- Accuracy required:

Proper method and proper instrument should be used depending upon amount of accuracyrequired. Accuracy of angular and linear values should be compatible.

In our survey camp,survey work is performed by considering the above fundamental principle of surveying.

Accuracy and Errors:

Precision:

Precision is the degree of perfection of measuring instruments, the methods and the observations.It is the degree to which the repeated observations under same condition shows the same result.

Accuracy:

Accuracy is the degree of perfection obtained as a result from observation. It is the degree of closeness of observation near to true value. Accuracy depends upon precise instruments, precise methods and good planning.

A discrepancy is the difference between two measured values of the same quantity, it is not an error.

Sources of error:

Error may arise from three sources:

- Instrumental errors: are those arising due to imperfection or faulty adjustment of the instrument with which measurement is being taken. E.g., a tape too short

- Personal errors: are those arising due to want of perfection of human sight in observing and of touch in manipulating instrument. E.g., error in taking level reading.

- Natural errors: are error due to variation in natural phenomenon such as temperature,refraction, magnetic declination etc.

Type of error:

Error may be classified as:

Mistakes: are errors arising from inattention, inexperience,carelessness and confusion in the mind of observer. If undetected, it produces a serious effect. Hence, every measurement to bear corded in the field must be checked by independent check.

Systematic error: are error that under the same condition will always be of same size and sign,a correction can be determined and applied, these make the result too great or too small accordingly treated as positive or negative error.

Accidental error: are those which remain after mistake and systematic error have been eliminated and caused by a combination of reason beyond the ability of observer to control.They tend some times in one direction and sometimes in other. Accidental error represented the limit of precision in the determination of value.

Permissible error: It is the maximum allowable limit that a measurement may vary from the true value or from a value previously adopted is correct. Its magnitude in any given case depends upon the scale, purpose of the survey, the instrument available, class of the work etc.The limit of error cannot be given once for all. The best surveyor is not he, who is extremely accurate in all his work, but he who does it just accurately enough for the purpose without waste of time & money.

In our survey camp, all the computations were made within the permissible error limit.

TOPOGRAPHICAL SURVEY

Introduction

Topographical surveying is the process of determining the positions of natural and artificial features of the locality by means of conventional signs up on a topographical map. Topographic surveys are three-dimensional;they provide the techniques of plane surveying and other special techniques to establish both horizontal and vertical control.Topographic is simply the graphical representation of positions of the earth's surface. Hence, the fieldwork in a topographical surveying consists of three parts.

- To establishes both horizontal and vertical control.

- To locates the contours.

- To locates the details such as rivers, streams, lakes,roads, houses,trees etc.

Objectives:

The main objective is to prepare the topographic map of the given area with horizontal control and vertical control with required accuracy. By topographic survey we can determine the position of both on plan and elevation, of any features of a locality for the purpose of delineating them by means of conventional sign and symbol upon the topographic map.

Brief description of the area:

The area, where topographic surveying was Performed, is situated at Pashchimanchal Campus, Pokhara. The major traverse was run throughout the campus area, which cover the half area of the campus. Our objective was to prepare a topographic map of the given small area, which is a part of the campus area. So,we were assigned to prepare the topographic map of the area including Library,Electronics and Computer Department, Administration Building, Workshop building, Applied Block , Geomatics Block, Civil Engineering Laboratory, Shahid Abinash park,BP park,BEST park,Civil Block, Saraswati Temple, Fountain, Water tanks, etc. that includes the entire natural and man-made features that may come in the general survey work.

Location and Accessibility:

Pashchimanchal Campus is situated in Pokhara-16, Lamachaur which lies in the north-central part of the Pokhara valley.  The detail of the area is:

Country: Nepal

Province: Province No. 4

District: Kaski

Metropolitan: Pokhara-Lekhnath

Ward No.: 16

Location: Pashchimanchal Campus

The major part of our survey was done in the compound of Pashchimanchal Campus, established in 2044 B.S.for the purpose of providing technical education which is foremost for producing skilled manpower for the development of our country, Nepal. It provides B.E. education in Geomatics,Civil,Computer,Electronics, Electrical and Mechanical  and also MSc in different Fields.

Topography and Geology:

Lamachaur, where Pashchimanchal Campus is situated has steep topography.It is said that Pokhara is standing on porous ground. The porous underground of the Pokhara valley favours the formation of caves and several caves can be found within city limits.

The latitude and longitude of Nepal are:

Latitude 26°22'N to 32°27'N

Longitude 80°04'E to 88°12'E

The latitude and longitude of Pokhara are:

Latitude 28.2096° N

Longitude 83.9856° E

Norms (Technical specifications):

- Conduct reconnaissance survey of the given area. Form a close traverse (major and minor) around the perimeter of the area by making traverse station. In the selection of the traverse station maintain the ratio of maximum traverse leg to minimum traverse leg less than 2:1 for major and less than 3:1 for minor.

- Measure the traverse legs in the forward and reverse directions by means of a tape calibrated against the standard length provided in the field,note that discrepancy between forward and backward measurements should be better than 1:2000.

- Measure traverse angle on two sets of reading by Total  Station. Note that difference between the mean angles of two sets reading should be within the square root of no of station times least count of the instrument.

- Determine the R.L.of traverse stations by fly levelling from the given B.M. Perform two-peg test before the start of fly levelling. Note that collimation error should be less than 1:10000.

- Maintain equal foresight and back sight distances to eliminate collimation error. The Permissible error for fly levelling is ( ±25√k)mm.

- Balance the traverse. The permissible angular error for the sum of interior angles of the traverse should be less then ±√n x1 minutes for Major Traverse and then ±√n x1.5 minutes  for Minor Traverse (n=no of traverse station).For major and minor traverse, the relative closing error should be less than 1:2000 and 1:1000 respectively.

- Plot the traverse stations by coordinate method in appropriate scale, i.e. 1:1000 for major traverse and 1:500 for minor traverses.

- Carry out the detail survey of the given area by tachometric method with reference to the major and minor traverse stations, which have been already plotted. Use conventional symbols for plotting.

Equipment used:

The equipment used in the survey during the preparation of topographic map are as follows:

- Total Station

- Staffs

- Ranging rods

- Tapes

- Levelling instruments

- Pegs

- Compass

- Marker pen

Methodology:

The methodology of surveying is based on the principle of surveying. They are as follows:

Fig:Flowchart of Methodology

The different methodologies were used in surveying to solve the problems arise in the field. These methodologies are as follows:

Reconnaissance(recci):

Reconnaissance (recci) means the exploration or scouting of an area. In survey,it involves walking around the survey area and roughly planning the number of stations and the position of the traverse stations. Recci is primarily done to get an overall idea of the site.This helps to make the necessary observations regarding the total area, type of land, topography,vegetation,climate,geology and indivisibility conditions that help in detailed planning.

The following points have to be taken into consideration for fixing traverse stations:

- The adjacent stations should be clearly inter-visible.

- The whole area should include the least number of station possible.

- The traverse station should maintain the ratio of maximum traverse leg to minimum traverse leg less than 1:2 for Major Traverse and 1:3 for Minor Traverse.

- The steep slopes and badly broken ground should be avoided as far as possible,which may cause inaccuracy in tapping.

- The stations should provide minimum level surface required for setting up the instrument.

- The traverse line of sight should not be near the ground level to avoid the refraction.

- Taking the above given points into consideration, the traverse stations were fixed.Then two- way taping was done for each traverse leg.Thus,permanent fixing of the control points completes recci.

Traversing:

Traversing is a type of surveying in which a number of connected survey lines form the framework.It is also a method of control surveying.The survey consists of the measurement of

- angles between successive lines or bearings of each line

- the length of each line.

There are two types of traverse, They are as follows:

- Closed traverse:

The traverse which either originates from a station and return to the same station completing a circuit or runs between two known stations, is called a closed traverse

- Open traverse:

The traverse which neither returns to its starting station nor closes on any other known station is called an open traverse.

Fig:Types of Traverse

The traversing was performed with the help of Total Station,a modern electronic surveying instrument.

TOTAL STATION:Introduction:

A total station is an optical instrument used a lot in modern surveying and archaeology and, in a minor way,as well as by police,crime scene investigators, private accident re-constructionist and insurance companies to take measurements of scenes.It is a combination of an electronic theodolite(transit), an electronic distance meter (EDM) and software running on an external computer known as a data collector.

Fig:Total Station

The basic principle of Total Station is that the distance between any two points can be known once the time light takes to travel the distance and back and the velocity of light is known. Then the following relation,which is already programmed in the memory of the instrument along with other correction factors,calculates the required horizontal distance and is displayed on the LCD screen.

Major Traverse

The skeleton of lines joining those control points, which covers the whole entire area, is called Major Traverse.Work on Major traverse must be precise.So two-set of reading should be taken for Major Traverse.For convenience, the readings are taken by setting the Total Station at 0°0'0" for one set and 90°0'0" for the second.

In the Field Survey Camp, two traverses-major and minor had to be established.The major traverse had 7 control stations. The control stations were named as ST1,ST2 . The leg ratio of maximum traverse leg to minimum traverse leg was maintained within 1:2.The discrepancy in length between the forward measurements and the backward measurements of all the traverse legs was within 1:1000. Two sets of readings were taken for measuring the horizontal traverse angles.The difference between the mean angles of two sets of readings was within 20" for all the angles.

Minor Traverse

It is not sufficient to detail the area by enclosing with the help of major traverse. Minor traverse is that one which runs through the area to make detailing easy. Minor Traverse covers only small area.Less precise work than that of major traverse is acceptable so that single set reading is sufficient. The minor traverse had 5 control stations and enclosed maximum details. The control stations were named as M1, M2 ....M5 along and it is linked with 2 major control station ST2 and ST7. The leg ratio of maximum traverse leg to minimum traverse leg was maintained within 1:3. The discrepancy in length between the forward measurements and the backward measurements of all the traverse legs was within 1:1000 .

Balancing the traverse :

There are different methods of adjusting a traverse such as Bow ditch's method, Transit method, Graphical method,and Axis method.Among them during the survey camp, Bow ditch's method was used to adjust the traverse.

The basis of this method is on the assumptions that the errors in linear measurements are proportional to L and that the errors in angular measurements are inversely proportional to L,where L is the length of a line.The Bow ditch's rule is mostly used to balance a traverse where linear and angular measurements are of equal precision. The total error in latitude and in the departure is distributed in proportion to the lengths of the sides.

Computation of the coordinates:

According to the accuracy aimed and the nature of the ground. the lengths of traverse legs are measured directly on the ground either by chaining or taping. The traverse angles are measured with a theodolite by setting up the instrument at each station in turn and the vertical angle at each station measured will help to find the tachometric distance and reduce level of that point.

The bearing of the any one of the traverse leg measured and the entire traverse angle measured, the bearing of all the legs can be calculated by:

N

A

L

θ

CP                                           E

Bearing of a line=(bearing of previous line +included angle)±(180)or (540) .

If θ is the bearing of line(c.p.A say),and L be the length of the line and provided that co. ordinate of the control point(c.p) is known then the co-ordinate of me point 'A'  can be calculated as follow:

X-coordinate of A=x-coordinate of control point (c.p)+l*sine

Y-coordinate of A=y-coordinate of control point(c.p)+l*cosθ

R.L or z-coordinate of A=R.L of point (c.p)+H.I±H*Tanθ -Height of signal.

where,H.I.=Height of instrument.

H=Horizontal distance

Balancing the consecutive coordinate:

The process of adjusting consecutive co-ordinates of each line by applying correction to them in such a way that each algebraic sum of the latitude and departure of a close circuit is equal to zero i.e.the sum of the northing should be exactly equal to the sum of the southing and sum of the easting should be exactly equal to the sum of the westing. The closing error however is distributed through-out the whole traverse stations such that its effect is not apparent on the plotted location of the station. And the error can be distributed among the stations if the closing error is within the permissible limit, 7 which is given by:-

Precision=(V(AX2+ΔY2))/P=e/P

This should be greater than 1:2000

Closing Error

If a closed traverse is plotted according to the field measurements,the end of the traverse will not coincide exactly with the starting point. Such and error is known as closing error.

Mathematically,   Closing error (e) =V{(ΣL)z+ (ΣD)2}

Direction, tan θ=(ΣD)/(ZL)

The sign of ΣLand ΣD will thus define the quadrant in which the closing error lies.

Relative error of closure = Error of Closure/Perimeter of the traverse

=e/p

=1/(p/e)

The error (e) in a closed traverse due to bearing may be determined by comparing the two bearings of the last line as observed at the first and last stations of traverse.If the closed traverse,has N number of sides then,

Correction for the first lšne =e/N

Correction for the second line= 2e/N

similarly, correction for the last line=Ne/N=e

In a closed traverse, by geometry,the sum of the interior angles should be (2n-4)x90°where n is the number of traverse sides. If the angles are measured with the same degree of precision,the error in the sum of the angles may be distributed equally among each angle of the traverse.

Detailing:

Detailing means locating and plotting relief in a topographic map. Detailing can be done by either plane table surveying or tachometric surveying. Plane tabling needs less office work than tachometric survey.Nevertheless, during our camp, we used Total Station(T.S.).

Levelling:

Levelling is a branch of surveying the object of which is used:

- To find the elevation of given points with respect to given or assumed datum.

- To establish points at a given elevation or at different elevations with respect to a given or assumed datum.

The first operation is required to enable the works to be designed while the second operation is required in the setting out of all kinds of engineering works. Levelling deals with measurements in a vertical plane. To provide vertical controls in topographic map, the elevations of the relevant points must be known so that complete topography of the area can be explored.

Two types of levelling were performed at the site, namely direct levelling(spirit levelling) and indirect levelling (trigonometric levelling).

- Direct levelling:

This is the most common method of levelling.In this method, a spirit fixed to the telescope of a levelling instrument is used to make the line of sight horizontal. The surveyor is mainly concerned with direct levelling. Following are some special methods of direct (spirit levelling):

- Differential levelling:

- It is the method of direct levelling the object of which is solely to determine the difference in elevation of two points regardless of the horizontal positions of the points with respect of each other.This type of levelling is also known as fly levelling.

- Profile levelling:

It is the method of direct levelling the object of which is to determine the elevations of points at measured intervals along a given line in order to obtain a profile of the surface along that line.

- Cross section levelling:

Cross-sectioning or cross levelling is the process of taking levels on each side of main line at right angles to that line, in order to determine a vertical cross-section of the surface of the ground,or of underlying strata, or of both.

- Reciprocal levelling:

lt is the method of levelling in which the difference in elevation between two points is accurately determined by two sets of reciprocal observations when it is not possible to set up the level between the two points

- 2.Indirect levelling:

Indirect method or trigonometric levelling is the process of levelling in which the elevations of points are computed from the vertical angles and horizontal distances measured in the field,just as the length of any side in any triangle can be computed from proper trigonometric relations.

Adjustments of level:

- Temporary Adjustments:

The adjustments which are made for every setting of alevel are called temporary adjustments. These include the followimg:

a. Setting up the level:

This operation includes fixing the instrument on the tripod and also levelling the instrument approximately by leg adjustment.

b. Levelling up:

This operation includes the accurate levelling with the help of foot screws and by using plate levels.

C.Elimination of the parallax:

Parallax is a condition when the image formed by the objective is not in the plane of the cross hairs. Parallax is eliminated by focusing the eyepiece for distinct vision of the cross hairs and by focusing the objective to bring the image of the object i the plane of cross hairs.

- Permanent Adjustment

To check for the permanent adjustment of level two-peg test method should be performed.

Fig:Two Peg Test Arrangemnet

Two staffs were placed at A and B of known length (about 60m). First the instrument was setup on the line near B and both staff readings (Top, Middle, and Bottom) were taken. Then, the instrument was setup at the middle C on the line and again both staff readings on A and B was taken. Then computation was done in order to check whether the adjustment was within the required accuracy or not. The collimation error was found to be 1:10000 which satisfied the permissible error limit(1:10,000). No permanent adjustment was required since the error was within the permissible value.

Booking and reducing levels:

There are two methods of booking and reducing the elevation of points from the observed staff reading.

1.Height of the Instrument method

Arithmetic Check:ΣBS-ΣF.S.= Last R.L.-ParStRL.

2.Rise and Fall method

Arithmetic Check:Z BS-ΣF.S.=Z Rise- Zfall- Last R.L.-First RL

Fly Levelling:

The RL of Control points was found by transferring the level from known BM located near Football Ground by the process of fly levelling. In this method auto level was used and the level was transferred directly by taking BS and FS at every Turning Point.

Level transfer to the major and minor traverse stations:

The R. L of the temporary benchmark.was then transferred to the control stations of the major and minor traverse. The closing error was found to be within the permissible limits.The misclosure was adjusted in each leg of the levelling path by using the following formula

Permissible error=±25 k1/2mm where k is perimeter in Km

Actual Error (e)=ΣBS-ZF.S.=Last R.L.-First R.L

Correction ith leg=-(e*(L1+L2+..+Lu)/P

Where Li,L2 Li=Length of 1st , 2nd ,…..ith  leg .P is perimeter

Relative Precision=1/(p/e)

Computations and plotting:

For the calculations as well as plotting, we applied the coordinate method (latitude and departure method).In this method,two terms latitude and  departure are used for calculation. Latitude of a survey line may be defined as its coordinate lengths measured parallel to an assumed meridian direction. The latitude (L) of a line is. positive when measured towards north, and termed Northing and it is negative when measured towards south, and termed Southing. The departure (D)of a line is positive when measured towards east, and termed Easting and it is negative when measured towards south,and termed Westing.

The latitude and departures of each control station can be calculated using the relation:

Latitude =L Cosθ

Departure=LSinθ

Where,L=distance of the traverse legs

θ=Reduced bearing

If a closed traverse is plotted according to the field measurements, the end of the traverse will not coincide exactly with the starting point. Such and error is known as closing error.

Mathematically,

Closing error (e) = [(L)2+(D)}

Relative error of closure =e/p

The error(e) in a closed traverse due to bearing may be determined by comparing the two bearings of the last line as observed at the first and last stations of traverse.If the closed traverse, has N number of sides then,

Correction for the first line -e/N

Correction for the second line = 2e/N

And similarly, correction for the last line = Ne/N=e

In a closed traverse,by geometry, the sum of the interior angles should be equal to (2n- 4)x90°where n is the number of traverse sides. If the angles are measured with the same degree of precision,the error in the sum of the angles may be distributed equally among each angle of the traverse.

Mathematically,

- Correction in departure of a side of traverse = - (Total departure misclosure/ traverse perimeter)x length of that side.

- Correction in latitude of a side of traverse= - (Total latitude misclosure/traverse perimeter)xlength of that side.

In the case of length,the difference in values obtained by forward and backward taping is called discrepancy.In addition,the reciprocal of the discrepancy divided by the mean of the two measurements is called precision. Both the discrepancy and the precision for each traverse leg should be within the given limits.

Plotting of Major and Minor traverse:

After computing the co-ordinate of each of the control points, they were plotted in A1 size grid paper. Both major and minor traverses were plotted to 1:1000 scales. The plotted traverse was made at the center of the sheet with the help of least co- ordinates and highest co-ordinates.

Result and calculation

As we have collected all the required data from the field which can be seen at thee ANNEX below :

Calculation for Major Traverse

Theoretical Sum of angles =900°

Observed angles sum= 900° 1' 21.25"

So,Error in each angle is =0° 0' 11.61"

Corrected angles are :

| 1-2 | 98° 48' 59.07" |
| --- | --- |
| 2-3 | 126° 53' 16.07" |
| 3-4 | 131° 07' 41.57" |
| 4-5 | 119° 38' 30.82" |
| 5-6 | 199° 38' 36.32" |
| 6-7 | 63° 22' 0.07" |
| 7-1 | 160° 30' 56.07" |

AND we have also gather the bearing of line 4-3 using compass which was 58° 17' 18" .

Even we have the independent coordinate of Benchmark point which was

Northing(y): 3128925.805

Easting(x):791888.701

Height(z):943.19

Using this and bearing of all lines are calculated which was:

Bearing of line 1-2(θ) =136° 18' 15.64"

Distance between 1-2  (Annex table)(D) : 167.4568m

Latitude=Dcos(θ)= -121.0770

Departure=Dsin(θ)=115.6861

Closing Error =√(L2 +D2)

=0.070411

Adjustment of closing error,

According to Bowditch's rules,

Correction in latitude =(L*distance between two respective lines)/perimeter

Correction :

Latitude=  -8.51*10^-3

Departure=   8.51*10^-3

Independent  Co-ordinate of line 1-2

Latitude =       3129115.986

Departure =   792149.0825

Using this rules the coordinate of all the station were calculated which can be seen in ANNEX-A TABLE BELOW.

Calculation For Minor Traverse

The process for the coordinate calculation of Minor station is as that of the Major where as in minor station error distribution in angle is done in different way that is :

The traversing closing in minor traverse was  e=1' 43". And this error was distributed over 5 minor station by the rule :

M1=e/5

M2=2*e/5

M3=3*e/5 and so on.

Results

In this way we applied the concept of traversing,levelling to establish the horizontal and vertical control and performed the detailing task of different features natural as well as man made to obtain the good topographical map of our campus which included the contours as well for better understanding of the features.

Comments and Conclusions:

Comments

The site for the Topographic  survey camp was Pashchimanchal Campus,Pokhara. Where we can find  different engineering structure and many natural feature as well. The survey camp was conducted under the very highly skilled lectures with proper guides and warning. However the working environment through out the Camp was maintain good and instrument were of best quality but due to the improper handling of previous batch student  and some instrument being old and defective which made incorrect reading which affect the whole data that made the camping a bit tides , laborious, time consuming and creates confusion among student. Hope this above mentioned problem will be solved and the upcoming survey camp will run smoothly without any problems.

Conclusion

The given Topography survey camp work was finished satisfactorily within the given span of time. The subject survey needs practice as much as possible. For surveying, theory can only be taken as the introduction but if there is practice, there will be much gain of knowledge about the techniques of surveying. Thus, this camp helps us by practicing the survey work to gain the much essential knowledge as far as possible. It is better to say that it provides us a confidence to perform survey and apply the techniques at any type of problem facing during the actual work in the future career.

ROAD ALIGNMENT

Introduction:

Road is an important infrastructure for development. It occupies a pivotal position in the growth of developing countries. The various.civilizations of the world that are known for their excellence and attainments have left traces of their art of road construction. Roads can be constructed to penetrate the interior of any region and to connect remote villages. The advantage becomes particularly evident when planning the communications system in hilly regions & sparsely populated areas. Road transport offers quick & assured deliveries, a flexible service free from fixed schedules, door to door service,permits simpler packing, has a high employment potential etc. The safe, efficient and economic operation of a highway is governed to a large extent by the care with which the geometric design has been worked out. Geometric design includes the design elements of horizontal & vertical alignment,sight distance,X-section components, lateral & vertical clearances, control of access, etc.

The general guide-lines in selecting the alignment & locating route are:

- Should handle the traffic most efficiently & serve inhabited localities

- Should have minimum Gradients & curvature, necessary for terrain.

- Should involve least impact. on the environment.

- Should be located along the edge of properties.

In case of hill road,

- Should attain change in elevation by adopting ruling gradient in most of length

- Should avoid unstable hill features &areas prone to land-slides Should avoid steep terrain.

- Should avoid hair-pin bends.

- Should align preferably on the side of hill exposed to sun during winter.

- Should avoid deep cuttings & costly tunnels.

- Should develop alignment to suit obligatory points like passes, saddles, valleys, crossing points of major rivers.

In short,road should be short, easy, safe and economic as far as possible.

Roads are specially prepared ways between different places for the

use of vehicles, people & animals.In countries like Nepal, where there are less chances of airways & almost negligible chances of water way,roads form a major part of the transportation system. Therefore, it would not be an exaggeration in saying that the roads have an almost importance. A road is an identifiable route, way or path between two or more places. Roads are typically smoothed, paved, or otherwise prepared to allow easy travel; though they need not be, and historically many roads were simply recognizable routes without any formal construction or maintenance. The road need to pass through positive obligatory points. Positive obligatory points include cities, schools, markets and negative obligatory points include temples, national parks and wild life conservation areas. Road must not pass through such negative obligatory points.

Before the construction of the road, preliminary survey is done. Road alignment is the preliminary stage of road construction. Selection of Intersection Points (IP) is the foundation of construction of the road. After that cross section, longitudinal section and formation level are required.

Objectives of road alignment survey:

The following are objectives of carrying out road alignment survey:

- To set out curve with appropriate radius and deflection angle.

- To prepare plan, cross-section and L-section of the road.

- To work in co-ordination with team members to ease out setting of curve.

- To get technical knowledge on working procedure & methodology on setting out curve.

Brief Description of the project area:

The area where the road alignment survey was performed is situated in Pokhara-19,Batulechaur. The road had to go along the bank of Kali stream(kali khola) on the side of a hill which was very undulating. Most of the places along the road were damp. There were many large stones and rocks along and on the road.

Norms (Technical Specifications):

Recci alignment selection was carried out of the road corridor considering permissible gradient, obligatory points and geometry of tentative horizontal and vertical curves. The road setting horizontal curve, cross sectional detail in 20m interval and longitudinal profile were prepared. Geometric curves, road formation width, right of way,crossings and other details were shown in the map. While performing the road alignment survey, the following norms were strictly followed :

- The road had to be designed starting near the given rock (which was PBM) and ending at the bridge.

- If the external deflection angle at the l.P.of the road is less than 5°,curves need not be fitted

- Simple horizontal curves had to be laid out where the road changed its direction, determining and pegging three points on the curve the beginning of the curve, the middle point of the curve and the end of the curve along the center line of the road.

- The radius of the curve had to be chosen such that it was convenient and safe i.e.not less than 15 m radius.

- The gradient of the road had to be maintained below 12%.

- Cross sections had to be taken at 20 m intervals and at the beginning,middle and end of the curve, along the center-line of the road -observations being taken for at least 5 m on either side of the center line. If undulations are there, then section at that place should be taken.

- Plan of the road had to be prepared on a scale of 1:700

- L-Section of the road had to be plotted on a scale of 1:1000 horizontally and 1:100 vertically.

- The cross section of the road had to be plotted on a scale of 1:100(both vertical and horizontal)

- The amount of cutting and filling required for the road construction had to be determined from the L-Section and the cross sections.However,the volume of cutting had to be roughly equal to the volume of filling.

Equipment & Accessories:

The following are the instruments used during the road alignment survey in the field: 

- Theodolite 

- Tripod Stand 

- Plumb Bob 

- Leveling Instrument

- Leveling Staffs 

- Ranging Rods 

- Measuring Tape 

- Pegs and Arrows 

- Marker Pen

Methodology:

The alignment of road includes several ways and procedures that need to be carried out. Following are the listed methodology:

Reconnaissance:

The reconnaissance survey was performed along the given route. Tentative estimation were done for the intersection points, where the direction had to be changed. While returning back the route, the IPs was fixed. For this the inter-visibility of the stations was checked and gradient between the two IPs was adjusted such that it does not exceed 12%, using the abney level. Meanwhile the pegs with IP no. were driven at these points.

Horizontal Alignment:

Horizontal alignment is done for fixing the road direction in horizontal plane. For this, the bearing of initial line connecting two initial stations was measured using compass. The interior angles were observed using theodolite at each IP and then deflection angles were calculated.

Deflection angle = 180° - observed angle

If the deflection angle is positive the deflection is towards right and if the deflection angle is negative the deflection is towards the left. The radius was assumed according to the deflection angle. Then the tangent length, Beginning of the Curve (BC), End of the Curve (EC), apex distance along with their chainage were found by using the following formulae:

Tangent length (T) = R x Tan (Δ/2)

Length of curve (L.C) = Π x R x Δ/180

Apex distance = R x( 1/(Cos(   Δ /   2)   -1)

Chainage of BC = Chainage of IP – T

Chainage of MC = Chainage of BC +LC/2

Chainage of EC = Chainage of MC + LC/2

= Chainage of BC + LC

The BC and EC points were located along the line by measuring the tangent length from the apex and the points were marked distinctly. The radius was chosen such that the tangent does not overlap. The apex was fixed at the length of apex distance from IP along the line bisecting the interior angle.

Vertical Alignment:

Vertical profile of the Road alignment is known by the vertical alignment. In the L-section of the Road alignment, vertical alignment was plotted with maximum gradient of 12 %. According to Nepal Road Standard, Gradient of the Road cannot be taken more than 12 %. In the vertical alignment, we set the vertical curve with proper design. Vertical curve may be either summit curve or valley curve. While setting the vertical alignment, it should keep in mind whether cutting and filling were balanced or not.

Leveling:

The method of fly leveling was applied in transferring the level from the given B.M. to all the I.Ps, beginnings, mid points and ends of the curves as well as to the points along the center line of the road where the cross sections were taken. After completing the work of one way fly leveling on the entire length of the road, check leveling was continued back to the B.M. making a closed loop for check and adjustment. The difference in the R.L. of the B.M. before and after forming the loops should be less than 25√ k mm, where k is the loop distance in km.

L-section & Cross Section:

Nature of the ground, the variation in the elevations of the different points along the length of road need to be known for the construction of the road. For this L-Section of the road is required. In order to obtain the data for L-Section, staff readings were taken at points at 20m intervals along the center line of the road with the help of a level by the method of fly leveling. Thus after performing the necessary calculations, the level was transferred to all those points with respect to the R.L. of the given B.M. Then finally the L-Section of the road was plotted on a graph paper on a vertical scale of 1:100 and a horizontal scale of 1:1000. The staff readings at BC, EC and apex were also taken. The RL of each point were calculated.

Cross sections at different points are drawn perpendicular to the

longitudinal section of the road on either side of its center line in order to present the lateral outline of the ground. Cross sections are also equally useful in determining the amount of cut and fill required for the road construction. Cross sections were taken at 20m intervals along the center line of the road and at points where there was a sharp change in the elevation. While doing so, the horizontal distances of the different points from the center line were measured with the help of a tape and the vertical heights with a measuring staff. The R.L. was transferred to all the points by performing the necessary calculations and finally, the cross sections at different sections were plotted on a graph paper on a scale of both vertical and 1:100 - horizontal.

Curve Setting:

A regular curved path followed by highway or railway alignment is curve. It is introduced wherever it is necessary to change the direction of motion due to the nature of terrain. A curve may be circular, parabola or spiral and is always tangential to two straight directions. There may be different types of curves: Simple curve, Compound curve, Reverse curve, Transition curve.

Simple Circular Curve: A simple circular

curve is the curve, which consists of a single arc of a circle. It is tangential to both the straight lines.

Setting Out OF Simple Circular Curves:

Linear method: - In this method, only a chain or tape is used. Linear methods are used when a high degree of accuracy is not required and the curve is short.

eg: Offsets from Long Chord

Offsets form Tangents

Successive bisection of Chords

Offsets from Chords produced

Angular method: - In this method, an instrument like theodolite is used with or without chain or tape.

eg.: Rankine’s Method of Tangential Deflection Angles

Two Theodolite Method

- Offset from Long Cord Method:

Mid-ordinate can be determined by the relation

Oo = R - √ [R2 – (L/2)2]

The Ordinate at a distance ‘x’ is given by,

Ox = √ [ ( R2 –  X2) – ( R - Oo ) ]

Where, Oo = mid-ordinate

Ox = ordinate at distance x from the mid point of the chord

L = length of the long chord

R = Radius of the curve

- Rankine’s Method:

In Rankine’s method, it’s assumed that the length of the curve and the chord length are equal (case for larger radius). The deflection angle to any point on the curve is an angle at the point of contact between the back tangent and the chord joining the point of contact and that point.  The angle subtended by first sub-chord is given by,

δ1 = 1718.9 C1/R minutes

The angle subtended by each normal chord is given by the formula,

δ= 1718.9 C/R minutes

If δ1, δ2…, δn are the tangential angles made by successive chords with their tangents and Δ1, Δ2……..Δn are the total deflection angles, then

Δ1 = δ1

Δ2 = Δ1 + δ1 = δ1+ δ2

…………………….

………………………………..

Similarly,

Δn =  Δ n -1 + δn = δ1+ δ2 + δ3 +………. + δn = Δ/2

Field Procedure:

The instrument is set at T1 and zero is set along P.I..

Then the theodolite is set to read an angle of δ1(=Δ1).

With T1 as center and C1 as radius, the tape is swung and arrow was marked at intersection of the tape with cross hairs.

Then angle Δ2 was set on the theodolite and with length of normal chord as radius, the next point on the curve was marked at the point of intersection. This procedure is continued till the point of tangency is located.

Result and calculation

Calculation

First we collected the deflection angle of  IP1 and from IP0 which was 43°44’52”

since we have the formula to calculate the elements of the curve

we know,

Chainage of IP = 0+00 m

Radius (R) = 40m

Deflection angle(∆) = 43°44'52”

From the formula,

Tangent length (TL)= R tan(∆2)=40 x tan(43°44'52”2)=16.058 m

Curve length (L)= ∏R∆180= ∏∗40∗43°44'52”180 = 20.54 m

Mid ordinate(MO) = R(1-cos(∆2)) = 40*(1-cos(43°44'52"2)) = 2.87 m

Apex distance = R(sec(∆2)-1) = 40*(sec(43°44'52”2)-1) = 3.10 m

Chainage of IP1 = 0+62.8 m

Chainage of BC = Chainage of IP1- tangent length = 0+46.74 m

Chaianage of MO = 0+62.012 m

Chainage of EC = Chainage of IP1+ tangent length = 0+77.282 m

In this way we calculated all the elements of the curve within the chainage of 0+1000m  which can be seen in the annex table below.

Result

By applying the principle of levelling and curves ,we finally obtained the cross sections and profile of the bridge providing different geographical information .the best plan was also obtained that would help further in future extension of roads.the best alignment of road was obtained that could be used to construct,extend as well as manage the roads.

Comments and Conclusion:

Survey of the road alignment was done to make most economical, comfortable, safe and durable.  Curves are set according to Road Design Standards for comfort and other factors. While setting the road alignment, it should be kept in mind that the minimum IP points should be taken as far as possible and deflection angles should be minimum as far as possible. The task was challenge able and tough due to unpleasant

environment and continuous traffic.

BRIDGE SITE SURVEY

Introduction

Bridges are the structures that are constructed with the purpose of connecting two places separated by deep valleys or gorges or rivers and streams. Bridges are usually the cross drainage and hence a part of roads making them shorter and hence economical. For places, where the ground is uneven and undulated and where the number of rivers is large, bridges are the most economic and efficient way. It is a very convenient way.

Objectives: 

- To select the possible bridge site and axis for the construction of bridge.

- To collect the preliminary data i.e. normal water flow level, high flood level. 

- To study about the geological features of the ground. 

- To carry out surveying for topographical mapping, longitudinal and cross sections at both the upstream and downstream side of the river.

Brief description of the site:

Bridge site survey was conducted over a Kali spring (Kali khola).The spring collects water coming through different hill slopes. Our site was near the Pig Farm and the Bhalaam spring (Bhalaam khola). The site was mossy and swampy. Huge boulders and rocks are to be found near the site.It was damp and hilly.

Hydrology,Geology and Soil Condition:

Sloppy hills with trees surrounded the site. There are rocks also.The ground was damp and swampy. The soil was soft and clayey. It was blackish brown in colour. The hill slopes on both sides are very steep and are thus geologically stable.

Technical Specification(Norms)

A bridge site topographical survey was carried out and the alignment of the bridge axis was fixed by triangulation.Two base lines were measured by tape with two-way linear measurement. Along with these we are also supposed to take L-section and X-section of the river downstream and upstream.A topographic map was prepared by tachometric surveying and longitudinal and cross-sectional profile of the area was drawn.

The following norms were followed while performing the bridge site survey in the field:

- Control point fixing as well as determining the length of the bridge axis had to be done by the method of triangulation. While forming triangles, proper care had to be taken such that the triangles were well conditioned, i.e. none of the angles of the triangle were greater than 120° or less than 30°.

- In triangulation, distance of Base Line must be measured in an accuracy of 1:2000.

- The triangulation angle had to be measured on two sets of readings by Theodolite and the difference between the mean angles of two sets of readings had to be within a minute. Angular misclosure for base triangle should be 30” √N and other triangle 1’√N  .

- Carry out reciprocal levelling to transfer level from one bank to other bank of the river within a precision of ±25√k mm Determine the RL of the other triangulation stations by fly levelling from the end point of bridge axis.

- Plot a topographic map indicating contour lines at suitable interval (contour interval = 1m).

- The scale for plotting the topographical map was given to be 1:500 .

- In order to plot the longitudinal section of the river, data had to be taken along the riverbed 150 m upstream and at least 100 m downstream. The plot for the longitudinal section along the flow line had to be done in a scale of 1:50 for vertical and 1:500 for horizontal, for cross-section v=H=1:50 or 100.

Equipment & Accessories:

The equipment used in the survey during the preparation of topographic map in bridge site are as follows:

- Theodolite

- Ranging Rods

- Measuring Tapes

- Leveling Staffs

- Pegs

- Marker Pen

Methodology:

The various methods performed during the bridge site survey were site selection, triangulation, leveling (fly levelling and reciprocal levelling), detailing by total station, cross section, and L-section. The brief descriptions of these methodologies are given below:

Site Selection:

Site selection is the first and foremost step for the construction of bridge. Several governing factors are there for the site selection of the bridge. Geological condition, socio-economic and ecological aspect etc. guides the way of selection of bridge site. Therefore, the site was chosen such that it is laid on the very stable rocks at the bed of river as far as possible and not affect the ecological balance of the flora and fauna of the site area. The location of the bridge was selected in such a way that the heights of the roads joined by the proposed bridge were almost the same. This prevented a lot of cutting and filling to maintain a gentle gradient. The bridge site was chosen in such a way that the bridge axis was perpendicular to the flow direction and was also shorter in span so as to make the construction economical. The starting point of bridge axis was not laid on the curve of the road.

Tacheometry:

It is the branch of surveying in which both the horizontal and vertical distances between stations are determined by making instrumental observations. Tacheometry is used in the preparation of contour maps and they also provides a good check on distances measured with tape or chain.  Principle of tacheometry:  In isosceles triangles, the ratio of the perpendiculars from the vertex on their bases is constant.  The formula for the horizontal distance is

H=k*sCos2  θ

The formula for the vertical distance is

V = (k*s Sin2 θ)/2

Where, s = staff intercept ;

θ = Vertical Angle

Topographic survey:

For the topographic survey of bridge site, triangulation was done. Triangulation is the process of measuring the angles of a chain or a network. The main purpose of the triangulation was to determine the length of the bridge axis. The triangulation also serves the control points for detailing. The bridge axis was set and horizontal control stations were fixed on either side. Distances between stations on the same sides of river i.e. base lines were measured with tape precisely. Then the interconnecting triangles were formed and angles were measured with the theodolite with two sets of observations. The bridge axis length or span was calculated by solving the triangles using the sine rule. For vertical control, the level was transferred from the arbitrary benchmark and RL was transferred to the stations on the next bank by reciprocal leveling while direct level transfer method was used or the same bank.

L-Section & Cross Section:

For gaining an idea about bed slope, nature of the riverbed, and the variation in the elevations of the different points along the length of the river, L-section is carried out. Keeping the instrument at the control (traverse) stations on the river banks, the staff readings were taken at different points along the center line of the river up to a 150 meters upstream and 50 m downstream. The R.Ls of the traverse stations being known previously, the levels of the different points on the river were calculated. Then the L-Section of the riverbed was plotted on a graph paper on scale for vertical and horizontal.

Cross-section of a river at a particular point is the profile

of the lateral sides from the center line of the river cut transverse to the L-Section at that point. The cross section can be used to calculate the volume and discharge of water at the particular section if the velocity at the cross section is known. Cross sections were taken at an interval of about 25 m extending 150 m upstream and 50 m downstream of the river. Staff readings of points along a line perpendicular to the flow of river were taken from the stations points and the elevations of the points were calculated using tachometric methods.

Leveling:

Transferring R.L. from B.M. to control points:

The benchmark was on the manhole near the cricket ground. R.L. was transferred to the triangular station from the B.M. by fly leveling by taking the back sight-reading to the bench mark which should be within the given accuracy. The R.L. was transferred to the opposite bank of the river by reciprocal leveling.

Reciprocal Leveling:

This method is applicable when taping is obstructed but not the vision. For transferring the RL across the bridge reciprocal leveling was performed. This method eliminates the error due to focusing, collimation, earth’s curvature and refraction of atmosphere etc. True difference in elevation between A and B = H =a1- (b1-e) Also the true difference in elevation = H = (a2 - e)-b2. Taking the average of the two differences we get the difference in elevation between A and B.

Fig:Reciprocal Leveling

Detailing:

Total station was used for detailing of the entire bridge site. The reading was taken from the different station set up. The detailing was done with respect to the skeleton formed by triangulation. The vertices of triangles serve as a control point. The details were booked, up to 150m upstream and 50m downstream. The data and the calculations have been tabulated in a systematic way.

Computation & Plotting:

The use of total station makes the detailing process easy and fast. The total station gives the direct vales of the horizontal distances and vertical height difference between the station point and the detailing point. The RLs of the points can be calculated by using following formula. RL of detail = RL of station + HI ± V-Target Height The topographic map, the longitudinal section and the cross section were plotted on the respective scales after the completion of calculations. By taking an A1 grid sheet, control stations were plotted accurately. Then all hard details as well as contours were plotted with reference to the control stations by the method of angle and distances.

Result and calculation

C                                     B

A                      25m           D

From ∆ABC,

AB1 =  25 x sin(50°45'21.75”) sin(32°51'12.75")

= 35.69 m

From ∆ABD,

AB2 = 25 x sin(53°40'14.75")sin(34°20'19.5")

= 35.71 m

Discrepancy = AB1−AB2(AB1+AB22)

= AB1−AB2AB1+AB2 x 2

= 0.00056      1:1785.714 ( Our result is within the range of  permissible Discrepancy (1:2000))

Result

Finally our main objective to obtain the axis of the bridge,span length and performing the tacheometric detailing of the river’s upstream and downstream was completed using the main principles of triangulation as well as tacheometry.the best detail of river was obtained the best site for bridge and different conditions were predicted as well like HFL,NF etc

Comments and Conclusions:

The bridge axis was set keeping in mind all the requisites that the proper site for the bridge has to be.The result of the computations of the triangulation gave the axis span of 35.70m. During the selection of the site all the considerations like geological, socio-economical and topographical considerations were made and the best site was selected. In site  there was presence of trees along with bushes.The bridge síte survey was conducted to give broad knowledge about importance of reciprocal levelling, necessities of triangulation concept for fixing bridge span &to give wide concept about bridge site.

BIBLIOGRAPHY:

1. Agor, R., A Textbook of Surveying and Leveling, Khanna Publishers. 2. Jain, A.K., Jain, A.K. and Punmiya, B.C.

3. Surveying Volume I and II. Lakshmi Prakashan.

4. Surveying and Levelling by NN Basak

5. SK Duggaal – Surveying – Tata McGraw Hill Education Private Limited , New Delhi

6. Website:

www.wikipedia.com, www.theconstructor.org/surveying

ANNEX-A

# Field Book and Calculation

OBSERVER:GROUP 3

RECODER:GROUP 3

TOPOGRAPHICAL SURVEY

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                        Date:2080/12/25

Recorder:Group 3                                                       Time:7:00am

Instrument: Total station and Reflector prism

TABLE :Horizontal Angle Measurement Sheet of Major Traverse

| Instrument At | Sighted To | Horizontal Circle Reading |  |  | Angle | Mean | Horizontal Angle | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D | M | S |  |  |  |  |  |
| 4 | 5 | 0 | 0 | 0 | 119° 38' 45" | 119° 38' 40" | 119° 38' 41.25" | 118.6295 |  |
|  |  | 180 | 0 | 10 |  |  |  | 118.6252 |  |
|  | 3 | 119 | 38 | 45 | 119° 38' 35" |  |  | 96.2365 |  |
|  |  | 299 | 38 | 45 |  |  |  | 96.2379 |  |
| 4 | 5 | 90 | 0 | 0 | 119° 38' 43" | 119° 38' 42.5" |  | 118.8376 |  |
|  |  | 270 | 0 | 9 |  |  |  | 118.6464 |  |
|  | 3 | 209 | 38 | 43 | 119° 38' 42" |  |  | 96.2352 |  |
|  |  | 29 | 38 | 51 |  |  |  | 96.2354 |  |
| 3 | 4 | 0 | 0 | 0 | 131° 07' 58" | 131° 07' 59" | 131° 07' 52" | 96.1983 |  |
|  |  | 179 | 59 | 50 |  |  |  | 96.1970 |  |
|  | 2 | 131 | 07 | 58 | 131° 08' 00" |  |  | 131.7973 |  |
|  |  | 311 | 08 | 05 |  |  |  | 131.7962 |  |
| 3 | 4 | 90 | 0 | 0 | 131° 07' 43" | 131° 07' 45" |  | 96.2302 |  |
|  |  | 269 | 59 | 57 |  |  |  | 96.2313 |  |
|  | 2 | 221 | 07 | 43 | 131° 07' 47" |  |  | 131.8027 |  |
|  |  | 41 | 07 | 44 |  |  |  | 131.8028 |  |
| 2 | 3 | 0 | 0 | 0 | 126° 53' 24" | 126° 53' 27.5" | 126° 53' 26.5" | 131.7919 |  |
|  |  | 179 | 59 | 53 |  |  |  | 131.7918 |  |
|  | 1 | 126 | 53 | 24 | 126° 53' 31" |  |  | 167.4623 |  |
|  |  | 306 | 53 | 24 |  |  |  | 167.4633 |  |
| 2 | 3 | 90 | 0 | 0 | 126° 53' 27" | 126° 53' 25.5" |  | 131.7914 |  |
|  |  | 179 | 59 | 54 |  |  |  | 131.7916 |  |
|  | 1 | 216 | 53 | 27 | 126° 53' 24" |  |  | 167.4636 |  |
|  |  | 36 | 53 | 18 |  |  |  | 167.4625 |  |
| 1 | 2 | 0 | 0 | 0 | 98° 49' 15" | 98° 49' 6.5" | 98° 49' 9.5" | 167.4579 |  |
|  |  | 180 | 0 | 07 |  |  |  | 167.4610 |  |
|  | 7 | 98 | 49 | 15 | 98° 48' 58" |  |  | 178.9687 |  |
|  |  | 278 | 49 | 05 |  |  |  | 178.9689 |  |
| 1 | 2 | 90 | 0 | 0 | 98° 49' 11" | 98° 49' 12.5" |  | 167.4568 |  |
|  |  | 269 | 59 | 51 |  |  |  | 167.4545 |  |
|  | 7 | 188 | 49 | 11 | 98° 49' 14" |  |  | 178.9682 |  |
|  |  | 8 | 49 | 05 |  |  |  | 178.9686 |  |
| 7 | 1 | 0 | 0 | 0 | 160° 31' 08" | 160° 31' 7" | 160° 31' 6.5" | 178.9592 |  |
|  |  | 180 | 0 | 09 |  |  |  | 178.9604 |  |
|  | 6 | 160 | 31 | 08 | 160° 31' 07" |  |  | 147.8424 |  |
|  |  | 340 | 31 | 16 |  |  |  | 147.8400 |  |
| 7 | 1 | 90 | 0 | 0 | 160° 31' 03" | 160° 31' 06" |  | 178.9621 |  |
|  |  | 270 | 0 | 02 |  |  |  | 178.9639 |  |
|  | 6 | 250 | 31 | 03 | 160° 31' 09" |  |  | 147.8411 |  |
|  |  | 70 | 31 | 11 |  |  |  | 147.8405 |  |
| 6 | 7 | 0 | 00 | 0 | 63° 22' 14" | 63° 22' 14" | 63° 22' 10.5" | 147.8105 |  |
|  |  | 180 | 00 | 03 |  |  |  | 147.8127 |  |
|  | 5 | 63 | 22 | 14 | 63° 22' 14" |  |  | 142.8387 |  |
|  |  | 243 | 22 | 17 |  |  |  | 142.8372 |  |
| 6 | 7 | 90 | 0 | 0 | 63° 22' 10" | 63° 22' 07" |  | 147.8122 |  |
|  |  | 270 | 0 | 10 |  |  |  | 147.8111 |  |
|  | 5 | 153 | 22 | 10 | 63° 22' 04" |  |  | 142.8374 |  |
|  |  | 333 | 22 | 14 |  |  |  | 142.8366 |  |
| 5 | 6 | 0 | 0 | 0 | 199° 38' 55" | 199° 38' 54" | 199° 38' 55" | 142.8481 |  |
|  |  | 180 | 0 | 06 |  |  |  | 142.8481 |  |
|  | 4 | 199 | 38 | 55 | 199° 38' 53" |  |  | 118.6435 |  |
|  |  | 19 | 38 | 59 |  |  |  | 118.6415 |  |
| 5 | 6 | 90 | 0 | 0 | 199° 38' 45" | 199° 38' 39.5" |  | 142.8497 |  |
|  |  | 270 | 0 | 8 |  |  |  | 142.8480 |  |
|  | 4 | 289 | 38 | 45 | 199° 38' 34" |  |  | 118.6439 |  |
|  |  | 109 | 38 | 42 |  |  |  | 118.6435 |  |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                             Date:2080/12/25

Recorder:Group3                                                              Time:7:00am

Instrument: Total station and Reflector prism

Table:Major Traverse Computation Sheet

| Line | Length | Point | Horizontal Angle | Corrected Horizontal Angle | WCB | Consecutive Co-ordinate |  | Correction |  | Corrected Consecutive Co-ordinate |  | Independent Co-ordinate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Latitude | Departure | Latitude | Departure | Latitude | Departure | Latitude | Departure |
| 1-2 | 167.4602 | 1 | 98° 49' 9.5" | 98° 48' 59.07" | 136° 18' 15.64" | -121.0770 | 115.6861 | -8.51*10^-3 | 8.51*10^-3 | -121.0851 | 115.6948 | 3129115.986 | 792149.0825 |
| 2-3 | 131.7973 | 2 | 126° 53' 26.5" | 126° 53' 16.07" | 189° 24' 59.57" | -130.0213 | -21.5639 | -6.69*10^-3 | 6.69*10^-3 | -130.027 | -21.5566 | 3128994.901 | 792264.7770 |
| 3-4 | 96.2363 | 3 | 131° 07' 52" | 131° 07' 41.57" | 238° 17' 18" | -50.5861 | -81.8686 | -4.89*10^-3 | 4.89*10^-3 | -50.5908 | -81.8636 | 3128864.873 | 792243.2207 |
| 4-5 | 118.6857 | 4 | 119° 38' 41.25" | 119° 38' 30.82" | 298° 38' 47.18" | 56.8983 | -104.1579 | -6.03*10^-3 | 6.03*10^-3 | 56.8925 | -104.1517 | 3128814.283 | 792161.3571 |
| 5-6 | 142.8492 | 5 | 199° 38' 55" | 199° 38' 36.32" | 279° 0' 10.85" | 22.3540 | -141.0893 | -2.17*10^-3 | 2.17*10^-3 | 22.3470 | -141.0818 | 3128871.175 | 792057.2054 |
| 6-7 | 147.8212 | 6 | 63° 22' 10.5" | 63° 22' 0.07" | 35° 38' 10.78" | 120.1390 | 86.1263 | -7.51*10^-3 | 7.51*10^-3 | 120.1318 | 86.13400 | 3128893.522 | 791916.1236 |
| 7-1 | 178.9650 | 7 | 160° 31' 6.5" | 160° 30' 56.07" | 55° 7' 14.71" | 102.3409 | 146.8156 | -9.09*10^-3 | 9.09*10^-3 | 102.3322 | 146.8249 | 3129013.654 | 792002.2576 |
|  | 983.81 |  |  |  |  | 0.05 | -0.05 |  |  | 0.00 | 0.00 |  |  |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                             Date:2080/12/26

Recorder:Group 3                                                              Time:7:00am

Instrument: Total station and Reflector prism

TABLE :Horizontal Angle Measurement Sheet of Minor Traverse

| Instrument At | Sighted To | Horizontal Circle Reading |  |  | Angle | Mean | Horizontal Angle | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D | M | S |  |  |  |  |  |
| 2 | 1 | 0 | 0 | 0 | 297° 17' 07" | 297° 17' 07" | 297° 17' 07" | 167.4591 |  |
|  |  | 180 | 0 | 04 |  |  |  | 167.4586 |  |
|  | TCC (M1) | 297 | 17 | 07 | 297° 17' 07" |  |  | 90.9057 |  |
|  |  | 117 | 17 | 16 |  |  |  | 90.9057 |  |
| 2 | 1 | 90 | 0 | 0 | 297° 17' 07" | 297° 17' 07" |  | 167.4554 |  |
|  |  | 269 | 59 | 59 |  |  |  | 167.4574 |  |
|  | TCC (M1) | 27 | 17 | 05 | 297° 17' 07" |  |  | 90.9032 |  |
|  |  | 207 | 17 | 14 |  |  |  | 90.9057 |  |
| TCC(M1) | M2 | 0 | 0 | 0 | 238° 14' 48" | 238° 14' 47" | 238° 14' 47.75" | 91.0022 |  |
|  |  | 180 | 0 | 06 |  |  |  | 91.0003 |  |
|  | 2 | 238 | 14 | 48 | 238° 14' 46" |  |  | 90.8996 |  |
|  |  | 58 | 14 | 52 |  |  |  | 90.8993 |  |
| TCC(M1) | M2 | 90 | 0 | 0 | 238° 14' 49" | 238° 14' 48.5" |  | 91.0017 |  |
|  |  | 270 | 0 | 01 |  |  |  | 91.0012 |  |
|  | 2 | 328 | 14 | 49 | 238° 14' 48" |  |  | 90.8996 |  |
|  |  | 148 | 14 | 49 |  |  |  | 90.8999 |  |
| M2 | M1 | 0 | 0 | 0 | 280° 40'29 “ | 280° 40' 29” | 280° 40' 33” | 91.0019 |  |
|  |  | 180 | 0 | 09 |  |  |  | 91.0017 |  |
|  | M3 | 280 | 40 | 29 | 280° 40' 29” |  |  | 88.4978 |  |
|  |  | 100 | 40 | 38 |  |  |  | 88.4994 |  |
| M2 | M1 | 90 | 0 | 0 | 280° 40' 42” | 280° 40' 37” |  | 91.0021 |  |
|  |  | 270 | 0 | 04 |  |  |  | 91.0043 |  |
|  | M3 | 10 | 40 | 42 | 280° 40' 32" |  |  | 88.4981 |  |
|  |  | 190 | 40 | 36 |  |  |  | 88.5001 |  |
| M3 | M2 | 0 | 0 | 0 | 187° 36' 42" | 187° 36' 46.5" | 187° 36' 54.5" | 88.4881 |  |
|  |  | 179 | 59 | 53 |  |  |  | 88.4896 |  |
|  | M4 | 187 | 36 | 42 | 187° 36' 51" |  |  | 49.2845 |  |
|  |  | 7 | 36 | 44 |  |  |  | 49.2845 |  |
| M3 | M2 | 90 | 0 | 0 | 187° 37' 01" | 187° 37' 2.5" |  | 88.4889 |  |
|  |  | 270 | 0 | 01 |  |  |  | 88.4894 |  |
|  | M4 | 277 | 37 | 01 | 187° 37' 04" |  |  | 49.2840 |  |
|  |  | 97 | 37 | 05 |  |  |  | 49.2841 |  |
| M4 | M3 | 0 | 0 | 0 | 289° 29' 57" | 289° 29' 54.5" | 289° 30' 3.5" | 49.2827 |  |
|  |  | 179 | 59 | 57 |  |  |  | 49.2834 |  |
|  | M5 | 289 | 29 | 57 | 289° 29' 52" |  |  | 78.8248 |  |
|  |  | 109 | 29 | 49 |  |  |  | 78.8248 |  |
| M4 | M3 | 90 | 0 | 0 | 289° 30' 09" | 289° 30' 12.5" |  | 49.2828 |  |
|  |  | 269 | 59 | 59 |  |  |  | 49.2831 |  |
|  | M5 | 19 | 30 | 09 | 289° 30' 16" |  |  | 78.8251 |  |
|  |  | 199 | 30 | 15 |  |  |  | 78.8249 |  |
| M5 | 7 | 0 | 0 | 0 | 311° 50' 11" | 311° 50' 11.5" | 311° 50' 13" | 95.6285 |  |
|  |  | 180 | 0 | 09 |  |  |  | 95.6285 |  |
|  | M4 | 311 | 50 | 11 | 311° 50' 12" |  |  | 78.8100 |  |
|  |  | 131 | 50 | 21 |  |  |  | 78.8112 |  |
| M5 | 7 | 90 | 0 | 0 | 311° 50' 14" | 311° 50' 14.5" |  | 96.6294 |  |
|  |  | 270 | 00 | 01 |  |  |  | 96.6285 |  |
|  | M4 | 41 | 50 | 14 | 311° 50' 15" |  |  | 78.8174 |  |
|  |  | 221 | 50 | 16 |  |  |  | 78.8147 |  |
| 7 | M5 | 0 | 0 | 0 | 313° 47' 36" | 313° 47' 31" | 313° 47' 36.75" | 95.6376 |  |
|  |  | 180 | 00 | 07 |  |  |  | 95.6378 |  |
|  | 1 | 313 | 47 | 36 | 313° 47' 26" |  |  | 178.9718 |  |
|  |  | 133 | 47 | 33 |  |  |  | 178.9732 |  |
| 7 | M5 | 90 | 0 | 0 | 313° 47' 45" | 313° 47' 42.5" |  | 95.6371 |  |
|  |  | 270 | 0 | 06 |  |  |  | 95.6316 |  |
|  | 1 | 43 | 47 | 45 | 313° 47' 40" |  |  | 178.9715 |  |
|  |  | 223 | 47 | 48 |  |  |  | 178.9723 |  |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                      Date:2080/12/26

Recorder:Group3                                                     Time:7:00am

Instrument: Total station and Reflector prism

Table:Minor Traverse Computation Sheet

| Line | Length | Point | Horizontal Angle | WCB | Correction in Angle | Corrected Bearing | Corrected Consecutive Co-ordinate |  | Independent Co-ordinate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Latitude | Departure | Latitude | Departure |
| 7-M5 | 95.7611 | 7 | 313° 47' 36.75" | 101° 47' 36.75" |  |  |  |  | 3129013.654 | 792002.2576 |
| M5-M4 | 78.8274 | M5 | 311° 50' 13" | 233° 09' 50.96" | -20.6” | 233° 09' 30.36" | -47.2574 | -63.9349 | 3128994.885 | 792096.1217 |
| M4-M3 | 49.2835 | M4 | 289° 30' 3.5" | 123° 39' 47.46" | -41.2” | 123° 39' 6.26" | -27.3053 | 41.011 | 3128947.598 | 792032.1922 |
| M3-M2 | 88.4938 | M3 | 187° 36' 54.5" | 116° 02' 52.96" | -1’1.8” | 116° 01' 51.78" | -38.8272 | 79.4925 | 3128920.293 | 792073.9875 |
| M2-M1 | 91.0019 | M2 | 280° 40' 33” | 15° 22' 19.96" | -1’ 22.4” | 15° 20' 57.56" | 87.7646 | 24.0615 | 3128881.466 | 792152.6957 |
| M1-2 | 90.8996 | M1 | 238° 14' 47.75" | 73° 37' 7.71" | -1’42.99” | 73° 35' 24.72" | 25.6884 | 87.1718 | 3128969.23 | 792176.7593 |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                            Date:2080/12/26

Recorder:Group3                                                             Time:1:00pm

Instrument: Level Mechine ,Staff and Tripod stand

TABLE: RL Transfer To The Major Traverse points

| S.No | Backsight |  |  | Foresight |  |  | Rise | Fall | Reduce Level | Distance |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom | Top | Middle | Bottom |  |  |  | B.S | F.S |  |
| 1 | 1.089 | 1.040 | 0.990 |  |  |  |  |  | 943.19 | 10 |  | Benchmark |
| 2 | 1.344 | 1.287 | 1.23 | 1.947 | 1.899 | 1.851 |  | 0.859 | 942.331 | 10 | 10 |  |
| 3 | 1.395 | 1.345 | 1.295 | 1.32 | 1.265 | 1.21 | 0.022 |  | 942.353 | 10 | 10 |  |
| 4 | 1.341 | 1.291 | 1.241 | 1.441 | 1.341 | 1.391 | 0.004 |  | 942.357 | 10 | 10 | Station 6 |
| 5 | 1.432 | 1.382 | 1.332 | 1.304 | 1.256 | 1.208 | 0.035 |  | 942.392 | 10 | 10 |  |
| 6 | 1.355 | 1.305 | 1.255 | 1.387 | 1.338 | 1.29 | 0.044 |  | 942.436 | 10 | 10 |  |
| 7 | 1.330 | 1.280 | 1.230 | 1.385 | 1.335 | 1.285 |  | 0.030 | 942.406 | 10 | 10 |  |
| 8 | 0.915 | 0.865 | 0.815 | 1.501 | 1.452 | 1.404 |  | 0.172 | 942.234 | 10 | 10 |  |
| 9 | 1.907 | 1.873 | 1.840 | 1.130 | 1.080 | 1.03 |  | 0.215 | 942.019 | 10 | 10 |  |
| 10 | 1.842 | 1.808 | 1.774 | 0.950 | 0.916 | 0.882 | 0.957 |  | 942.976 | 7 | 10 |  |
| 11 | 2.275 | 2.230 | 2.185 | 0.896 | 0.862 | 0.828 | 0.946 |  | 943.922 | 9 | 7 |  |
| 12 | 2.000 | 1.968 | 1.936 | 0.720 | 0.675 | 0.63 | 1.555 |  | 945.477 | 7 | 9 |  |
| 13 | 1.603 | 1.580 | 1.556 | 1.027 | 0.993 | 0.96 | 0.975 |  | 946.452 | 4.96 | 7 |  |
| 14 | 1.192 | 1.141 | 1.092 | 1.112 | 1.093 | 1.066 | 0.487 |  | 946.939 | 10 | 4.96 | Station 5 |
| 15 | 1.037 | 0.990 | 0.943 | 1.627 | 1.579 | 1.53 |  | 0.438 | 946.501 | 10 | 10 |  |
| 16 | 1.135 | 1.130 | 1.125 | 1.587 | 1.537 | 1.487 |  | 0.547 | 945.954 | 10 | 10 |  |
| 17 | 1.327 | 1.227 | 1.127 | 1.143 | 1.377 | 1.327 |  | 0.247 | 945.707 | 10 | 10 |  |
| 18 | 1.499 | 1.449 | 1.399 | 2.032 | 1.982 | 1.932 |  | 0.755 | 944.952 | 10 | 10 |  |
| 19 | 1.665 | 1.615 | 1.565 | 1.502 | 1.454 | 1.406 |  | 0.005 | 944.947 | 10 | 10 |  |
| 20 | 1.505 | 1.455 | 1.405 | 1.702 | 1.650 | 1.600 |  | 0.035 | 944.912 | 10 | 10 | Station 4 |
| 21 | 1.7 | 1.651 | 1.602 | 0.911 | 0.863 | 0.815 | 0.592 |  | 945.504 | 10 | 10 |  |
| 22 | 1.611 | 2 | 1.513 | 1.660 | 1.610 | 1.560 | 0.041 |  | 945.545 | 10 | 10 |  |
| 23 | 1.573 | 1.523 | 1.473 | 1.335 | 1.286 | 1.240 | 0.276 |  | 945.821 | 10 | 10 |  |
| 24 | 1.651 | 1.608 | 1.565 | 1.4 | 1.350 | 1.3 | 0.173 |  | 945.994 | 8.6 | 10 |  |
| 25 | 1.266 | 1.216 | 1.166 | 1.245 | 1.204 | 1.163 | 0.404 |  | 946.398 | 10 | 8.6 | Station 3 |
| 26 | 1.485 | 1.435 | 1.385 | 1.147 | 1.096 | 1.047 | 0.120 |  | 946.518 | 10 | 10 |  |
| 27 | 1.52 | 1.47 | 1.42 | 1.125 | 1.075 | 1.025 | 0.360 |  | 946.878 | 10 | 10 |  |
| 28 | 1.505 | 1.457 | 1.41 | 1.165 | 1.118 | 1.07 | 0.352 |  | 947.230 | 10 | 10 |  |
| 29 | 1.559 | 1.51 | 1.461 | 1.109 | 1.059 | 1.009 | 0.398 |  | 947.628 | 10 | 10 |  |
| 30 | 1.104 | 1.654 | 1.604 | 1.110 | 1.061 | 1.011 | 0.449 |  | 948.077 | 10 | 10 |  |
| 31 | 1.511 | 1.476 | 1.442 | 1.190 | 1.144 | 1.098 | 0.510 |  | 948.587 | 6 | 10 | Station 2 |
| 32 | 1.585 | 1.537 | 1.49 | 1.301 | 1.270 | 1.239 | 0.206 |  | 948.793 | 10 | 6 |  |
| 33 | 1.593 | 1.544 | 1.495 | 1.269 | 1.220 | 1.171 | 0.317 |  | 949.110 | 10 | 10 |  |
| 34 | 1.547 | 1.499 | 1.451 | 1.230 | 1.181 | 1.132 | 0.363 |  | 949.473 | 10 | 10 |  |
| 35 | 1.591 | 1.542 | 1.493 | 1.233 | 1.183 | 1.134 | 0.316 |  | 949.789 | 10 | 10 |  |
| 36 | 1.586 | 1.537 | 1.487 | 1.269 | 1.219 | 1.170 | 0.323 |  | 950.112 | 10 | 10 |  |
| 37 | 1.568 | 1.519 | 1.47 | 1.194 | 1.145 | 1.096 | 0.392 |  | 950.504 | 10 | 10 |  |
| 38 | 1.695 | 1.646 | 1.598 | 1.301 | 1.252 | 1.204 | 0.267 |  | 950.771 | 10 | 10 |  |
| 39 | 1.656 | 1.607 | 1.558 | 1.245 | 1.196 | 1.146 | 0.450 |  | 951.221 | 10 | 10 |  |
| 40 | 1.617 | 1.568 | 1.579 | 1.172 | 1.123 | 1.074 | 0.484 |  | 951.705 | 10 | 10 |  |
| 41 | 1.832 | 1.783 | 1.734 | 1.727 | 1.678 | 1.623 |  | 0.110 | 951.595 | 10 | 10 | Station 1 |
| 42 | 1.433 | 1.383 | 1.333 | 1.352 | 1.302 | 1.252 | 0.481 |  | 952.076 | 10 | 10 |  |
| 43 | 1.484 | 1.434 | 1.385 | 1.388 | 1.340 | 1.293 | 0.043 |  | 952.119 | 10 | 10 |  |
| 44 | 1.454 | 1.405 | 1.356 | 1.404 | 1.355 | 1.305 | 0.079 |  | 952.198 | 10 | 10 |  |
| 45 | 1.453 | 1.405 | 1.357 | 1.307 | 1.257 | 1.208 | 0.148 |  | 952.346 | 10 | 10 |  |
| 46 | 1.475 | 1.426 | 1.378 | 1.373 | 1.322 | 1.272 | 0.083 |  | 952.429 | 10 | 10 |  |
| 47 | 1.705 | 1.655 | 1.604 | 1.419 | 1.370 | 1.321 | 0.056 |  | 952.485 | 10 | 10 |  |
| 48 | 1.509 | 1.460 | 1.411 | 1.213 | 1.164 | 1.115 | 0.491 |  | 952.976 | 10 | 10 |  |
| 49 | 1.405 | 1.37 | 1.335 | 1.436 | 1.387 | 1.339 | 0.073 |  | 953.049 | 7.65 | 10 |  |
| 50 | 1.200 | 1.177 | 1.155 | 1.354 | 1.315 | 1.276 | 0.055 |  | 953.104 | 4.45 | 7.65 |  |
| 51 |  |  |  | 1.464 | 1.442 | 1.419 |  | 0.265 | 952.839 |  | 4.45 | Station 7 |
|  |  |  |  |  |  |  |  |  |  | 474.66 |  |  |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group 3                                                                    Date:2080/12/27

Recorder:Group 3                                                                     Time:7:00am

Instrument: Level Mechine ,Staff and Tripod stand

Table:Fly Leveling

| S.NO | Backsight |  |  | Foresight |  |  | RISE | FALL | Reduce Level | Distance |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TOP | Middle | Bottom | TOP | Middle | Bottom |  |  |  | BS | FS |  |
| 1 | 1.335 | 1.287 | 1.239 |  |  |  |  |  | 952.839 | 10 |  | Station 7 |
| 2 | 1.51 | 1.41 | 1.31 | 1.774 | 1.724 | 1.674 |  | 0.437 | 952.402 | 20 | 10 |  |
| 3 | 1.4 | 1.3 | 1.2 | 1.458 | 1.359 | 1.261 | 0.051 |  | 952.453 | 20 | 20 |  |
| 4 | 1.19 | 1.092 | 0.995 | 1.432 | 1.332 | 1.232 |  | 0.032 | 952.421 | 20 | 20 |  |
| 5 | 1.18 | 1.084 | 0.99 | 1.452 | 1.352 | 1.252 |  | 0.26 | 952.161 | 20 | 20 |  |
| 6 | 0.917 | 0.817 | 0.717 | 1.670 | 1.570 | 1.470 |  | 0.486 | 951.675 | 20 | 20 |  |
| 7 | 1.052 | 0.973 | 0.892 | 1.820 | 1.720 | 1.620 |  | 0.903 | 950.772 | 20 | 20 |  |
| 8 | 1.056 | 0.960 | 0.863 | 1.567 | 1.489 | 1.411 |  | 0.516 | 950.256 | 16 | 20 |  |
| 9 | 1.065 | 0.965 | 0.865 | 1.760 | 1.658 | 1.556 |  | 0.698 | 949.558 | 20 | 16 |  |
| 10 | 0.979 | 0.881 | 0.784 | 1.773 | 1.676 | 1.580 |  | 0.711 | 948.847 | 20 | 20 |  |
| 11 | 0.921 | 0.824 | 0.726 | 1.886 | 1.786 | 1.686 |  | 0.905 | 947.942 | 20 | 20 |  |
| 12 | 0.924 | 0.893 | 0.791 | 1.764 | 1.664 | 1.563 |  | 0.84 | 947.102 | 10 | 20 |  |
| 13 | 1.269 | 1.240 | 1.211 | 1.716 | 1.622 | 1.528 |  | 0.729 | 946.373 | 10 | 10 |  |
| 14 | 0.931 | 0.831 | 0.733 | 1.190 | 1.164 | 1.138 | 0.076 |  | 946.449 | 8 | 10 |  |
| 15 | 1.170 | 1.07 | 0.970 | 1.572 | 1.472 | 1.372 |  | 0.641 | 945.808 | 10 | 8 |  |
| 16 | 1.127 | 1.029 | 0.931 | 1.675 | 1.575 | 1.475 |  | 0.505 | 945.303 | 10 | 10 |  |
| 17 | 1.455 | 1.355 | 1.255 | 1.378 | 1.278 | 1.178 |  | 0.249 | 945.054 | 12.5 | 10 |  |
| 18 | 1.615 | 1.525 | 1.435 | 1.306 | 1.220 | 1.134 | 0.135 |  | 945.189 | 17.5 | 12.5 |  |
| 19 | 1.170 | 1.118 | 1.066 | 1.364 | 1.277 | 1.190 | 0.248 |  | 945.437 | 10 | 17.5 |  |
| 20 | 1.081 | 1.052 | 1.023 | 1.796 | 1.742 | 1.693 |  | 0.624 | 944.813 | 6 | 10 |  |
| 21 | 1.015 | 0.986 | 0.957 | 1.737 | 1.707 | 1.677 |  | 0.655 | 944.158 | 6 | 6 |  |
| 22 | 0.911 | 0.881 | 0.851 | 1.777 | 1.750 | 1.723 |  | 0.764 | 943.394 | 6 | 6 |  |
| 23 | 0.760 | 0.722 | 0.684 | 1.769 | 1.739 | 1.709 |  | 0.858 | 942.536 | 8 | 6 |  |
| 24 | 2.002 | 1.902 | 1.802 | 1.696 | 1.654 | 1.612 |  | 0.932 | 941.604 | 20 | 8 |  |
| 25 | 1.545 | 1.445 | 1.345 | 1.352 | 1.253 | 1.154 | 0.649 |  | 942.253 | 20 | 20 |  |
| 26 | 1.370 | 1.3 | 1.230 | 1.532 | 1.435 | 1.338 | 0.01 |  | 942.263 | 14.2 | 20 |  |
| 27 | 1.360 | 1.285 | 1.210 | 1.335 | 1.265 | 1.195 | 0.035 |  | 942.298 | 15 | 14.2 |  |
| 28 | 1.186 | 1.132 | 1.079 | 1.262 | 1.190 | 1.118 | 0.095 |  | 942.393 | 11 | 15 |  |
| 29 | 1.909 | 1.858 | 1.808 | 1.290 | 1.231 | 1.172 |  | 0.099 | 942.294 | 10.3 | 11 |  |
| 30 |  |  |  | 1.032 | 0.980 | 0.929 | 0.878 |  | 943.172 |  | 10.3 | Known Benchmark |
|  |  | 33.22 |  |  | 42.884 |  | 2.177 | 11.844 |  | 410.50 |  |  |

Arithmetic Check:     Total Loop Distance= 949.32+821

∑BS-∑FS=∑Rise-∑Fall=Last RL-First RL Permissible error=25√k =33.26mm      33.22-42.88= 2.177 -11.844=943.172- 952.839   Observed error = 18 mm

-9.664=-9.664=-9.664

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group 3                                                                    Date:2080/12/27

Recorder:Group 3                                                                     Time:1:00mm

Instrument: Total station and prism set

Table:Detailing Coordinates

| S.N | Easting | Northing | Elevation | Code |
| --- | --- | --- | --- | --- |
| 6 | 3128893.522 | 791916.1236 | 942.367 | ST6 |
| 7 | 3129013.654 | 792002.2576 | 952.839 | ST7 |
| 8 | 3129013.673 | 792002.2713 | 953.0106 |  |
| 9 | 3128913.531 | 791901.273 | 942.2553 | FC |
| 10 | 3128965.215 | 791950.8134 | 942.2282 | FC |
| 11 | 3128899.114 | 792020.8067 | 942.1642 | FC |
| 12 | 3128847.144 | 791970.807 | 942.2323 | FC |
| 13 | 3128897.693 | 792026.3715 | 942.209 | TP |
| 20 | 3128946.008 | 791993.397 | 945.4414 | PS |
| 21 | 3128940.804 | 791998.9713 | 945.4192 | PS |
| 24 | 3128933.914 | 791912.7267 | 943.2304 | SH |
| 25 | 3128943.874 | 791919.3015 | 943.9561 | SH |
| 26 | 3128958.009 | 791925.8677 | 945.1854 | SH |
| 27 | 3128921.686 | 791899.0673 | 942.9669 | TR |
| 28 | 3128948.184 | 791907.7611 | 944.3896 | TR |
| 29 | 3128950.16 | 791911.223 | 944.5221 | TR |
| 30 | 3128955.357 | 791918.7879 | 944.814 | TR |
| 31 | 3128960.207 | 791925.0479 | 945.317 | TR |
| 32 | 3128898.088 | 791900.5923 | 942.9331 | TR |
| 33 | 3128891.302 | 791906.0919 | 942.4931 | TR |
| 34 | 3128887.718 | 791911.7575 | 942.5674 | TR |
| 35 | 3128862.185 | 791938.5851 | 942.6553 | TR |
| 36 | 3128892.675 | 791906.0278 | 942.2595 | CR |
| 37 | 3128895.437 | 791903.533 | 942.4711 | CR |
| 38 | 3128916.527 | 791885.0502 | 942.9967 | PR |
| 39 | 3128931.814 | 791883.2471 | 943.0646 | R |
| 40 | 3128937.252 | 791876.6694 | 943.0822 | R |
| 41 | 3128948.983 | 791898.7711 | 943.3441 | R |
| 42 | 3128953.339 | 791896.3328 | 943.4149 | R |
| 43 | 3128971.608 | 791933.0607 | 945.0495 | R |
| 44 | 3128976.305 | 791932.3407 | 945.1803 | R |
| 45 | 3128981.247 | 791969.4468 | 948.9294 | R |
| 46 | 3128986.602 | 791971.8375 | 949.2569 | R |
| 47 | 3128967.106 | 791992.4792 | 949.4971 | TT |
| 48 | 3128965.475 | 791994.4932 | 949.4799 | TT |
| 49 | 3128966.61 | 791995.5608 | 949.4473 | TT |
| 50 | 3128968.344 | 791993.4912 | 949.5014 | TT |
| 51 | 3128897.697 | 792030.0599 | 942.5547 | TR |
| 52 | 3128838.471 | 791978.2545 | 940.9308 | R1 |
| 53 | 3128835.34 | 791982.2749 | 940.9262 | R1 |
| 69 | 3128969.066 | 791962.5167 | 942.747 | PF |
| 70 | 3128969.916 | 791963.2143 | 943.2858 | PF |
| 71 | 3128970.621 | 791963.9687 | 943.8208 | PF |
| 72 | 3128971.376 | 791964.6086 | 944.3497 | PF |
| 73 | 3128972.199 | 791965.4458 | 944.8674 | PF |
| 74 | 3128972.844 | 791966.0642 | 945.4664 | PF |
| 75 | 3128973.592 | 791966.8231 | 945.9822 | PF |
| 76 | 3128974.25 | 791967.4179 | 946.5315 | PF |
| 77 | 3128975.049 | 791968.2105 | 947.0687 | PF |
| 78 | 3128975.684 | 791968.796 | 947.5998 | PF |
| 79 | 3128976.443 | 791969.6003 | 948.1375 | PF |
| 80 | 3128977.386 | 791970.4398 | 948.8851 | PF |
| 81 | 3128970.255 | 791977.9208 | 948.8438 | PCH |
| 82 | 3128969.258 | 791976.923 | 948.143 | PCH |
| 83 | 3128967.965 | 791976.9027 | 947.6256 | PCH |
| 84 | 3128966.475 | 791976.889 | 947.0502 | PCH |
| 85 | 3128965.33 | 791978.1602 | 947.0315 | PCH |
| 86 | 3128965.368 | 791979.5724 | 947.5851 | PCH |
| 87 | 3128965.478 | 791980.9982 | 948.0713 | PCH |
| 88 | 3128966.471 | 791981.8481 | 948.8584 | PCH |
| 89 | 3128960.179 | 791988.4016 | 948.8617 | PCH2 |
| 90 | 3128956.455 | 791987.4298 | 946.5243 | PCH2 |
| 91 | 3128955.329 | 791988.5654 | 946.4863 | PCH2 |
| 92 | 3128955.368 | 791991.4907 | 947.5434 | PCH2 |
| 93 | 3128959.214 | 791987.3751 | 947.5522 | PCH2 |
| 94 | 3128956.482 | 791992.4604 | 948.8859 | PCH2 |
| 95 | 3128948.584 | 791991.5012 | 945.4463 | PS |
| 96 | 3128938.359 | 792002.1104 | 945.4248 | PS |
| 97 | 3128940.183 | 792009.4682 | 948.7933 | PCH3 |
| 98 | 3128939.29 | 792008.4654 | 947.6327 | PCH3 |
| 99 | 3128936.407 | 792008.4404 | 946.5647 | PCH3 |
| 100 | 3128935.259 | 792009.4869 | 946.571 | PCH3 |
| 101 | 3128935.347 | 792012.0519 | 947.08 | PCH3 |
| 102 | 3128936.511 | 792013.3206 | 948.8041 | PCH3 |
| 103 | 3128930.168 | 792019.9512 | 948.8689 | PCH3 |
| 104 | 3128929.156 | 792019.0222 | 948.1624 | PCH3 |
| 105 | 3128926.457 | 792018.9447 | 947.0447 | PCH3 |
| 106 | 3128925.181 | 792020.1651 | 947.0651 | PCH3 |
| 107 | 3128926.417 | 792024.0243 | 948.889 | PCH3 |
| 108 | 3128919.295 | 792031.4987 | 948.8722 | PF |
| 109 | 3128918.318 | 792030.4273 | 948.0823 | PF |
| 110 | 3128917.513 | 792029.7334 | 947.576 | PF |
| 111 | 3128916.88 | 792029.051 | 947.0423 | PF |
| 112 | 3128916.093 | 792028.3468 | 946.556 | PF |
| 113 | 3128915.361 | 792027.6417 | 945.9534 | PF |
| 114 | 3128914.649 | 792026.9726 | 945.4219 | PF |
| 115 | 3128913.919 | 792026.3118 | 944.8613 | PF |
| 116 | 3128913.279 | 792025.6048 | 944.2853 | PF |
| 117 | 3128912.509 | 792024.9097 | 943.7741 | PF |
| 118 | 3128912.315 | 792024.1601 | 943.2224 | PF |
| 119 | 3128911.068 | 792023.5566 | 942.7263 | PF |
| 120 | 3128881.404 | 792021.1104 | 941.7238 | R1 |
| 121 | 3128876.433 | 792023.0116 | 941.6356 | R1 |
| 122 | 3128882.213 | 792034.2464 | 942.2225 | R1 |
| 123 | 3128876.571 | 792033.4563 | 942.2201 | R1 |
| 124 | 3128869.916 | 792046.9184 | 943.3065 | R1 |
| 125 | 3128925.711 | 791902.0061 | 943.2098 | SL |
| 126 | 3128955.195 | 791897.0537 | 943.8621 | SL |
| 127 | 3128847.835 | 791995.3091 | 941.2671 | SL |
| 128 | 3128875.198 | 792022.3387 | 941.9936 | SL |
| 5 | 3128871.175 | 792057.2054 | 946.32 | ST5 |
| ST6 | 3128893.522 | 791916.1236 | 942.367 | ST06 |
| ST7 | 3128893.526 | 791916.1054 | 941.9208 | DB |
| 133 | 3128888.264 | 792054.7864 | 946.993 | DB |
| 134 | 3128886.118 | 792066.4029 | 946.9886 | DB |
| 135 | 3128888.175 | 792066.381 | 946.995 | DB |
| 136 | 3128886.746 | 792072.773 | 946.9696 | DB |
| 137 | 3128884.559 | 792072.2425 | 946.9459 | DB |
| 138 | 3128882.622 | 792083.4235 | 946.9443 | DB |
| 139 | 3128909.773 | 792036.7445 | 947.7329 | GP |
| 140 | 3128908.354 | 792034.2012 | 947.8689 | GP |
| 141 | 3128912.986 | 792031.6043 | 947.7459 | GP |
| 142 | 3128914.489 | 792034.1513 | 947.8465 | GP |
| 143 | 3128901.189 | 792046.9224 | 947.4186 | TR |
| 144 | 3128904.544 | 792049.7746 | 947.1691 | TR |
| 145 | 3128916.028 | 792053.4659 | 947.6707 | TR |
| 146 | 3128860.563 | 792021.4271 | 941.3314 | TR |
| 147 | 3128865.556 | 792025.0771 | 940.6476 | TR |
| 148 | 3128866.243 | 792033.9304 | 940.5796 | TR |
| 149 | 3128868.478 | 792035.8586 | 940.9172 | TR |
| 150 | 3128850.832 | 792036.4732 | 939.6613 | TR |
| 151 | 3128859.755 | 792027.5365 | 939.5799 | VBC |
| 152 | 3128852.928 | 792033.7273 | 939.5266 | VBC |
| 153 | 3128847.124 | 792014.4937 | 939.5759 | VBC |
| 154 | 3128840.755 | 792020.7492 | 939.5932 | VBC |
| 155 | 3128843.175 | 792010.1164 | 939.6136 | BBC |
| 156 | 3128822.755 | 792030.5031 | 939.5957 | BBC |
| 157 | 3128831.545 | 791998.3621 | 939.6062 | BBC |
| 158 | 3128811.162 | 792018.9232 | 939.587 | BBC |
| 159 | 3128830.257 | 791997.9454 | 939.7171 | TR |
| 160 | 3128804.191 | 792019.457 | 939.5474 | TR |
| 161 | 3128829.669 | 792038.6465 | 939.5706 | TT |
| 5000 | 3128871.175 | 792057.2054 | 946.32 | ST5 |
| 5001 | 3128828.537 | 792039.5901 | 939.5319 | TT |
| 5002 | 3128831.674 | 792041.1371 | 939.5383 | TT |
| 5003 | 3128828.248 | 792043.8804 | 939.5116 | TT |
| 5004 | 3128826.35 | 792041.4646 | 939.5288 | TT |
| 5005 | 3128825.158 | 792042.4286 | 939.5812 | TT |
| 5006 | 3128843.003 | 792046.9558 | 941.3554 | TR |
| 5007 | 3128857.693 | 792047.3776 | 943.6999 | SL |
| 5008 | 3128826.88 | 792064.6796 | 945.2297 | SL |
| 5009 | 3128859.622 | 792047.373 | 943.2596 | R1 |
| 5010 | 3128861.614 | 792051.5634 | 943.3415 | R1 |
| 5011 | 3128832.664 | 792061.191 | 944.7953 | R1 |
| 5012 | 3128836.601 | 792064.3129 | 944.7096 | R1 |
| 5013 | 3128837.859 | 792068.9981 | 945.3059 | PV |
| 5014 | 3128875.896 | 792080.9169 | 946.0459 | PV |
| 5015 | 3128873.663 | 792089.6209 | 945.7539 | PV |
| 5016 | 3128887.725 | 792092.1661 | 946.0933 | PV |
| 5017 | 3128879.369 | 792092.0554 | 945.7933 | PV |
| 5018 | 3128876.856 | 792103.8171 | 945.7271 | PV |
| 5019 | 3128876.65 | 792110.5345 | 946.7261 | TO |
| 5020 | 3128890.905 | 791904.2699 | 942.0598 | CR |
| 5021 | 3128852.711 | 792080.2988 | 945.7697 | SL |
| 5022 | 3128872.282 | 792105.4623 | 945.7608 | TR |
| 5023 | 3128861.584 | 792119.6271 | 945.5683 | TR |
| 5024 | 3128843.402 | 792104.9543 | 945.2473 | TT |
| 5025 | 3128843.182 | 792106.1894 | 945.1898 | TT |
| 5026 | 3128840.908 | 792104.7071 | 945.1761 | TT |
| 5027 | 3128840.761 | 792106.032 | 945.1809 | TT |
| 5028 | 3128831.847 | 792108.2228 | 944.648 | R1 |
| 5029 | 3128827.093 | 792108.2959 | 944.6475 | R1 |
| 5030 | 3128826.019 | 792133.9614 | 944.9478 | SL |
| 5031 | 3128837.389 | 792137.3657 | 944.5884 | MSC |
| 5032 | 3128864.816 | 792138.9866 | 947.2039 | MSC |
| 5033 | 3128907.997 | 792140.9593 | 949.3026 | O1 |
| 8000 | 3128907.996 | 792140.9595 | 949.3044 | O1 |
| 8001 | 3128902.534 | 792140.0371 | 949.4791 | CB |
| 8002 | 3128916.011 | 792142.7607 | 949.4745 | CB |
| 8003 | 3128911.16 | 792135.3657 | 949.5873 | GB |
| 8004 | 3128920.568 | 792137.3676 | 949.545 | GB |
| 8005 | 3128915.739 | 792113.1688 | 949.6042 | GB |
| 8006 | 3128913.321 | 792108.7584 | 949.5924 | GB |
| 8007 | 3128916.44 | 792094.8017 | 949.6997 | GB |
| 8008 | 3128905.57 | 792101.3612 | 949.2277 | PV |
| 8009 | 3128910.142 | 792107.7803 | 949.8391 | TR |
| 8010 | 3128906.653 | 792128.1784 | 949.8256 | TR |
| 8011 | 3128920.251 | 792143.6663 | 949.4647 | CB |
| 8012 | 3128939.615 | 792147.6924 | 949.4479 | CB |
| 8013 | 3128918.102 | 792137.5378 | 949.5187 | PV |
| 8014 | 3128917.498 | 792142.6639 | 949.521 | PV |
| 8015 | 3128950.132 | 792149.4637 | 949.687 | AB |
| 8016 | 3128952.945 | 792149.8351 | 949.6524 | AB |
| 8017 | 3128953.471 | 792147.2528 | 949.5971 | AB |
| 8018 | 3128950.609 | 792146.582 | 949.6816 | AB |
| 8019 | 3128952.778 | 792136.1342 | 949.6404 | AB |
| 8020 | 3128954.718 | 792126.8352 | 949.6064 | AB |
| 8021 | 3128963.396 | 792128.8694 | 949.6127 | AB |
| 8022 | 3128950.001 | 792128.0564 | 949.5468 | GB |
| 8023 | 3128946.039 | 792138.7201 | 949.8512 | TR |
| 8024 | 3128920.232 | 792142.7094 | 949.5787 | SL |
| 8025 | 3128880.181 | 792112.2905 | 947.2299 | TO |
| 8026 | 3128882.343 | 792106.6492 | 947.3166 | TO |
| 8027 | 3128883.324 | 792105.2338 | 947.4806 | PV |
| 4000 | 3128841.283 | 792161.3571 | 944.912 | ST4 |
| 400 | 3128814.283 | 792161.3571 | 944.912 | ST4 |
| 401 | 3128831.696 | 792157.8501 | 945.4861 | MSC |
| 402 | 3128831.501 | 792161.5094 | 945.4834 | MSC |
| 403 | 3128834.92 | 792166.3578 | 947.0801 | MSC |
| 404 | 3128837.789 | 792167.4412 | 947.6461 | MSC |
| 405 | 3128837.093 | 792170.2459 | 947.6676 | MSC |
| 406 | 3128841.666 | 792177.8211 | 947.9043 | MSC |
| 407 | 3128843.646 | 792181.4324 | 947.8477 | CT |
| 408 | 3128844.26 | 792178.5191 | 947.8753 | CT |
| 409 | 3128846.137 | 792175.6548 | 948.487 | CT |
| 410 | 3128841.395 | 792168.5551 | 948.4674 | MSC |
| 411 | 3128793.18 | 792171.9937 | 943.7297 | PH |
| 412 | 3128797.362 | 792180.9841 | 943.6401 | PH |
| 413 | 3128778.775 | 792178.7627 | 943.6917 | PH |
| 414 | 3128820.311 | 792162.233 | 944.9239 | R1 |
| 415 | 3128825.052 | 792163.2177 | 944.9163 | R1 |
| 416 | 3128825.4 | 792180.9632 | 945.2104 | R1 |
| 417 | 3128820.65 | 792181.5695 | 945.1789 | R1 |
| 418 | 3128818.669 | 792173.0769 | 945.3476 | SL |
| 419 | 3128815.43 | 792169.3637 | 945.0922 | TR |
| 420 | 3128816.168 | 792163.3276 | 945.2781 | TR |
| 3000 | 3128864.873 | 792243.2207 | 946.398 | ST3 |
| 3001 | 3128840.321 | 792206.237 | 945.8076 | PV |
| 3002 | 3128847.478 | 792217.468 | 945.8472 | PV |
| 3003 | 3128859.42 | 792235.379 | 946.1149 | R1 |
| 3004 | 3128857.394 | 792239.2778 | 946.1924 | R1 |
| 3005 | 3128857.066 | 792240.4401 | 946.4472 | SL |
| 3006 | 3128884.34 | 792240.5811 | 946.5152 | PV |
| 3007 | 3128890.732 | 792241.3909 | 946.6859 | PV |
| 3008 | 3128882.282 | 792235.8814 | 947.9474 | SH |
| 3009 | 3128875.376 | 792230.3971 | 947.9396 | MS |
| 3010 | 3128862.629 | 792227.0231 | 947.8181 | MS |
| 3011 | 3128862.54 | 792232.6102 | 946.7066 | EP |
| 3012 | 3128887.626 | 792216.8332 | 947.8035 | PV |
| 3013 | 3128842.191 | 792201.7078 | 947.8912 | CP |
| 3014 | 3128845.615 | 792190.4194 | 947.9473 | CP |
| 3015 | 3128835.321 | 792190.549 | 947.8834 | CP |
| 3016 | 3128840.651 | 792195.7473 | 948.592 | TR |
| 3017 | 3128855.661 | 792184.2422 | 947.9595 | CT |
| 3018 | 3128897.22 | 792203.8137 | 948.2707 | MW |
| 3019 | 3128894.603 | 792208.5265 | 948.0469 | PV |
| 3020 | 3128885.393 | 792246.5894 | 946.7646 | SL |
| 3021 | 3128891.712 | 792230.1205 | 948.0586 | MW |
| 3022 | 3128910.632 | 792244.9221 | 947.3941 | EP |
| 3023 | 3128910.68 | 792233.9808 | 948.3766 | MW |
| 3024 | 3128913.351 | 792244.3008 | 947.2262 | MR |
| 3025 | 3128919.846 | 792245.6791 | 947.1792 | MR |
| 3026 | 3128910.18 | 792251.4528 | 947.3566 | SL |
| 3027 | 3128892.937 | 792234.5096 | 948.1168 | TR |
| 3028 | 3128897.744 | 792239.6283 | 947.6592 | SH |
| 3029 | 3128907.934 | 792239.4811 | 948.154 | SH |
| 3030 | 3128926.058 | 792243.0688 | 948.0519 | O2 |
| 3031 | 3128926.143 | 792239.0428 | 948.0796 | MW |
| 3032 | 3128933.944 | 792240.6923 | 948.0932 | MW |
| 3033 | 3128937.688 | 792242.3865 | 948.3051 | MW |
| 3034 | 3128944.741 | 792243.831 | 948.315 | MW |
| 3035 | 3128930.85 | 792253.9217 | 947.3773 | PV |
| 3036 | 3128878.729 | 792237.7293 | 945.9145 | SW |
| 3037 | 3128874.128 | 792246.2745 | 945.8732 | SW |
| 3038 | 3128859.264 | 792232.4503 | 945.7686 | SW |
| 3039 | 3128852.041 | 792238.2917 | 945.8123 | SW |
| 9000 | 3128926.058 | 792243.0688 | 948.0519 | O2 |
| 9001 | 3128920.222 | 792267.1314 | 947.0015 | AW |
| 9002 | 3128912.469 | 792300.5263 | 946.7171 | AW |
| 9003 | 3128935.322 | 792261.8374 | 947.9884 | KP |
| 9004 | 3128939.831 | 792270.5059 | 947.6281 | KPC |
| 9005 | 3128916.297 | 792207.4852 | 948.3833 | MW |
| 9006 | 3128918.442 | 792221.2335 | 948.0121 | SL |
| 9007 | 3128939.155 | 792237.6156 | 948.1569 | MW |
| 9008 | 3128933.82 | 792246.5568 | 947.8615 | TR |
| 9009 | 3128945.376 | 792248.8495 | 948.3213 | TR |
| 2000 | 3128994.901 | 792264.777 | 948.587 | ST2 |
| 2001 | 3128947.396 | 792246.954 | 948.1631 | O3 |
| 2002 | 3129003.238 | 792291.62 | 948.6835 | OG |
| 2003 | 3129007.454 | 792289.3499 | 948.7095 | OG |
| 2004 | 3128999.833 | 792287.1991 | 948.9945 | SL |
| 2005 | 3128991.035 | 792268.9398 | 948.4653 | R1 |
| 2006 | 3128997.269 | 792270.3609 | 948.5428 | R1 |
| 2007 | 3129003.904 | 792267.5851 | 948.7109 | TR |
| 2008 | 3129002.982 | 792259.1565 | 949.091 | TR |
| 2009 | 3129001.703 | 792257.2188 | 948.7929 | R1 |
| 2010 | 3128998.028 | 792253.7271 | 948.7042 | R1 |
| 2011 | 3128983.583 | 792249.5311 | 948.4353 | R1 |
| 2012 | 3128976.129 | 792248.3216 | 948.3519 | R1 |
| 2013 | 3128964.174 | 792254.8851 | 947.9089 | R1 |
| 2014 | 3128964.537 | 792259.6692 | 947.94 | R1 |
| 2015 | 3128979.381 | 792263.3274 | 948.4511 | SL |
| 2016 | 3128984.686 | 792249.0234 | 948.8573 | TR |
| 2017 | 3128983.251 | 792266.5388 | 948.5882 | EP |
| 2018 | 3128952.322 | 792241.2301 | 947.7973 | MW |
| 2019 | 3128959.929 | 792243.03 | 947.9086 | MW |
| 2020 | 3128961.122 | 792243.9626 | 948.0806 | MW |
| 2021 | 3128967.899 | 792245.4184 | 948.059 | MW |
| 2022 | 3128968.864 | 792241.1102 | 948.0808 | MW |
| 2023 | 3128971.914 | 792241.6918 | 947.9258 | MW |
| 2024 | 3128976.484 | 792220.2379 | 947.9907 | MW |
| 2025 | 3128973.241 | 792194.371 | 948.442 | MW |
| 2026 | 3128976.546 | 792217.1065 | 948.6387 | MW |
| 2027 | 3128920.518 | 792254.6062 | 947.0347 | KP |
| 2028 | 3128943.081 | 792258.2174 | 947.7969 | KP |
| 2029 | 3128944.804 | 792264.1329 | 947.5728 | EL |
| 2030 | 3128950.847 | 792257.5644 | 947.6623 | EP |
| 2031 | 3128954.84 | 792258.2712 | 947.764 | PV |
| 2032 | 3128964.409 | 792266.9987 | 947.5926 | EL |
| 2033 | 3128958.778 | 792294.5727 | 947.6337 | EL |
| 2034 | 3128984.451 | 792230.5622 | 948.9796 | SL |
| 2035 | 3128985.672 | 792229.5372 | 949.0723 | WW |
| 2036 | 3128998.203 | 792233.7049 | 949.2128 | WW |
| 2037 | 3128996.291 | 792242.703 | 949.1397 | WW |
| 2038 | 3129003.329 | 792244.3364 | 949.2216 | WW |
| 2039 | 3129053.902 | 792208.301 | 949.928 | CW |
| 2040 | 3129032.781 | 792242.0656 | 949.7317 | BR |
| 2041 | 3129038.802 | 792238.8203 | 949.606 | TR |
| 10000 | 3128894.739 | 792246.9537 | 948.1614 | O3 |
| 20000 | 3129026.665 | 792227.3554 | 949.4539 | O4 |
| 20001 | 3129004.812 | 792232.672 | 949.0747 | WW |
| 20002 | 3129012.374 | 792200.0156 | 949.4426 | WW |
| 20003 | 3129027.969 | 792200.9178 | 949.504 | RC |
| 20004 | 3129035.339 | 792202.3141 | 949.6371 | CW |
| 20005 | 3129054.199 | 792206.2515 | 949.6345 | CW |
| 20006 | 3129022.1 | 792212.4903 | 949.3954 | TR |
| 20007 | 3129017.309 | 792210.4817 | 949.3893 | TR |
| 20008 | 3129015.227 | 792216.5225 | 949.1316 | TR |
| 20009 | 3129029.524 | 792247.5004 | 949.3989 | NG |
| 20010 | 3129026.407 | 792249.3908 | 949.3513 | NG |
| 20011 | 3129054.699 | 792226.2525 | 950.3824 | SL |
| 20012 | 3129070.476 | 792215.4485 | 950.2514 | R1 |
| 20013 | 3129068.786 | 792210.8678 | 950.1522 | R1 |
| 20014 | 3129062.855 | 792209.0586 | 950.1873 | PV |
| 20015 | 3129004.668 | 792235.3183 | 949.2112 | WW |
| 20016 | 3129021.947 | 792245.7781 | 949.5292 | SL |
| 20017 | 3129010.819 | 792197.3421 | 950.0105 | RC |
| 20018 | 3129010.874 | 792197.3302 | 949.5139 | RC |
| 11000 | 3128947.396 | 792246.9537 | 948.1614 | O5 |
| 12000 | 3128969.239 | 792177.585 | 949.6221 | M1 |
| 12001 | 3128986.131 | 792129.12 | 950.1945 | AP |
| 12002 | 3128987.066 | 792154.8357 | 949.0331 | ED |
| 12003 | 3128980.193 | 792164.4638 | 949.2874 | ED |
| 12004 | 3128978.974 | 792171.5402 | 949.1299 | ED |
| 12005 | 3128979.894 | 792189.5088 | 948.8808 | ED |
| 12006 | 3128978.26 | 792126.6564 | 950.2155 | AD |
| 12007 | 3128971.763 | 792149.7507 | 950.1512 | AD |
| 12008 | 3128974.454 | 792142.0653 | 950.2148 | AD |
| 12009 | 3128969.738 | 792159.0889 | 949.6928 | AD |
| 12010 | 3128966.715 | 792158.5622 | 949.6842 | AD |
| 12011 | 3128966.01 | 792163.0906 | 949.6409 | AD |
| 12012 | 3128957.761 | 792165.351 | 949.5942 | PV |
| 12013 | 3128957.044 | 792161.1422 | 949.5614 | AD |
| 12014 | 3128962.372 | 792172.151 | 949.6948 | TCC |
| 12015 | 3128966.532 | 792173.6327 | 950.1663 | TCC |
| 12016 | 3128965.906 | 792177.3641 | 949.6821 | TCC |
| 12017 | 3128971.329 | 792174.2104 | 949.6825 | TCC |
| 12018 | 3128967.704 | 792168.7088 | 949.7918 | TCC |
| 12019 | 3128951.789 | 792165.9871 | 949.6446 | AB |
| 12020 | 3128953.096 | 792161.8357 | 949.6336 | AB |
| 12021 | 3128947.145 | 792165.1756 | 949.641 | AB |
| 12022 | 3128953.041 | 792174.0169 | 948.9248 | TO |
| 12023 | 3128945.929 | 792172.3779 | 948.814 | TO |
| 12024 | 3128950.717 | 792184.7116 | 948.8675 | TO |
| 12025 | 3128953.712 | 792184.8455 | 948.9728 | PV |
| 12026 | 3128952.113 | 792184.7826 | 948.9803 | MW |
| 12027 | 3128929.278 | 792205.0754 | 948.1156 | MW |
| 12028 | 3128928.791 | 792207.0331 | 948.6107 | MW |
| 12029 | 3128953.51 | 792180.3036 | 948.9979 | PV |
| 12030 | 3128955.762 | 792169.8791 | 949.481 | PV |
| 12031 | 3128940.102 | 792170.7578 | 949.6233 | TR |
| 12032 | 3128939.198 | 792164.0374 | 950.0674 | TR |
| 12033 | 3128944.106 | 792167.3982 | 949.2133 | PV |
| 12034 | 3128938.397 | 792159.4301 | 949.4653 | MB |
| 12035 | 3128930.41 | 792162.0766 | 949.459 | MB |
| 12036 | 3128917.084 | 792159.2886 | 949.4583 | MB |
| 12037 | 3128912.664 | 792158.398 | 949.4663 | MB |
| 12038 | 3128914.974 | 792163.04 | 948.9571 | PV |
| 12039 | 3128913.095 | 792162.7782 | 948.9851 | PV |
| 12040 | 3128893.517 | 792154.4954 | 949.4726 | MB |
| 13000 | 3128916.494 | 792161.0982 | 949.5261 | O5 |
| 13001 | 3128943.367 | 792181.5453 | 948.9067 | TO |
| 13002 | 3128933.831 | 792183.4298 | 947.8514 | MW |
| 13003 | 3128923.05 | 792176.623 | 948.2404 | MW |
| 13004 | 3128915.934 | 792175.2942 | 948.2525 | MW |
| 13005 | 3128915.398 | 792180.0479 | 948.1405 | MW |
| 13006 | 3128910.612 | 792201.2015 | 947.9867 | MW |
| 13007 | 3128903.88 | 792199.6173 | 947.82 | MD |
| 13008 | 3128908.889 | 792175.2938 | 948.0417 | MD |
| 13009 | 3128889.781 | 792171.2917 | 947.84 | MD |
| 13010 | 3128888.555 | 792160.4575 | 948.3192 | PV |
| 13011 | 3128886.217 | 792156.2813 | 948.7141 | TR |
| 13012 | 3128912.911 | 792155.1623 | 949.5401 | CB |
| 13013 | 3128917.598 | 792156.0217 | 949.5297 | CB |
| 13014 | 3128914.825 | 792145.7936 | 949.5676 | CB |
| 13015 | 3128919.979 | 792146.9493 | 949.5416 | CB |
| 13016 | 3128923.727 | 792122.8163 | 949.6453 | GB |
| 13017 | 3128927.661 | 792169.2647 | 949.2372 | TR |
| 13018 | 3128921.26 | 792168.2455 | 949.4114 | TR |
| 14000 | 3128881.466 | 792153.4928 | 948.4083 | M2 |
| 14001 | 3128884.889 | 792195.7625 | 947.899 | MD |
| 14002 | 3128882.892 | 792195.8706 | 947.8594 | PV |
| 14003 | 3128879.45 | 792195.3583 | 948.099 | PV |
| 14004 | 3128881.296 | 792200.5754 | 947.7984 | PV |
| 14005 | 3128884.088 | 792200.955 | 947.8059 | PV |
| 14006 | 3128875.342 | 792208.5048 | 947.7807 | TO |
| 14007 | 3128876.923 | 792208.765 | 948.0698 | TO |
| 14008 | 3128874.765 | 792210.4852 | 947.6797 | TO |
| 14009 | 3128866.003 | 792208.9697 | 947.4465 | MS |
| 14010 | 3128862.135 | 792192.0737 | 947.8512 | CSP |
| 14011 | 3128860.976 | 792164.0684 | 947.7878 | CT |
| 14012 | 3128854.82 | 792162.5443 | 947.8767 | PV |
| 14013 | 3128843.533 | 792160.5395 | 947.7078 | CT |
| 14014 | 3128840.852 | 792158.69 | 947.7681 | MSC |
| 14015 | 3128843.007 | 792152.6984 | 947.9107 | PV |
| 14016 | 3128865.65 | 792152.7472 | 947.8768 | MSC |
| 14017 | 3128862.016 | 792152.7739 | 947.9221 | PV |
| 14018 | 3128864.454 | 792153.3614 | 947.9053 | PV |
| 14019 | 3128863.651 | 792155.723 | 947.9251 | PV |
| 14020 | 3128867.957 | 792156.2632 | 948.0162 | PV |
| 14021 | 3128871.735 | 792139.7448 | 947.8272 | PV |
| 14022 | 3128868.855 | 792137.5512 | 947.5846 | PV |
| 14023 | 3128909.947 | 792097.2712 | 949.6577 | PV |
| 14024 | 3128872.584 | 792141.8047 | 947.8345 | PV |
| 14025 | 3128896.288 | 792143.0964 | 949.5281 | CB |
| 14026 | 3128890.836 | 792143.7038 | 948.7252 | PV |
| 14027 | 3128900.965 | 792086.3668 | 946.9907 | DB |
| 14028 | 3128903.527 | 792078.4486 | 946.9802 | DB |
| 14029 | 3128876.512 | 792133.5998 | 948.0048 | TR |
| 14030 | 3128880.416 | 792128.6246 | 947.9929 | TR |
| 14031 | 3128915.03 | 792100.9472 | 949.7069 | GB |
| 14032 | 3128916.747 | 792094.8518 | 949.7571 | GB |
| 14033 | 3128868.318 | 792139.6839 | 947.6703 | MSC |
| 14034 | 3128872.821 | 792153.3652 | 948.598 | TR |
| 15000 | 3128920.293 | 792073.9875 | 949.8655 | M3 |
| 15001 | 3128922.672 | 792072.8677 | 949.7934 | TO |
| 15002 | 3128923.607 | 792068.083 | 949.8094 | TO |
| 15003 | 3128923.631 | 792073.0729 | 949.8248 | TO |
| 15004 | 3128923.111 | 792075.4502 | 949.8106 | TO |
| 15005 | 3128930.249 | 792077.0307 | 949.8477 | TO |
| 15006 | 3128923.044 | 792077.7333 | 949.6956 | GB |
| 15007 | 3128920.875 | 792086.0494 | 949.6917 | GB |
| 15008 | 3128918.954 | 792094.8292 | 949.7187 | GB |
| 15009 | 3128914.3 | 792077.1864 | 949.8095 | PV |
| 15010 | 3128912.395 | 792076.374 | 949.8654 | PV |
| 15011 | 3128912.875 | 792073.8674 | 949.8497 | PV |
| 15012 | 3128906.425 | 792074.9511 | 950.9921 | PV |
| 15013 | 3128906.902 | 792072.746 | 951.0059 | PV |
| 15014 | 3128905.907 | 792077.8146 | 951.0074 | WWS |
| 15015 | 3128902.89 | 792077.2641 | 951.0128 | WWS |
| 15016 | 3128915.004 | 792073.9321 | 949.8272 | PV |
| 15017 | 3128916.014 | 792075.009 | 949.816 | PV |
| 15018 | 3128915.668 | 792067.4608 | 948.9563 | PV |
| 15019 | 3128907.649 | 792058.8955 | 946.9798 | DB |
| 15020 | 3128919.391 | 792069.8156 | 950.2592 | R |
| 15021 | 3128930.894 | 792078.0479 | 949.7919 | PV |
| 15022 | 3128932.214 | 792079.5201 | 949.7821 | GB |
| 16000 | 3128947.598 | 792033.1922 | 950.9926 | M4 |
| 16001 | 3128930.662 | 792069.7885 | 949.8407 | TO |
| 16002 | 3128930.732 | 792088.4103 | 949.7165 | GB |
| 16003 | 3128935.419 | 792089.4251 | 949.73 | GB |
| 16004 | 3128930.768 | 792110.3334 | 949.7128 | GB |
| 16005 | 3128943.844 | 792112.566 | 949.6316 | GB |
| 16006 | 3128944.655 | 792110.7139 | 949.9102 | FSU |
| 16007 | 3128946.896 | 792099.2663 | 949.9619 | FSU |
| 16008 | 3128939.781 | 792104.1941 | 950.3104 | BR |
| 16009 | 3128951.134 | 792067.9794 | 951.4198 | BPP |
| 16010 | 3128953.216 | 792056.2762 | 951.1025 | PV |
| 16011 | 3128954.324 | 792056.5275 | 951.1249 | PV |
| 16012 | 3128942.935 | 792053.8616 | 950.5945 | BPP |
| 16013 | 3128964.674 | 792058.7276 | 951.1371 | BPP |
| 16014 | 3128946.568 | 792002.3533 | 947.6783 | PF |
| 16015 | 3128949.364 | 791999.3601 | 947.6107 | PF |
| 16016 | 3128950.502 | 792001.8178 | 948.2585 | PV |
| 16017 | 3128948.365 | 792011.0717 | 948.698 | TR |
| 16018 | 3128948.768 | 792019.0413 | 949.0173 | TR |
| 16019 | 3128951.494 | 792025.8995 | 950.9873 | LB |
| 16020 | 3128950.059 | 792034.2512 | 951.1153 | LB |
| 16021 | 3128958.15 | 792047.6036 | 951.1962 | LB |
| 16022 | 3128938.557 | 792039.18 | 948.3764 | TR |
| 16023 | 3128935.926 | 792036.3142 | 948.0262 | TR |
| 16024 | 3128922.084 | 792044.4341 | 947.597 | PV |
| 16025 | 3128937.29 | 792044.5192 | 949.1777 | SH |
| 16026 | 3128940.827 | 792042.6364 | 950.1267 | SH |
| 16027 | 3128943.078 | 792053.7874 | 950.5884 | BPP |
| 16028 | 3128937.785 | 792078.1843 | 950.563 | BPP |
| 16029 | 3128964.603 | 792058.6813 | 951.1596 | BPP |
| 17000 | 3128994.916 | 792096.1217 | 950.7157 | M5 |
| 17001 | 3128967.676 | 792079.5907 | 951.1029 | O5 |
| 17004 | 3129004.433 | 792075.3797 | 951.3139 | RIC |
| 17005 | 3129006.317 | 792078.8658 | 951.3284 | RIC |
| 17006 | 3129012.326 | 792074.5342 | 951.329 | RIC |
| 17007 | 3129014.719 | 792077.7215 | 951.367 | RIC |
| 17008 | 3129015.389 | 792077.4339 | 951.3651 | RIC |
| 17009 | 3129019.305 | 792082.3407 | 951.3767 | RIC |
| 17010 | 3129017.353 | 792083.9062 | 951.373 | RIC |
| 17011 | 3129017.829 | 792084.7361 | 951.3703 | RIC |
| 17012 | 3129015.558 | 792086.2793 | 951.3787 | RIC |
| 17013 | 3129020.279 | 792093.0889 | 951.39 | RIC |
| 17014 | 3129022.71 | 792091.4958 | 951.3774 | RIC |
| 17015 | 3129025.148 | 792090.9785 | 951.3876 | RIC |
| 17016 | 3129030.028 | 792092.1265 | 951.3734 | RIC |
| 17017 | 3129036.26 | 792101.996 | 951.4186 | RIC |
| 17018 | 3129033.059 | 792097.3583 | 951.4058 | RIC |
| 17019 | 3129038.356 | 792103.9341 | 951.456 | RIC |
| 17020 | 3129044.484 | 792106.7907 | 951.4267 | RIC |
| 17021 | 3129050.619 | 792115.1196 | 950.674 | ADMI |
| 17022 | 3129049.494 | 792123.2003 | 950.5994 | ADMI |
| 17023 | 3129051.686 | 792123.5965 | 950.792 | ADMI |
| 17024 | 3129048.898 | 792133.9418 | 950.764 | ADMI |
| 17025 | 3129049.6 | 792140.6603 | 950.4356 | PV |
| 17026 | 3129049.422 | 792143.036 | 950.5119 | ADMI |
| 17027 | 3129048.823 | 792146.4665 | 950.1075 | ADMI |
| 17028 | 3129047.734 | 792146.0126 | 950.0578 | ADMI |
| 17029 | 3129046.552 | 792150.5711 | 950.0034 | ADMI |
| 17030 | 3129044.259 | 792150.2077 | 950.0425 | ADMI |
| 17031 | 3129042.683 | 792157.7286 | 949.9975 | ADMI |
| 17032 | 3129034.169 | 792140.7632 | 950.7309 | TR |
| 17033 | 3129028.69 | 792138.2565 | 950.7798 | TR |
| 17034 | 3129026.705 | 792137.207 | 950.7124 | TR |
| 17035 | 3129040.883 | 792160.0659 | 950.2358 | P |
| 17036 | 3129039.426 | 792167.1845 | 950.1086 | P |
| 17037 | 3129045.901 | 792161.4733 | 950.1736 | P |
| 17038 | 3129044.36 | 792168.3849 | 950.0513 | P |
| 17039 | 3129033.05 | 792177.4867 | 950.1187 | RC |
| 17040 | 3129027.588 | 792170.4372 | 950.3021 | RC |
| 17041 | 3129017.218 | 792167.9423 | 950.4087 | RC |
| 17042 | 3129011.274 | 792155.9862 | 951.0786 | BR |
| 17043 | 3129011.93 | 792176.4069 | 950.1066 | RC |
| 17044 | 3129009.766 | 792182.3335 | 949.584 | RC |
| 17045 | 3128999.245 | 792160.6436 | 949.9395 | AP |
| 17046 | 3129009.635 | 792139.1071 | 950.6892 | AP |
| 17047 | 3129010.096 | 792139.7454 | 950.722 | PV |
| 17048 | 3129008.969 | 792141.1298 | 950.7466 | PV |
| 17050 | 3128985.977 | 792127.6559 | 950.5899 | PV |
| 17051 | 3128987.417 | 792128.2798 | 950.6174 | PV |
| 17052 | 3128993.752 | 792142.0931 | 951.2675 | AP |
| 17053 | 3128978.851 | 792108.4641 | 950.4829 | PV |
| 17054 | 3128980.141 | 792107.721 | 950.5645 | PV |
| 17055 | 3128982.197 | 792151.8422 | 950.9527 | AP |
| 17056 | 3128975.176 | 792124.7827 | 950.5279 | AD |
| 17057 | 3128977.861 | 792118.3914 | 950.4555 | AD |
| 17058 | 3128970.962 | 792115.4944 | 950.6413 | AD |
| 17059 | 3128973.931 | 792107.2054 | 950.7513 | AD |
| 17060 | 3128965.344 | 792103.309 | 950.6834 | AD |
| 17061 | 3128962.799 | 792087.6796 | 950.6235 | TP |
| 17062 | 3128962.386 | 792082.9884 | 951.1758 | PV |
| 17063 | 3128960.514 | 792081.4752 | 951.1809 | LP |
| 17064 | 3128947.14 | 792097.0264 | 951.1371 | LP |
| 17065 | 3128961.083 | 792079.3792 | 951.1442 | BPP |
| 17066 | 3128981.555 | 792064.167 | 952.1405 | MH |
| 17067 | 3128983.757 | 792065.3501 | 951.2691 | PV |
| 17068 | 3128986.349 | 792066.9203 | 951.2494 | PV |
| 17069 | 3128984.609 | 792071.5439 | 952.0851 | PV |
| 17070 | 3128981.692 | 792070.1295 | 952.1267 | PV |
| 17071 | 3128991.973 | 792069.4357 | 952.1638 | MH |
| 17072 | 3128994.384 | 792061.8434 | 952.1727 | MH |
| 17073 | 3128999.011 | 792055.7184 | 952.1372 | WWS |
| 17074 | 3129001.142 | 792056.6384 | 952.1197 | WWS |
| 17075 | 3128994.556 | 792064.6435 | 952.2774 | WWS |
| 17076 | 3128997.838 | 792066.3315 | 952.2275 | WWS |
| 17077 | 3129004.628 | 792056.1915 | 951.2262 | PV |
| 17078 | 3129005.928 | 792044.1297 | 952.3151 | PV |
| 17079 | 3129001.604 | 792054.3903 | 952.2889 | PV |
| 17080 | 3129002.916 | 792055.4731 | 951.2165 | PV |
| 17081 | 3128989.51 | 792048.985 | 952.172 | MH |
| 17082 | 3129013.739 | 792038.7593 | 952.3945 | EP |
| 17083 | 3128991.859 | 792037.9077 | 952.2884 | MH |
| 17084 | 3128994.648 | 792111.6293 | 950.7257 | WWS |
| 17085 | 3128989.506 | 792114.9374 | 950.7343 | WWS |
| 17086 | 3128993.248 | 792116.8908 | 950.7106 | WWS |
| 17087 | 3128995.973 | 792116.3567 | 950.7043 | WWS |
| 17088 | 3128996.841 | 792113.3915 | 950.8418 | WWS |
| 17089 | 3128979.679 | 792099.5419 | 950.9746 | SUPA |
| 17090 | 3128978.125 | 792092.1791 | 951.5628 | SUPA |
| 17091 | 3128977.464 | 792095.2078 | 951.0286 | SUPA |
| 17092 | 3128976.542 | 792089.5724 | 951.5298 | SUPA |
| 17093 | 3128974.428 | 792095.4841 | 950.9131 | SUPA |
| 17094 | 3128973.013 | 792091.9455 | 951.4561 | SUPA |
| 17095 | 3129068.167 | 792119.2637 | 950.8954 | ADMI |
| 17096 | 3129076.182 | 792123.2724 | 951.2075 | TEM |
| 17097 | 3129063.281 | 792109.6161 | 951.5019 | TR |
| 17098 | 3129076.632 | 792110.0579 | 952.2477 | TR |
| 17099 | 3129039.845 | 792180.4073 | 949.7823 | CW |
| 17100 | 3129058.704 | 792184.4078 | 949.6342 | CW |
| 17101 | 3128976.069 | 792055.3003 | 951.5347 | TP |
| 1800 | 3128975.676 | 792066.5907 | 951.1029 | O6 |
| 1801 | 3128966.609 | 792049.0672 | 951.3295 | LB |
| 1802 | 3128968.01 | 792043.6021 | 951.8529 | LB |
| 1803 | 3128973.531 | 792044.2782 | 952.0391 | LB |
| 1804 | 3128975.938 | 792045.45 | 951.9819 | LB |
| 1805 | 3128974.902 | 792050.3313 | 951.918 | LB |
| 1806 | 3128974.508 | 792053.0663 | 951.8318 | LB |
| 1807 | 3128978.558 | 792053.9334 | 952.3722 | MH |
| 1808 | 3128974.463 | 792055.765 | 951.8386 | PV |
| 1809 | 3128969.609 | 792062.54 | 952.0363 | TR |
| 1810 | 3129013.667 | 792063.7102 | 951.5112 | RIC |
| 1811 | 3128953.02 | 792111.7918 | 950.0215 | FSU |
| 1812 | 3128953.366 | 792117.244 | 949.7819 | PV |
| 1813 | 3128955.693 | 792117.7304 | 949.8201 | PV |
| 1814 | 3128954.973 | 792114.9043 | 949.8548 | PV |
| 1815 | 3128930.534 | 792109.8741 | 949.7983 | GB |
| 7000 | 3129013.654 | 792002.2576 | 952.839 | ST7 |
| 7001 | 3128979.535 | 792033.4916 | 952.2027 | LB |
| 7002 | 3128977.504 | 792034.4263 | 952.1766 | LB |
| 7003 | 3128979.069 | 792026.2163 | 951.6159 | LB |
| 7004 | 3128971.408 | 792024.1263 | 951.5618 | LB |
| 7005 | 3128971.743 | 792021.2928 | 951.4786 | LB |
| 7006 | 3128959.555 | 792018.548 | 951.224 | LB |
| 7007 | 3128984.537 | 791993.1682 | 952.3806 | GYM |
| 7008 | 3128967.797 | 792000.758 | 952.0008 | GYM |
| 7009 | 3128983.648 | 791991.0454 | 952.3335 | GYM |
| 7010 | 3128966.829 | 791998.6319 | 951.8408 | GYM |
| 7011 | 3128985.081 | 791989.5157 | 951.6435 | TR |
| 7012 | 3128987.006 | 791988.557 | 950.7617 | R |
| 7013 | 3128992.592 | 791990.5439 | 950.662 | R |
| 7014 | 3128994.97 | 791987.2223 | 951.4297 | SL |
| 7015 | 3128991.85 | 792004.6498 | 952.2099 | SH |
| 7016 | 3128998.484 | 792014.3684 | 952.4242 | R |
| 7017 | 3129001.585 | 792022.1949 | 952.4486 | R |
| 7018 | 3129011.69 | 792023.3148 | 952.6688 | SL |
| 7019 | 3129011.937 | 792015.5373 | 952.9 | P |
| 7020 | 3129028.228 | 792010.6623 | 953.2088 | P |
| 7021 | 3129020.814 | 792048.2557 | 951.8096 | RIC |
| 7022 | 3129009.082 | 792057.8125 | 951.7425 | RIC |
| 7023 | 3129029.952 | 792044.8346 | 952.5046 | R |
| 7024 | 3129036.308 | 792051.351 | 952.4835 | R |
| 7025 | 3129034.925 | 792065.2849 | 951.7911 | RIC |
| 7026 | 3129037.003 | 792063.3369 | 951.8316 | RIC |
| 7027 | 3129043.014 | 792071.2141 | 952.0267 | RIC |
| 7028 | 3129044.638 | 792067.2327 | 952.6225 | SL |
| 7029 | 3129053.84 | 792090.8905 | 951.8026 | RIC |
| 7030 | 3129010.168 | 791994.7185 | 952.8482 | TR |
| 7031 | 3129009.635 | 791987.6036 | 952.4618 | TR |
| 7032 | 3129008.936 | 791974.4801 | 952.536 | TR |
| 7033 | 3128989.281 | 792004.0557 | 952.2415 | TT |
| 7034 | 3128967.664 | 792000.8234 | 951.98 | TR |
| 7035 | 3128985.779 | 792002.8441 | 952.1912 | TT |
| 7036 | 3128987.237 | 792001.6812 | 952.1955 | TT |
| 7037 | 3128987.55 | 792005.1936 | 952.1781 | TT |
| 7038 | 3128974.33 | 792045.9477 | 952.227 | MH |
| 7039 | 3128972.603 | 792048.7318 | 952.2123 | MH |
| 2500 | 3129115.986 | 792149.0825 | 951.595 | ST1 |
| 2501 | 3129107.819 | 792149.8572 | 951.9968 | FOT |
| 2502 | 3129101.862 | 792139.3193 | 952.0311 | FOT |
| 2503 | 3129086.725 | 792121.4724 | 951.4666 | TEM |
| 2504 | 3129084.579 | 792127.8284 | 951.3744 | TEM |
| 2505 | 3129112.916 | 792159.54 | 951.9633 | R |
| 2506 | 3129117.425 | 792156.2393 | 951.9612 | R |
| 2507 | 3129093.401 | 792126.8993 | 952.0378 | R |
| 2508 | 3129097.282 | 792123.0825 | 952.1443 | R |
| 2509 | 3129094.811 | 792156.823 | 951.6192 | R1 |
| 2510 | 3129088.791 | 792156.8142 | 951.537 | R1 |
| 2511 | 3129095.209 | 792118.8037 | 952.2925 | SL |
| 2512 | 3129081.381 | 792096.7751 | 952.1963 | P |
| 2513 | 3129094.321 | 792084.3078 | 952.708 | P |
| 2514 | 3129149.299 | 792221.5921 | 951.9509 | CCG |
| 2515 | 3129148.757 | 792221.9342 | 951.8402 | CCG |
| 2516 | 3129147.903 | 792222.8406 | 951.8331 | CCG |
| 2517 | 3129153.342 | 792219.4941 | 951.6753 | CCG |
| 2518 | 3129078.447 | 792100.4892 | 952.1983 | R |
| 2519 | 3129084.925 | 792109.6861 | 952.1966 | R |
| 2520 | 3129131.695 | 792190.8422 | 951.8499 | R |
| 2521 | 3129134.108 | 792194.9471 | 951.7965 | R |
| 2522 | 3129130.048 | 792196.3749 | 951.5024 | P |
| 2523 | 3129126.81 | 792189.437 | 951.4976 | P |
| 2524 | 3129106.038 | 792167.9579 | 951.518 | WT |
| 2525 | 3129101.515 | 792166.3715 | 951.4277 | WT |
| 2526 | 3129122.246 | 792149.051 | 951.9804 | AC |
| 2527 | 3129115.122 | 792133.239 | 951.6436 | AC |
| O7 | 3129074.982 | 792161.4412 | 951.2162 | O7 |
| O8 | 3129058.511 | 792160.6824 | 950.1831 | ADMI |
| O9 | 3129059.976 | 792153.2814 | 950.1716 | ADMI |
| O10 | 3129058.384 | 792146.8531 | 950.1524 | ADMI |
| O11 | 3129061.437 | 792145.2395 | 950.6199 | ADMI |
| O12 | 3129060.224 | 792141.895 | 950.6199 | ADMI |
| O13 | 3129062.057 | 792136.1595 | 950.6199 | ADMI |
| O14 | 3129064.225 | 792126.432 | 950.9693 | ADMI |
| O15 | 3129066.709 | 792126.8257 | 950.9627 | ADMI |
| O16 | 3129082.609 | 792174.1188 | 950.7242 | ppp |
| O17 | 3129066.296 | 792168.7479 | 950.6307 | PPP |
| O18 | 3129074.983 | 792151.0177 | 951.0767 | PPP |
| O19 | 3129077.965 | 792180.232 | 950.9693 | R1 |
| O20 | 3129097.188 | 792149.0722 | 951.8091 | FOT |
| O21 | 3129100.763 | 792171.1372 | 951.317 | WT |
| O22 | 3129084.746 | 792180.1473 | 950.4831 | R1 |
| O23 | 3129084.157 | 792185.7058 | 951.0293 | SL |
| O24 | 3129089.093 | 792206.4338 | 950.1845 | P |
| O25 | 3129079.133 | 792145.5017 | 950.955 | G |
| O26 | 3129077.342 | 792148.7164 | 950.9977 | G |
| O27 | 3129075.369 | 792127.4902 | 950.9306 | BO |
| O28 | 3129079.851 | 792126.7259 | 951.0793 | PV |
| O29 | 3129072.002 | 792124.5891 | 950.2453 | PV |

ROAD SURVEY

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                             Date:2080/12/20

Recorder:Group3                                                              Time:8am

Instrument: Level Mechine, Staff, Tripod stand and Tape

Table:Differntial Levelling-Level Transfer From Benchmark To Starting Point

| S.No | Backsight Reading (B.S) |  |  | Foresight Reading (F.S) |  |  | Rise | Fall | Reduce level (RL) | Distance |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom | Top | Middle | Bottom |  |  |  | BS | FS |  |
| B.M | 0.693 | 0.654 | 0.615 |  |  |  |  |  | 905 | 7.6 |  |  |
| 1 | 1.093 | 1.058 | 1.024 | 1.942 | 1.904 | 1.866 |  | -1.25 | 903.75 | 7.6 | 7.6 |  |
| 2 | 1.136 | 1.099 | 1.060 | 1.698 | 1.660 | 1.622 |  | 0.602 | 903.148 | 7.5 | 7.6 |  |
| 3 | 1.279 | 1.249 | 1.220 | 1.646 | 1.608 | 1.571 |  | 0.509 | 902.639 | 6 | 7.5 |  |
| 4 |  |  |  | 1.60 | 1.575 | 1.550 |  | 0.326 | 902.313 |  | 6 |  |

The Reduce level for the starting point for the road is found to be 902.313 which is the center line reduce level of road alignment.

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                             Date:2080/12/20

Recorder:Group3                                                              Time:10am

Instrument: Thedolite, Tripod stand and Tape

Table:Road Alignment Sheet

| IP | Distance Between IP(m) | Deflection angle | Radius (R) | Tangent Distance (T) | Curve Length (L) | Mid Ordinate (MO) | Apex Distance | Chain-age of IP | Chain-age of BC | Chain-age of MO | Chain-age of EC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IP0 |  |  |  |  |  |  |  | 0+000 |  |  |  |  |
| IP1 | 62.8 | 43° 44' 52" | 40 | 16.058 | 20.54 | 2.87 | 3.10 | 0+62.8 | 0+46.74 | 0+62.012 | 0+77.282 | L |
| IP2 | 53.08 | 51° 04' 49" | 30 | 14.33 | 26.74 | 2.931 | 3.248 | 0+118.88 | 0+101.55 | 0+114.92 | 0+128.2982 | R |
| IP3 | 58.54 | 81° 33' 18" | 20 | 17.249 | 28.470 | 4.850 | 6.410 | 0+174.42 | 0+157.171 | 0+171.406 | 0+185.641 | (L ) |
| IP4 | 55 | 54° 23' 14" | 35 | 17.980 | 33.220 | 3.860 | 4.340 | 0+229.42 | 0+211.440 | 0+228.050 | 0+244.65 | (R) |
| IP5 | 91.05 | 70° 22' 58" | 20 | 14.100 | 24.570 | 3.655 | 4.480 | 0+321.37 | 0+307.270 | 0+319.555 | 0+331.84 | (R ) |
| IP6 | 78.63 | 21° 12' 6" |  |  |  |  |  | 0+400 |  |  |  |  |
| IP7 | 69.82 | 54° 49' 51" | 35 | 17.380 | 32.270 | 3.654 | 4.080 | 0+469.82 | 0+452.440 | 0+468.575 | 0+484.71 | (R) |
| IP8 | 56.18 | 13° 39' 21" |  |  |  |  |  | 0+526 |  |  |  |  |
| IP9 | 44 | 31° 30' 31" | 25 | 7.131 | 13.890 | 0.959 | 0.990 | 0+570 | 0+562.869 | 0+569.814 | 0+576.759 | (R) |
| IP10 | 40.9 | 52° 18' 26" | 30 | 14.730 | 27.380 | 3.071 | 3.420 | 0+610.90 | 0+596.170 | 0+609.860 | 0+623.55 | (L) |
| IP11 | 60.4 | 27° 52' 32" | 30 | 7.440 | 14.590 | 0.883 | 0.910 | 0+671.30 | 0+663.860 | 0+671.155 | 0+678.45 | (R) |
| IP12 | 57.15 | 46° 26' 13" | 20 | 8.579 | 16.200 | 1.610 | 1.760 | 0+728.45 | 0+719.871 | 0+727.971 | 0+736.071 | (L) |
| IP13 | 28.773 | 23° 47' 56" |  |  |  |  |  | 0+757.229 |  |  |  |  |
| IP14 | 57.961 | 18° 17' 40" |  |  |  |  |  | 0+815.19 |  |  |  |  |
| IP15 | 32.7 | 43° 12' 6" | 25 | 9.890 | 18.850 | 1.755 | 1.880 | 0+848.10 | 0+838.21 | 0+847.635 | 0+857.060 | (L) |
| IP16 | 46.7 | 12° 55' 11" |  |  |  |  |  | 0+894.8 |  |  |  |  |
| Ip17 | 101.22 | 28° 1' 33" | 30 | 7.480 | 14.67 | 0.890 | 0.920 | 0+996.02 | 0+988.54 | 0+995.875 | 0+1003.21 | (R) |

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                       Date:2080/12/21

Recorder:Group3                                                        Time:10am

Instrument: Level Machine,Staff and Tape

Table:Profile Leveling and Cross-section Leveling Sheet(Road Alignment)

| Chain-age | Distance |  |  | Backsight (BS) | Intermediate sight (IS) | Forsight (FS) | Height of Instrument (HI) | Reduce Level (RL) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Left | Center | Right |  |  |  |  |  |  |
| 0+00 |  | 0+00 |  | 0.939 |  |  | 903.252 | 902.313 |  |
|  | 2.84 |  |  |  | 1.018 |  |  | 902.234 |  |
|  | 5 |  |  |  | 0.860 |  |  |  | Down Slope |
|  |  |  | 2.5 |  | 0.979 |  |  | 902.392 |  |
|  |  |  | 5 |  | 1.596 |  |  | 902.273 |  |
| 0+20 |  | 0+20 |  |  | 1.495 |  |  | 901.656 |  |
|  | 2.5 |  |  |  | 1.360 |  |  | 901.757 |  |
|  | 5 |  |  |  | 1.745 |  |  | 901.892 |  |
|  |  |  | 2.5 |  | 1.980 |  |  | 901.507 |  |
|  |  |  | 5 |  |  |  |  | 901.272 |  |
| 0+40 |  | 0+40 |  |  | 2.229 |  |  | 901.023 |  |
|  |  |  | 0.75 |  | 2.240 |  |  | 901.012 |  |
|  |  |  | 2.5 |  | 2.312 |  |  | 900.940 |  |
|  |  |  | 5 |  |  |  |  |  | Slope of 51° 56' 35.9" |
|  | 2.5 |  |  |  | 2.234 |  |  | 901.018 |  |
|  | 5 |  |  | 1.002 |  | 2.334 | 901.92 | 900.918 |  |
| 0+46.74 |  | BC1 |  |  | 0.968 |  |  | 900.952 |  |
|  | 2.5 |  |  |  | 1.024 |  |  | 900.896 |  |
|  | 5 |  |  |  | 1.125 |  |  | 900.795 |  |
|  |  |  | 2.5 |  | 1.070 |  |  | 900.850 |  |
|  |  |  | 5 |  | 1.120 |  |  | 900.800 |  |
| 0+62.012 |  | M0 |  |  | 1.055 |  |  | 900.865 |  |
|  |  |  | 2.5 |  | 1.100 |  |  | 900.820 |  |
|  |  |  | 5 |  |  |  |  |  | Slope of 51° 56' 35.9" |
|  | 2.5 |  |  |  | 1.115 |  |  | 900.805 |  |
|  | 5 |  |  |  | 1.285 |  |  | 900.635 |  |
| 0+77.282 |  | EC1 |  |  | 1.090 |  |  | 900.830 |  |
|  |  |  | 2.5 |  | 1.170 |  |  | 900.750 |  |
|  |  |  | 5 |  |  |  |  |  |  |
|  | 5 |  |  | 2.12 |  | 0.985 | 903.055 | 900.935 |  |
| 0+100 |  | 0+100 |  |  | 1.585 |  |  | 901.470 |  |
|  |  |  | 2.5 |  | 1.469 |  |  | 901.586 |  |
|  |  |  | 5 |  | 1.964 |  |  | 901.091 |  |
|  | 2.5 |  |  |  | 1.765 |  |  | 901.290 |  |
|  | 5 |  |  |  |  |  |  |  | slope |
| 0+101.55 |  | BC2 |  |  | 1.481 |  |  | 901.574 |  |
|  |  |  | 5 |  | 1.090 |  |  | 901.965 |  |
|  | 5 |  |  |  | 1.775 |  |  | 901.280 |  |
| 0+114.92 |  | MO |  |  | 1.464 |  |  | 901.591 |  |
|  |  |  | 2.5 |  | 1.644 |  |  | 901.411 |  |
|  |  |  | 5 |  | 1.832 |  |  | 901.223 |  |
|  | 5 |  |  |  | 1.113 |  |  | 901.942 |  |
|  | 10 |  |  |  | 1.035 |  |  | 902.020 |  |
| 0+128.2982 |  | EC2 |  |  | 1.382 |  |  | 901.673 |  |
|  |  |  | 3 |  | 1.700 |  |  | 901.355 |  |
|  |  |  | 5 |  |  |  |  |  | slope |
|  | 2.5 |  |  |  | 1.265 |  |  | 901.790 |  |
|  | 5 |  |  |  | 1.157 |  |  | 901.898 |  |
| 0+140 |  | 0+140 |  |  | 2.535 |  |  | 900.520 |  |
|  |  |  | 2 |  | 2.580 |  |  | 900.475 |  |
|  |  |  | 5 |  |  |  |  |  | slope |
|  | 2.5 |  |  |  | 2.477 |  |  | 900.580 |  |
|  | 6 |  |  | 0.769 |  | 2.809 | 901.015 | 900.246 |  |
| 0+157.171 |  | Bc3 |  |  | 2.121 |  |  | 898.894 |  |
|  |  |  | 2.5 |  | 2.208 |  |  | 898.807 |  |
|  |  |  | 5 |  |  |  |  |  | River |
|  | 2.5 |  |  |  | 1.958 |  |  | 899.057 |  |
|  | 5 |  |  |  | 1.905 |  |  | 899.110 |  |
|  | 7.5 |  |  | 0.53 |  | 1.643 | 899.902 | 899.372 |  |
| 0+171.406 |  | Mo |  |  | 2.128 |  |  | 897.774 |  |
|  |  |  | 2.5 |  | 2.098 |  |  | 897.904 |  |
|  |  |  | 5 |  | 2.079 |  |  | 897.823 |  |
|  | 2.5 |  |  |  | 2.024 |  |  | 897.878 |  |
|  | 5 |  |  |  |  |  |  |  | River |
| 0+185.641 |  | Ec3 |  | 1.430 |  | 2.191 | 899.141 | 897.711 |  |
|  | 3 |  |  |  | 1.420 |  |  | 897.721 |  |
|  |  |  | 2 |  | 1.420 |  |  | 897.724 |  |
|  |  |  | 5 |  |  |  |  |  | Pre made bridge |
| 0+211.44 |  | Bc4 |  |  | 1.400 |  |  | 897.741 |  |
|  | 2 |  |  |  | 1.400 |  |  | 897.741 |  |
|  |  |  | 3 |  | 1.400 |  |  | 897.741 |  |
| 0+228.05 |  | Mo |  |  | 1.354 |  |  | 897.787 |  |
|  |  |  | 2.5 |  | 1.369 |  |  | 897.772 |  |
|  | 2.5 |  |  |  | 1.363 |  |  | 897.778 |  |
|  |  |  | 5 | 2.270 |  | 1.356 | 900.064 | 897.785 |  |
| 244.65 |  | Ec4 |  |  | 0.564 |  |  | 899.500 |  |
|  |  |  | 2 |  | 0.550 |  |  | 899.514 |  |
|  |  |  | 4 |  | 0.529 |  |  | 899.535 |  |
|  | 2 |  |  | 2.018 |  | 0.544 | 901.538 | 899.520 |  |
| 0+252.35 |  | EC4+7.7 |  | 2.061 |  | 0.714 | 902.885 | 900.824 |  |
| 0+260 |  | 0+260 |  |  | 0.807 |  |  | 902.078 |  |
|  |  |  | 2.5 |  | 0.714 |  |  | 902.171 |  |
|  |  |  | 5 |  | 0.676 |  |  | 902.209 |  |
|  | 2.5 |  |  |  | 0.769 |  |  | 902.116 |  |
|  | 5 |  |  | 2.351 |  | 0.735 | 904.501 | 902.150 |  |
| 0+271.47 |  | 0+271.47 |  | 1.775 |  | 0.56 | 905.716 | 903.941 |  |
| 0+280 |  | 0+280 |  |  | 0.575 |  |  | 905.141 |  |
|  | 2.5 |  |  |  | 0.660 |  |  | 905.056 |  |
|  | 5 |  |  |  | 0.816 |  |  | 904.900 |  |
|  | 7.5 |  |  | 1.135 |  | 0.960 | 905.075 | 903.940 |  |
|  |  |  | 2 |  |  |  |  |  | Building |
| 0+287.78 |  | 0+287.78 |  | 1.225 |  | 0.644 | 905.656 | 904.431 |  |
| 0+300 |  | 0+300 |  |  | 0.595 |  |  | 905.061 |  |
|  |  |  | 2.5 |  | 0.685 |  |  | 904.921 |  |
|  | 2.5 |  |  |  | 0.794 |  |  | 904.862 |  |
|  | 5 |  |  | 1.951 |  | 1.103 | 906.504 | 904.553 |  |
| 0+301.27 |  | Bc5 |  |  | 0.846 |  |  | 905.658 |  |
|  |  |  | 2.5 |  | 0.591 |  |  | 905.913 |  |
|  | 2.5 |  |  |  | 1.132 |  |  | 905.372 |  |
|  | 5 |  |  | 2.442 |  | 1.294 | 907.652 | 905.210 |  |
| 0+319.555 |  | MO |  |  | 0.783 |  |  | 906.869 |  |
|  |  |  | 2.5 |  | 0.634 |  |  | 907.018 |  |
|  |  |  | 5 |  | 0.864 |  |  | 906.788 |  |
|  | 2.5 |  |  |  | 1.036 |  |  | 906.626 |  |
|  | 6.5 |  |  | 2.321 |  | 1.318 | 908.655 | 906.334 |  |
| 0+331.37 |  | 0+331.37 |  | 1.401 |  | 0.623 | 909.433 | 908.032 |  |
| 0+331.84 |  | Ec5 |  |  | 0.865 |  |  | 908.568 |  |
|  | 1 |  |  |  |  |  |  |  | Park |
|  |  |  | 2.5 |  | 0.948 |  |  | 908.485 |  |
|  |  |  | 5 |  | 1.056 |  |  | 908.377 |  |
|  |  |  | 7.5 | 2.223 |  | 1.123 | 910.533 | 908.310 |  |
| 0+352.75 |  | 0+352.75 |  | 1.841 |  | 0.734 | 911.640 | 909.799 |  |
| 0+360 |  | 0+360 |  |  | 0.907 |  |  | 910.733 |  |
|  |  |  | 2.5 |  | 0.688 |  |  | 910.952 |  |
|  |  |  | 5 |  | 0.515 |  |  | 911.125 |  |
|  | 2.5 |  |  |  | 1.087 |  |  | 910.553 |  |
|  | 5 |  |  |  |  |  |  |  | Slope |
| 0+373.65 |  | 0+373.65 |  | 2.344 |  | 1.560 | 912.424 | 910.08 |  |
| 0+380 |  | 0+380 |  |  | 0.970 |  |  | 911.454 |  |
|  | 1.5 |  |  |  | 1.076 |  |  | 911.348 |  |
|  | 5 |  |  |  |  |  |  |  | Slope |
|  |  |  | 2.5 |  | 0.794 |  |  | 911.630 |  |
|  |  |  | 5 | 1.460 |  | 0.634 | 913.250 | 911.790 |  |
| 0+389.63 |  | 0+389.63 |  | 1.663 |  | 0.785 | 914.128 | 912.465 |  |
| 0+400 |  | 0+400 |  |  | 0.601 |  |  | 913.527 |  |
|  |  |  | 2.5 |  | 0.560 |  |  | 913.568 |  |
|  |  |  | 5 |  |  |  |  |  | Slope of 54° 49' 51" |
|  | 5 |  |  | 2.130 |  | 0.970 | 915.288 | 913.158 |  |
| 0+416.38 |  | 0+416.38 |  | 1.865 |  | 0.615 | 916.538 | 914.673 |  |
| 0+420 |  | 0+420 |  |  | 1.565 |  |  | 914.973 |  |
|  |  |  | 2.5 |  | 1.185 |  |  | 915.353 |  |
|  | 2.5 |  |  |  | 1.815 |  |  | 914.723 |  |
|  | 3 |  |  |  | 1.945 |  |  | 914.593 |  |
| 0+430.36 |  | 0+430.36 |  | 1.970 |  | 0.690 | 917.818 | 915.848 |  |
| 0+440 |  | 0+440 |  |  | 1.190 |  |  | 916.628 |  |
|  |  |  | 2 |  |  |  |  | 916.868 |  |
|  |  |  | 5 |  | 0.950 |  |  |  | slope |
|  | 2.5 |  |  |  | 1.490 |  |  | 916.328 |  |
|  | 5 |  |  |  | 1.660 |  |  | 916.158 |  |
| 0+446.2 |  | 0+446.2 |  | 1.990 |  | 0.720 | 919.088 | 917.098 |  |
| 0+452.44 |  | BC6 |  |  | 1.565 |  |  | 917.523 |  |
|  |  |  | 2.5 |  | 1.315 |  |  | 917.773 |  |
|  | 2.5 |  |  |  | 1.749 |  |  | 917.339 |  |
|  | 5 |  |  |  | 1.899 |  |  | 917.189 |  |
| 0+468.575 |  | Mo |  |  | 0.575 |  |  | 918.513 |  |
|  | 1 |  |  |  |  |  |  |  | Down-ward slope |
|  |  |  | 2.5 |  | 0.530 |  |  | 918.558 |  |
|  |  |  | 5 |  | 0.778 |  |  | 918.310 |  |
|  |  |  | 7.5 | 2.091 |  | 0.930 | 920.249 | 918.158 |  |
|  |  | 0+482.07 |  | 1.799 |  | 0.742 | 921.306 | 919.507 |  |
| 0+484.71 |  | EC6 |  |  | 1.193 |  |  | 920.113 |  |
|  |  |  | 2.5 |  | 1.412 |  |  | 919.894 |  |
|  | 2.5 |  |  |  | 1.688 |  |  | 919.618 |  |
|  | 5 |  |  |  | 1.954 |  |  | 919.352 |  |
|  |  | 0+493.52 |  | 1.360 |  | 0.665 | 922.001 | 920.641 |  |
| 0+500 |  | 0+500 |  | 1.77 |  | 0.681 | 923.09 | 921.32 |  |
|  | 2.5 |  |  |  | 1.815 |  |  | 921.275 |  |
|  | 4 |  |  |  | 1.895 |  |  | 921.195 |  |
|  | 5 |  |  |  |  |  |  |  | Cliff |
|  |  |  | 2.5 |  | 1.801 |  |  | 921.289 |  |
|  |  |  | 5 |  | 1.829 |  |  | 921.261 |  |
| 0+507.40 |  | 0+507.40 |  | 1.955 |  | 0.691 | 924.354 | 922.399 |  |
|  |  | 0+514.45 |  | 2.032 |  | 0.640 | 925.746 | 923.714 |  |
| 0+520 |  | 0+520 |  |  | 1.314 |  |  | 924.432 |  |
|  | 2.5 |  |  |  | 1.235 |  |  | 924.511 |  |
|  | 5 |  |  |  | 1.155 |  |  | 924.591 |  |
|  |  |  | 2.5 |  | 1.405 |  |  | 924.341 |  |
|  |  |  | 5 |  | 1.362 |  |  | 924.384 |  |
|  |  |  | 6.5 |  | 1.345 |  |  | 924.401 |  |
| 0+526 |  | IP8 |  |  | 1.095 |  |  | 924.651 |  |
|  | 2.5 |  |  |  | 1.112 |  |  | 924.634 |  |
|  | 5 |  |  |  | 1.112 |  |  | 924.634 |  |
|  | 7.5 |  |  |  |  |  |  |  | Cliff |
|  |  |  | 2.5 |  | 0.980 |  |  | 924.766 |  |
|  |  |  | 5 |  | 0.968 |  |  | 924.778 |  |
|  |  |  | 7.5 |  |  |  |  |  | Filling Material Deposited |
| 0+540 |  | 0+540 |  |  | 0.835 |  |  | 924.911 |  |
|  | 2.5 |  |  |  | 0.800 |  |  | 924.946 |  |
|  | 5 |  |  | 0.756 |  | 0.788 | 925.714 | 924.958 |  |
|  | 7 |  |  |  |  |  |  |  | Upper Cliff |
|  |  |  | 2.5 |  | 0.685 |  |  | 925.029 |  |
|  |  |  | 5 |  | 0.930 |  |  | 924.784 |  |
| 0+562.869 |  | BC7 |  |  | 2.335 |  |  | 923.379 |  |
|  |  |  | 2.5 |  | 2.290 |  |  | 923.424 |  |
|  |  |  | 5 |  |  |  |  |  | Down Cliff |
|  | 2.5 |  |  |  | 2.320 |  |  | 923.394 |  |
|  | 4 |  |  | 0.570 |  | 2.110 | 924.174 | 923.604 |  |
| 0+569.814 |  | MO |  |  | 1.405 |  |  | 922.604 |  |
|  |  |  | 2.5 |  | 1.456 |  |  | 922.769 |  |
|  |  |  | 5 |  | 1.269 |  |  | 922.718 |  |
|  | 2.5 |  |  |  | 1.249 |  |  | 922.925 |  |
|  | 3.5 |  |  |  | 1.227 |  |  | 922.947 |  |
| 0+576.759 |  | EC7 |  |  | 1.730 |  |  | 922.444 |  |
|  | 1.5 |  |  |  | 1.693 |  |  | 922.481 |  |
|  | 5 |  |  |  |  |  |  |  | Upper Cliff of slope 72° 14' 29.25" |
|  |  |  | 2.5 |  | 1.903 |  |  | 922.271 |  |
|  |  |  | 5 |  | 1.865 |  |  | 922.309 |  |
|  |  |  | 6 | 0.700 |  | 1.825 | 923.049 | 922.349 |  |
| 0+596.17 |  | BC8 |  |  | 1.385 |  |  | 921.664 |  |
|  | 2.5 |  |  |  | 1.262 |  |  | 921.787 |  |
|  | 5 |  |  |  |  |  |  |  | UPPER CLIFF |
|  |  |  | 2.5 |  | 1.465 |  |  | 921.584 |  |
|  |  |  | 5 |  | 1.301 |  |  | 921.748 |  |
| 0+609.86 |  | MO |  |  | 1.610 |  |  | 921.439 |  |
|  | 2.5 |  |  |  | 1.830 |  |  | 921.219 |  |
|  | 4 |  |  |  | 1.630 |  |  | 921.419 |  |
|  |  |  | 2.5 |  | 1.485 |  |  | 921.564 |  |
|  |  |  | 5 | 1.225 |  | 1.595 | 922.679 | 921.454 |  |
| 0+623.550 |  | EC8 |  |  | 1.489 |  |  | 921.190 |  |
|  |  |  | 2 |  | 1.290 |  |  | 921.389 |  |
|  | 2.5 |  |  |  | 1.518 |  |  | 921.161 |  |
|  | 5 |  |  |  | 1.249 |  |  | 921.430 |  |
| 0+640 |  | 0+640 |  |  | 1.222 |  |  | 921.457 |  |
|  | 1.5 |  |  |  | 1.150 |  |  | 921.529 |  |
|  |  |  | 2.5 |  | 1.190 |  |  | 921.489 |  |
|  |  |  | 5 | 0.845 |  | 0.625 | 922.899 | 922.054 |  |
| 0+663.86 |  | BC9 |  |  | 2.151 |  |  | 920.748 |  |
|  |  |  | 2.5 |  | 1.905 |  |  | 920.994 |  |
|  |  |  | 5 |  |  |  |  |  | Fence |
|  | 2.5 |  |  | 0.550 |  | 2.080 | 921.369 | 920.819 |  |
| 0+671.155 |  | MO |  |  | 1.064 |  |  | 920.305 |  |
|  | 2.5 |  |  |  | 0.836 |  |  | 920.533 |  |
|  | 5 |  |  |  |  |  |  |  | UPPER CLIFF |
|  |  |  | 2.5 |  | 0.842 |  |  | 920.527 |  |
|  |  |  | 5 |  |  |  |  |  | FENCH |
| 0+678.45 |  | EC9 |  |  | 1.624 |  |  | 919.745 |  |
|  | 2.5 |  |  |  | 1.565 |  |  | 919.804 |  |
|  |  |  | 2.5 | 0.595 |  | 1.487 | 920.477 | 919.882 |  |
| 0+693.04 |  | 0+693.04 |  | 0.927 |  | 2.230 | 919.174 | 918.247 |  |
| 0+700 |  | 0+700 |  |  | 2.211 |  |  | 916.963 |  |
|  |  |  | 2.5 |  | 2.200 |  |  | 916.974 |  |
|  |  |  | 5 |  | 1.916 |  |  | 917.258 |  |
|  | 1.5 |  |  | 0.595 |  | 2.067 | 917.702 | 917.107 |  |
|  | - |  |  |  |  |  |  |  | Cliff |
| 0+711.071 |  | 0+711.071 |  | 0.732 |  | 2.285 | 2.285 | 915.417 |  |
| 0+719.871 |  | BC10 |  |  | 1.368 |  |  | 914.781 |  |
|  | 2.5 |  |  |  | 1.375 |  |  | 914.774 |  |
|  | 3.5 |  |  |  | 1.210 |  |  | 914.939 |  |
|  |  |  | 2.5 |  | 1.260 |  |  | 914.889 |  |
|  |  |  | 5 |  |  |  |  |  | Slope of 62°14' 29.25" |
| 0+727.971 |  | MO |  |  | 1.519 |  |  | 914.630 |  |
|  | 2.5 |  |  |  | 1.586 |  |  | 914.563 |  |
|  | 5 |  |  |  | 1.485 |  |  | 914.664 |  |
|  |  |  | 2.5 |  | 1.250 |  |  | 914.899 |  |
|  |  |  | 5 |  |  |  |  |  | CLIFF |
| 0+736.071 |  | EC10 |  |  | 1.748 |  |  | 914.401 |  |
|  |  |  | 2 |  | 1.660 |  |  | 914.489 |  |
|  |  |  | - |  |  |  |  |  | Cliff |
|  | 2.5 |  |  |  | 1.789 |  |  | 914.360 |  |
|  | 4 |  |  | 1.090 |  | 1.713 | 915.526 | 914.436 |  |
| 0+757.229 |  | IP13 |  |  | 1.583 |  |  | 913.943 |  |
|  |  |  | 2.5 |  | 1.426 |  |  | 914.100 |  |
|  |  |  | 5 |  | 1.141 |  |  | 914.385 |  |
|  | 2 |  |  |  | 1.459 |  |  | 914.067 |  |
|  | 5 |  |  |  |  |  |  |  | Cliff |
| 0+780 |  | 0+780 |  | 1.261 |  | 1.751 | 915.036 | 913.775 |  |
|  |  |  | 2.5 |  | 1.207 |  |  | 913.829 |  |
|  |  |  | - |  |  |  |  |  | Slope of 50° 3' 13.7" |
|  | 1.5 |  |  |  | 1.226 |  |  | 913.810 |  |
| 0+800 |  | 0+800 |  |  | 1.526 |  |  | 913.510 |  |
|  |  |  | 2.5 |  | 1.428 |  |  | 913.608 |  |
|  |  |  | 5 |  | 1.091 |  |  | 913.945 |  |
|  | 2 |  |  | 1.008 |  | 1.600 | 914.444 | 913.436 |  |
|  | 5 |  |  |  |  |  |  |  | Slope of 47° 46'27.99" |
| 0+815.19 |  | IP14 |  |  | 2.093 |  |  | 912.351 |  |
|  |  |  | 2.5 |  | 2.137 |  |  | 912.307 |  |
|  |  |  | 5 |  | 1.967 |  |  | 912.477 |  |
|  | 2 |  |  | 0.638 |  | 1.984 | 913.098 | 912.460 |  |
| 0+828.11 |  | 0+828.11 |  | 0.865 |  | 2.330 | 911.633 | 910.768 |  |
| 0+838.21 |  | BC11 |  |  | 1.592 |  |  | 910.041 |  |
|  | 3 |  |  |  | 1.445 |  |  | 910.188 |  |
|  | - |  |  |  |  |  |  |  | Cliff |
|  |  |  | 2.5 |  | 1.541 |  |  | 910.092 |  |
|  |  |  | 4.5 |  | 1.480 |  |  | 910.153 |  |
| 0+847.635 |  | MO |  |  | 2.240 |  |  | 909.393 |  |
|  |  |  | 2.5 |  | 2.078 |  |  | 909.555 |  |
|  |  |  | -- |  |  |  |  |  | River Cliff |
|  | 2.5 |  |  |  | 2.231 |  |  | 909.402 |  |
|  | 5 |  |  | 1.027 |  | 2.130 |  | 909.503 |  |
| 0+857.06 |  | EC11 |  |  | 1.807 |  |  | 908.723 |  |
|  | 2.5 |  |  |  | 1.916 |  |  | 908.614 |  |
|  | 5 |  |  |  | 1.495 |  |  | 909.035 |  |
|  |  |  | 2 | 0.590 |  | 1.440 | 909.680 | 909.090 |  |
| 0+865.86 |  | 0+865.86 |  | 0.850 |  | 2.469 | 908.061 | 907.211 |  |
| 0+880 |  | 0+880 |  |  | 1.975 |  |  | 906.086 |  |
|  | 3 |  |  | 0.806 |  | 1.824 | 907.043 | 906.237 |  |
|  |  |  | 1 |  |  |  |  |  | Filling Material Deposited |
| 0+894.8 |  | IP16 |  |  | 2.430 |  |  | 904.613 |  |
|  |  |  | 1 |  |  |  |  |  | Bamboo Tree |
|  | 2.5 |  |  |  | 2.445 |  |  | 904.598 |  |
|  | 5 |  |  | 0.765 |  | 2.325 |  | 904.718 |  |
| 0+900 |  | 0+900 |  |  | 1.120 |  |  | 904.363 |  |
|  | 2.5 |  |  |  | 1.220 |  |  | 904.263 |  |
|  | 5 |  |  |  | 1.210 |  |  | 904.273 |  |
|  |  |  | --- |  |  |  |  |  | Private Property |
| 0+920 |  | 0+920 |  |  | 1.571 |  |  | 903.912 |  |
|  | 2.5 |  |  |  | 1.575 |  |  | 903.908 |  |
|  | 5 |  |  | 1.710 |  | 1.662 | 905.531 | 903.821 |  |
| 0+940 |  | 0+940 |  |  | 1.271 |  |  | 904.260 |  |
|  | 2.5 |  |  |  | 1.267 |  |  | 904.264 |  |
|  | 5 |  |  | 1.965 |  | 1.225 | 906.271 | 904.306 |  |
| 0+960 |  | 0+960 |  |  | 1.132 |  |  | 905.139 |  |
|  |  |  | 2.5 |  | 1.641 |  |  | 904.630 |  |
|  | 2.5 |  |  |  | 1.160 |  |  | 905.111 |  |
|  | 5 |  |  |  | 1.236 |  |  | 905.035 |  |
| 0+980 |  | 0+980 |  |  | 0.900 |  |  | 905.371 |  |
|  |  |  | 2.5 |  | 0.885 |  |  | 905.386 |  |
|  | 2.5 |  |  | 1.445 |  | 0.760 | 906.956 | 905.511 |  |
| 0+988.54 |  | BC12 |  |  | 1.534 |  |  | 905.422 |  |
|  |  |  | 2.5 |  | 1.634 |  |  | 905.322 |  |
|  |  |  | 5 |  | 1.714 |  |  | 905.242 |  |
|  | 3 |  |  |  | 1.304 |  |  | 905.652 |  |
| 0+995.875 |  | MO |  |  | 1.387 |  |  | 905.569 |  |
|  |  |  | 2.5 |  | 1.436 |  |  | 905.520 |  |
|  | 2.5 |  |  |  | 1.678 |  |  | 905.278 |  |
| 0+1000 |  | TBM |  |  | 1.316 |  |  | 905.640 |  |
|  |  |  | 3 |  | 1.356 |  |  | 905.600 |  |
|  |  |  | ---- |  |  |  |  |  | Buildings |
|  | 2 |  |  |  | 1.127 |  |  | 905.829 |  |
| 0+1003.21 |  | EC12 |  |  | 1.185 |  |  | 905.771 |  |
|  |  |  | 3 |  | 1.150 |  |  | 905.841 |  |
|  | 2 |  |  |  | 1.074 |  |  | 905.882 |  |

A Temporary Benchmarks is set near the point 1000 so that the location RL can be reused with the help of that benchmark.

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                             Date:2080/12/21

Recorder:Group3                                                           Time:7:30am

Instrument: Level Machine,Staff and Tape

Table:Fly Leveling

| S.No | Back-sight (BS) |  |  | Foresight (FS) |  |  | Rise | Fall | Reduce Level (RL) | Distance |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom | Top | Middle | Bottom |  |  |  | BS | FS |  |
|  |  | 0.610 |  |  |  |  |  |  | 921.32 | 5 |  |  |
| 1 |  | 1.049 |  |  | 2.265 |  |  | 1.655 | 919.665 | 5 | 5 |  |
| 2 |  | 0.954 |  |  | 1.965 |  |  | 0.916 | 918.749 | 8 | 5 |  |
| 3 |  | 1.001 |  |  | 0.972 |  |  | 0.018 | 918.731 | 8 | 8 |  |
| 4 |  | 0.842 |  |  | 1.882 |  |  | 0.881 | 917.85 | 6 | 8 |  |
| 5 |  | 0.691 |  |  | 1.856 |  |  | 1.014 | 916.836 | 8 | 6 |  |
| 6 |  | 0.665 |  |  | 1.779 |  |  | 1.088 | 915.748 | 8 | 8 |  |
| 7 |  | 0.633 |  |  | 1.928 |  |  | 1.263 | 914.485 | 8 | 8 |  |
| 8 |  | 0.516 |  |  | 2.070 |  |  | 1.437 | 913.048 | 8 | 8 |  |
| 9 |  | 0.556 |  |  | 1.950 |  |  | 1.434 | 911.614 | 8 | 8 |  |
| 10 |  | 0.573 |  |  | 2.052 |  |  | 1.496 | 910.118 | 8 | 8 |  |
| 11 |  | 0.660 |  |  | 2.133 |  |  | 1.560 | 908.558 | 5 | 8 |  |
| 12 |  | 0.617 |  |  | 2.068 |  |  | 1.408 | 907.15 | 5 | 5 |  |
| 13 |  | 0.562 |  |  | 1.025 |  |  | 0.408 | 906.742 | 5 | 5 |  |
| 14 |  | 0.617 |  |  | 1.020 |  |  | 0.651 | 906.091 | 5 | 5 |  |
| 15 |  | 0.753 |  |  | 1.060 |  |  | 1.035 | 905.056 | 5 | 5 |  |
| 16 |  | 0.601 |  |  | 1.367 |  |  | 0.814 | 904.242 | 5 | 5 |  |
| 17 |  | 0.587 |  |  | 1.643 |  |  | 1.042 | 903.2 | 5 | 5 |  |
| 18 |  | 0.544 |  |  | 1.560 |  |  | 1.073 | 902.127 | 5 | 5 |  |
| 19 |  | 0.727 |  |  | 1.587 |  |  | 1.043 | 901.084 | 5 | 5 |  |
| 20 |  | 0.586 |  |  | 1.487 |  |  | 1.020 | 900.064 | 4 | 5 |  |
| 21 |  | 0.582 |  |  | 1.479 |  |  | 0.893 | 899.171 | 4 | 4 |  |
| 22 |  | 0.576 |  |  | 1.284 |  |  | 0.702 | 898.469 | 5 | 4 |  |
| 23 |  | 1.360 |  |  | 1.375 |  |  | 0.799 | 897.67 | 7 | 5 |  |
| 24 |  | 1.404 |  |  | 1.282 |  | 0.078 |  | 897.748 | 20 | 7 |  |
| 25 |  | 2.113 |  |  | 0.702 |  | 0.702 |  | 898.45 | 5 | 20 |  |
| 26 |  | 2.140 |  |  | 0.627 |  | 1.486 |  | 899.936 | 6 | 5 |  |
| 27 |  | 1.880 |  |  | 1.465 |  | 0.675 |  | 900.611 | 14 | 6 |  |
| 28 |  | 0.70 |  |  | 1.465 |  | 0.415 |  | 901.026 | 15 | 24 |  |
| 29 |  | 1.413 |  |  | 1.019 |  |  | 0.319 | 900.707 | 17 | 15 |  |
| 30 |  | 1.968 |  |  | 1.027 |  | 0.386 |  | 901.093 | 15 | 17 |  |
| 31 |  | 1.762 |  |  | 0.760 |  | 1.208 |  | 902.301 | 15.2 | 15 |  |
| 32 |  | 1.514 |  |  | 0.810 |  | 0.952 |  | 903.253 | 15 | 15.2 |  |
| 33 |  | 1.498 |  |  | 0.683 |  | 0.831 |  | 904.084 | 8 | 15 |  |
| 34 |  | 1.491 |  |  | 1.090 |  | 0.408 |  | 904.492 | 4.9 | 8 |  |
| 35 |  |  |  |  | 0.977 |  | 0.514 |  | 905.006 |  | 4.9 |  |
|  |  | ∑= 34.75 |  |  | ∑= 49.714 |  | ∑ = 7.655 | ∑ = 23.969 |  |  |  |  |

Arithmetic Check:                                                              Total Loop Distance= 500+500

∑BS-∑FS=∑Rise-∑Fall=Last RL-First RL                            Permissible error=25√k =25mm       34.750-49.714=7.655-23.969=905.006-921.32              Observed error = 6mm

-16.314=-16.314=-16.314

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                       Date:2080/12/23

Recorder:Group3                                                        Time:7:30am

Instrument: Level Machine,Staff and Tape

Table:Fly Leveling

| S.No | Back-sight (BS) |  |  | Foresight (FS) |  |  | Rise | Fall | Reduce Level (RL) | Distance |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom | Top | Middle | Bottom |  |  |  | BS | FS |  |
|  |  | 1.012 |  |  |  |  |  |  | 905.640 | 15 |  |  |
| 1 |  | 0.748 |  |  | 1.119 |  |  | 0.107 | 905.533 | 15 | 15 |  |
| 2 |  | 1.025 |  |  | 1.920 |  |  | 1.172 | 904.361 | 15 | 15 |  |
| 3 |  | 1.611 |  |  | 1.399 |  |  | 0.374 | 903.987 | 8 | 15 |  |
| 4 |  | 1.608 |  |  | 1.399 |  | 0.212 |  | 904.199 | 5 | 8 |  |
| 5 |  | 2.108 |  |  | 0.965 |  | 0.643 |  | 904.842 | 5 | 5 |  |
| 6 |  | 1.672 |  |  | 0.921 |  | 1.187 |  | 906.029 | 5 | 5 |  |
| 7 |  | 1.999 |  |  | 0.951 |  | 0.721 |  | 906.75 | 4 | 5 |  |
| 8 |  | 1.900 |  |  | 0.682 |  | 1.317 |  | 908.067 | 4 | 4 |  |
| 9 |  | 1.566 |  |  | 1.580 |  | 0.32 |  | 908.387 | 5 | 4 |  |
| 10 |  | 1.932 |  |  | 0.947 |  | 0.619 |  | 909.006 | 7 | 5 |  |
| 11 |  | 2.046 |  |  | 0.640 |  | 1.292 |  | 910.298 | 7 | 7 |  |
| 12 |  | 1.996 |  |  | 0.516 |  | 1.53 |  | 911.828 | 8 | 7 |  |
| 13 |  | 1.639 |  |  | 0.731 |  | 1.265 |  | 913.093 | 10 | 8 |  |
| 14 |  | 1.419 |  |  | 1.365 |  | 0.274 |  | 913.367 | 11 | 10 |  |
| 15 |  | 1.486 |  |  | 1.257 |  | 0.162 |  | 913.529 | 10 | 11 |  |
| 16 |  | 1.488 |  |  | 1.008 |  | 0.478 |  | 914.007 | 8 | 10 |  |
| 17 |  | 1.816 |  |  | 1.119 |  | 0.369 |  | 914.376 | 5 | 8 |  |
| 18 |  | 1.894 |  |  | 0.844 |  | 0.972 |  | 915.348 | 5 | 5 |  |
| 19 |  | 1.973 |  |  | 0.991 |  | 0.903 |  | 916.251 | 4 | 5 |  |
| 20 |  | 2.061 |  |  | 0.828 |  | 1.145 |  | 917.396 | 4 | 4 |  |
| 21 |  | 2.420 |  |  | 1.104 |  | 0.957 |  | 918.353 | 4 | 4 |  |
| 22 |  | 1.835 |  |  | 1.205 |  | 1.215 |  | 919.568 | 4 | 4 |  |
| 23 |  | 2.361 |  |  | 1.022 |  | 0.813 |  | 920.381 | 11 | 4 |  |
| 24 |  | 1.399 |  |  | 1.322 |  | 1.039 |  | 921.42 | 10 | 11 |  |
| 25 |  | 1.528 |  |  | 1.470 |  |  | 0.071 | 921.349 | 12 | 10 |  |
| 26 |  | 1.987 |  |  | 0.825 |  | 0.703 |  | 922.052 | 7 | 12 |  |
| 27 |  | 2.265 |  |  | 1.023 |  | 0.964 |  | 923.016 | 11 | 7 |  |
| 28 |  | 1.055 |  |  | 0.551 |  | 1.714 |  | 924.73 | 10 | 11 |  |
| 29 |  | 0.978 |  |  | 1.534 |  |  | 0.479 | 924.251 | 5 | 10 |  |
| 30 |  | 0.645 |  |  | 2.103 |  |  | 1.125 | 923.126 | 4 | 5 |  |
| 31 |  | 0.988 |  |  | 2.033 |  |  | 1.388 | 921.538 | 2 | 4 |  |
| 32 |  |  |  |  | 1.393 |  |  | 0.405 | 921.333 |  | 2 |  |
|  |  | ∑ = 52.46 |  |  | ∑ = 36.767 |  | ∑ = 20.814 | ∑ = 5.121 |  |  |  |  |

Arithmetic Check:                                                             Total Loop Distance= 500+500           ∑BS-∑FS=∑Rise-∑Fall=Last RL-First RL                          Permissible error=25√k =25mm      52.46-36.767= 20.814- 5.121=921.333- 905.640                     Observed error = 13 mm

15.593=15.593=15.593

BRIDGE SITE SURVEY

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                         Date:2080/12/23

Recorder:Group3                                                          Time:1:30pm

Instrument: Thedolite,Staff and Ranging Rod

Table:Reciprocal Levelling

| Reading at | Staff Reading |  |  | Level Difference |
| --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom |  |
| A | 0.81 | 0.620 | 0.43 | 1.186-0.620 = 0.566 |
| B | 1.194 | 1.186 | 1.178 |  |

| Reading at | Staff Reading |  |  | Level Difference |
| --- | --- | --- | --- | --- |
|  | Top | Middle | Bottom |  |
| A | 1.309 | 1.293 | 1.275 | 1.845-1.293 = 0.552 |
| B | 2.040 | 1.845 | 1.650 |  |

True Level Difference between the points=(0.552+0.566)/2=0.559

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                           Date:2080/12/23

Recorder:Group3                                                        Time:1:30pm

Instrument: Thedolite,Staff and Ranging Rod

Table:Bridge Survey (Computation of Triangulation)

C                                 B

A                      D

25m

| Station |  | Face | Horizontal Angle Reading |  |  |  |  |  | Mean |  |  | Horizontal Angle |  |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal Circle Reading |  |  | Angle |  |  |  |  |  |  |  |  |  |
| From | To |  | D | M | S | D | M | S | D | M | S | D | M | S |  |
| A | B | L | 0 | 0 | 0 | 92 | 00 | 20 | 92 | 00 | 20.5 | 92 | 00 | 17.5 |  |
|  |  | R | 180 | 0 | 04 |  |  |  |  |  |  |  |  |  |  |
|  | D | L | 92 | 00 | 20 | 92 | 00 | 21 |  |  |  |  |  |  |  |
|  |  | R | 272 | 00 | 25 |  |  |  |  |  |  |  |  |  |  |
| A | B | L | 90 | 00 | 00 | 92 | 00 | 15 | 92 | 00 | 14.5 |  |  |  |  |
|  |  | R | 270 | 00 | 04 |  |  |  |  |  |  |  |  |  |  |
|  | D | L | 182 | 00 | 15 | 92 | 00 | 14 |  |  |  |  |  |  |  |
|  |  | R | 02 | 00 | 18 |  |  |  |  |  |  |  |  |  |  |
| D | A | L | 0 | 00 | 0 | 53 | 40 | 15 | 53 | 40 | 17 | 53 | 40 | 14.75 |  |
|  |  | R | 179 | 59 | 53 |  |  |  |  |  |  |  |  |  |  |
|  | B | L | 53 | 40 | 15 | 53 | 40 | 19 |  |  |  |  |  |  |  |
|  |  | R | 233 | 40 | 12 |  |  |  |  |  |  |  |  |  |  |
| D | A | L | 90 | 0 | 0 | 53 | 40 | 10 | 53 | 40 | 12.5 |  |  |  |  |
|  |  | R | 270 | 0 | 02 |  |  |  |  |  |  |  |  |  |  |
|  | B | L | 143 | 40 | 10 | 53 | 40 | 15 |  |  |  |  |  |  |  |
|  |  | R | 323 | 40 | 17 |  |  |  |  |  |  |  |  |  |  |
| B | D | L | 0 | 0 | 0 | 34 | 20 | 25 | 34 | 20 | 22.5 | 34 | 20 | 19.5 |  |
|  |  | R | 180 | 0 | 07 |  |  |  |  |  |  |  |  |  |  |
|  | A | L | 34 | 20 | 25 | 34 | 20 | 20 |  |  |  |  |  |  |  |
|  |  | R | 214 | 20 | 27 |  |  |  |  |  |  |  |  |  |  |
| B | D | L | 90 | 0 | 0 | 34 | 20 | 15 | 34 | 20 | 16.5 |  |  |  |  |
|  |  | R | 269 | 59 | 54 |  |  |  |  |  |  |  |  |  |  |
|  | A | L | 124 | 20 | 15 | 34 | 20 | 18 |  |  |  |  |  |  |  |
|  |  | R | 304 | 20 | 12 |  |  |  |  |  |  |  |  |  |  |
| C | B | L | 0 | 0 | 0 | 50 | 45 | 22 | 50 | 45 | 20 | 50 | 45 | 21.75 |  |
|  |  | R | 179 | 59 | 52 |  |  |  |  |  |  |  |  |  |  |
|  | A | L | 50 | 45 | 22 | 50 | 45 | 18 |  |  |  |  |  |  |  |
|  |  | R | 230 | 45 | 10 |  |  |  |  |  |  |  |  |  |  |
| C | B | L | 90 | 0 | 0 | 50 | 45 | 25 | 50 | 45 | 22.5 |  |  |  |  |
|  |  | R | 269 | 59 | 57 |  |  |  |  |  |  |  |  |  |  |
|  | A | L | 130 | 45 | 25 | 50 | 45 | 20 |  |  |  |  |  |  |  |
|  |  | R | 320 | 45 | 17 |  |  |  |  |  |  |  |  |  |  |
| A | C | L | 0 | 0 | 0 | 32 | 51 | 15 | 32 | 51 | 16.5 | 32 | 51 | 12.75 |  |
|  |  | R | 179 | 59 | 57 |  |  |  |  |  |  |  |  |  |  |
|  | B | L | 32 | 51 | 12 | 32 | 51 | 18 |  |  |  |  |  |  |  |
|  |  | R | 212 | 51 | 15 |  |  |  |  |  |  |  |  |  |  |
| A | C | L | 90 | 0 | 0 | 32 | 51 | 10 | 32 | 51 | 09 |  |  |  |  |
|  |  | R | 270 | 0 | 05 |  |  |  |  |  |  |  |  |  |  |
|  | B | L | 122 | 51 | 10 | 32 | 51 | 08 |  |  |  |  |  |  |  |
|  |  | R | 302 | 51 | 13 |  |  |  |  |  |  |  |  |  |  |
| B | A | L | 0 | 0 | 0 | 96 | 23 | 33 | 96 | 23 | 31.5 | 96 | 23 | 30.5 |  |
|  |  | R | 179 | 59 | 54 |  |  |  |  |  |  |  |  |  |  |
|  | C | L | 96 | 23 | 33 | 96 | 23 | 30 |  |  |  |  |  |  |  |
|  |  | R | 276 | 23 | 24 |  |  |  |  |  |  |  |  |  |  |
| B | A | L | 90 | 0 | 0 | 96 | 23 | 27 | 96 | 23 | 29.5 |  |  |  |  |
|  |  | R | 270 | 0 | 03 |  |  |  |  |  |  |  |  |  |  |
|  | C | L | 186 | 23 | 27 | 96 | 23 | 32 |  |  |  |  |  |  |  |
|  |  | R | 06 | 23 | 35 |  |  |  |  |  |  |  |  |  |  |

Length of the Bridge Calculated using both the traingales ABD and ABC are found to be 35.71m and 35.69m, which are at the same so the furture work can be processed.

Tribhuvan University

Institute of Engineering

Paschimanchal Campus, Pokhara

BGE Survey Camp - 2080

Observer:Group3                                                          Date:2080/12/23

Recorder:Group3                                                          Time:1:30pm

Instrument: Thedolite,Staff and Ranging Rod

Table:Bridge Detailing

Downstream of river

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(ѳ) | Distance D=KScos^2(ѳ) | Vertical(V)=Dtan(ѳ) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |  |
|  | A | 210°51’11’’ | 0.765 | 0.582 | 0.400 | 90°14’44’’ | -0°14’44’’ | 36.499 | -0.156 | 939.906 | A |
|  | 1 |  | 2.578 | 2.565 | 2.553 | 104°41’40’’ | -14°41’40’’ | 2.339 | -0.613 | 937.466 | HFL(R) |
|  | 2 |  | 2.964 | 2.918 | 2.871 | 108°13’11’’ | -18°13’11’’ | 8.390 | -2.762 | 937.964 | GW(R) |
|  | 3 |  | 3.930 | 3.880 | 3.832 | 108°11’53’’ | -18°11’53’’ | 8.844 | -2.907 | 933.857 | Normal level(R) |
|  | 4 |  | 1.681 | 1.592 | 1.505 | 109°17’47’’ | -19°17’47’’ | 15.678 | -5.489 | 933.563 | Bed level |
|  | 5 |  | 1.498 | 1.375 | 1.252 | 102°11’47’’ | -12°11’47’’ | 23.502 | -5.079 | 934.189 | Normal level(L) |
|  | 6 |  | 1.110 | 0.951 | 0.820 | 95°26’44’’ | -5°26’44’’ | 28.738 | -2.739 | 936.953 | HFL(L) |
|  | 7 |  | 0.856 | 0.658 | 0.400 | 90°14’50’’ | -0°14’50’’ | 45.499 | -0.196 | 939.79 | Road end(L) |
|  | 8 | 30°50’40’’ | 1.475 | 1.384 | 1.293 | 89°18’02’’ | 0°41’58’’ | 18.197 | -0.222 | 939.482 | Mid field(R) |
|  | 9 |  | 1.205 | 1.051 | 0.901 | 89°17’46’’ | 0°42’14’’ | 30.395 | -0.373 | 939.966 | End field(R) |

For 20m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical Angle(ѳ) | Distance(D)=KScos^2(ѳ) | Vertical(V)=Dtan(ѳ) | RL=RL of station +HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| B | a | 187°26’33’’ | 0.605 | 0.380 | 0.155 | 90°27’08’’ | -0°27’08’’ | 44.697 | -0.352 | 939.911 | Road end |
|  | b | 183°06’25’’ | 0.721 | 0.521 | 0.321 | 90°27’10’’ | -0°27’10’’ | 40.097 | -0.317 | 939.806 | Road edge |
|  | c | 181°39’08’’ | 1.410 | 1.215 | 1.020 | 91°53’52’’ | -1°°53’52’’ | 38.957 | -1.291 | 938.138 | HFL(L) |
|  | d | 178°22’24’’ | 1.460 | 1.280 | 1.095 | 96°08’14’’ | -6°08’14’’ | 36.083 | -3.870 | 935.484 | GW |
|  | e | 166°58’05’’ | 1.275 | 1.130 | 0.990 | 101°44’08’’ | -11°44’08’’ | 27.321 | -5.675 | 933.838 | NFL(L) |
|  | f | 162°26’45’’ | 0.990 | 0.851 | 0.720 | 103°20’49’’ | -13°20’49’’ | 25.561 | -6.065 | 933.728 | Bed level |
|  | g | 158°39’44’’ | 0.910 | 0.771 | 0.640 | 103°20’57’’ | -13°20’57’’ | 25.561 | -6.065 | 933.807 | NFL(R) |
|  | h | 134°32’15’’ | 1.465 | 1.358 | 1.250 | 103°21’14’’ | -13°21’14’’ | 20.353 | -4.831 | 934.455 | GW(R) |
|  | i | 126°20’30’’ | 1.486 | 1.395 | 1.295 | 96°49’54’’ | -6°49’54’’ | 18.829 | -2.256 | 936.993 | HFL(R) |
|  | j | 98°56’07’’ | 0.730 | 0.620 | 0.512 | 96°50’58’’ | -6°50’58’’ | 21.489 | -2.581 | 937.463 | Field edge |
|  | k | 63°24’09’’ | 1.060 | 0.905 | 0.755 | 90°51’14’’ | -0°51’14’’ | 30.493 | -0.454 | 939.284 | Mid Field(R) |
|  | l | 57°45’03’’ | 0.901 | 0.725 | 0.551 | 90°50’06’’ | -0°50’06’’ | 34.993 | -0.510 | 939.409 | Field end(R) |

For 40m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical Angle(t) | Distance(D)=KScos^2(t) | Vertical(V)=Dtan(t) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
|  | a | 166°09’18’’ | 1.920 | 1.630 | 1.340 | 89°11’59’’ | 0°48’01’’ | 57.988 | 0.810 | 939.834 | Road end |
| B | b | 162°00’07’’ | 1.975 | 1.705 | 1.495 | 89°11’53’’ | 0°48’07’’ | 47.990 | 0.671 | 939.610 | Road edge |
|  | c | 159°27’38’’ | 1.965 | 1.695 | 1.430 | 93°12’53’’ | -3°12’53’’ | 53.330 | -2.995 | 935.953 | HFL(L) |
|  | d | 152°58’00’’ | 1.300 | 1.060 | 0.815 | 97°01’48’’ | -7°01’48’’ | 47.770 | -5.891 | 933.692 | NFL(L) |
|  | e | 146°38’39’’ | 1.180 | 0.953 | 0.725 | 97°32’19’’ | -7°32’19’’ | 44.716 | -5.917 | 933.771 | Bed level |
|  | f | 142°09’12’’ | 1.860 | 1.633 | 1.405 | 96°45’34’’ | -6°45’34’’ | 44.869 | -5.318 | 933.693 | NFL(L) |
|  | g | 127°24’07’’ | 3.920 | 3.720 | 3.520 | 93°35’04’’ | -3°35’04’’ | 39.843 | -2.495 | 934.428 | GW(R) |
|  | h | 124°11’05’’ | 2.130 | 1.930 | 1.730 | 98°08’30’’ | -8°08’30’’ | 39.197 | -5.607 | 935.106 | HFL(R) |
|  | i | 100°17’00’’ | 1.225 | 1.013 | 0.800 | 93°11’10’’ | -3°11’10’’ | 42.360 | -2.358 | 937.370 | Field edge |
|  | j | 95°29’19’’ | 1.230 | 1.014 | 0.800 | 90°04’16’’ | -0°04’16’’ | 42.999 | -0.053 | 939.581 | RMF |
|  | k | 76°58’55’’ | 1.330 | 1.065 | 0.800 | 89°21’17’’ | 0°38’17’’ | 52.999 | 0.134 | 939.710 | Field end |
|  | l | 131°52’17’’ | 0.935 | 0.780 | 0.620 | 98°23’41’’ | -8°23’41’’ | 30.828 | 4.549 | 944.413 | Rock |
|  | m | 183°23’19’’ | 1.885 | 1.705 | 1.520 | 91°32’10’’ | -1°32’10’’ | 36.473 | -0.978 | 937.960 | Tree(3) |
|  | n | 175°42’03’’ | 1.585 | 1.395 | 1.205 | 92°27’59’’ | -2°27’59’’ | 37.929 | -1.633 | 937.615 | Tree(3) |

For 60m chainage

| Station | Sighted to | HCR | Top | Middle | Bottom | VCR | Vertical angle(t) | Distance(D)=KScos^2(t) | Vertical(V)=Dtan(t) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | a | 41°15’16’’ | 1.200 | 1.100 | 1.000 | 87°33’09’’ | 2°26’51’’ | 19.960 | 0.853 | 937.613 | Mid field(R) |
|  | b | 48°25’49’’ | 1.300 | 1.257 | 1.215 | 89°53’02’’ | 0°6’58’’ | 8.499 | 0.017 | 936.618 | GW(R) |
|  | c | 192°36’47’’ | 1.625 | 1.595 | 1.565 | 100°55’13’’ | -10°55’13’’ | 5.785 | -1.116 | 935.148 | HFL(R) |
|  | d | 196°10’47’’ | 0.965 | 0.945 | 0.925 | 101°45’40’’ | -11°45’40’’ | 3.834 | -0.798 | 934.117 | Normal level(R) |
|  | e | 206°30’20’’ | 0.900 | 0.800 | 0.700 | 100°31’16’’ | -10°31’16’’ | 19.330 | -3.591 | 933.469 | Bed level |
|  | f | 207°39’19’’ | 0.820 | 0.700 | 0.580 | 99°20’42’’ | -9°20’42’’ | 23.367 | -3.845 | 934.314 | Normal level(L) |
|  | g | 207°44’46’’ | 1.735 | 1.600 | 1.465 | 95°53’08’’ | -5°53’08’’ | 28.716 | -2.754 | 935.505 | HFL(L) |
|  | h | 208°20’59’’ | 1.495 | 1.325 | 1.155 | 88°17’46’’ | 1°42’14’’ | 33.470 | 0.995 | 937.530 | Tree(2) |
|  | i | 206°58’38’’ | 1.660 | 1.490 | 1.320 | 88°12’46’’ | 1°47’14’’ | 33.960 | 1.059 | 937.429 | Road edge(L) |
|  | j | 208°15’18’’ | 1.035 | 0.845 | 0.660 | 85°51’14’’ | 4°48’46’’ | 37.304 | 2.704 | 939.719 | Road end |
|  | k | 208°18’15’’ | 1.84 | 1.62 | 1.40 | 85°36’25’’ | 4°23’35’’ | 43.742 | 3.360 | 939.600 | WALL |

For 80m Chainage

| Station | Sighted to | HCR | Top | Middle | Bottom | VCR | Vertical angle(t) | Distance(D)=KScos^2(t) | Vertical(V)=Dtan(t) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | 1’ | 78°40’47’’ | 1.045 | 0.890 | 0.735 | 87°51’18’’ | 2°08’42’’ | 30.956 | 1.159 | 938.127 | Field end |
|  | 2’ | 101°40’23’’ | 1.312 | 1.000 | 0.882 | 90°58’24’’ | -0°58’24’’ | 22.993 | -0.391 | 936.469 | Field junction |
|  | 3’ | 120 °30’54’’ | 1.106 | 1.000 | 0.894 | 91°05’50’’ | -1°05’50’’ | 25.591 | -0.490 | 936.370 | Mid field |
|  | 4’ | 132°29’28’’ | 1.501 | 1.400 | 1.295 | 90°42’39’’ | -0°42’39’’ | 20.597 | -0.255 | 936.205 | HFL(R) |
|  | 5’ | 136°28’37’’ | 2.045 | 1.946 | 1.838 | 91°44’17’’ | -1°44’17’’ | 20.681 | -0.627 | 935.292 | GW |
|  | 6’ | 147°57’ 48’’ | 1.960 | 1.850 | 1.740 | 97°07’12’’ | -7°07’12’’ | 21.662 | -2.701 | 933.309 | Big rock |
|  | 7’ | 163°05’18’’ | 1.510 | 1.383 | 1.254 | 97°06’53’’ | -7°06’53’’ | 25.207 | -3.146 | 933.331 | Normal level(R) |
|  | 8’ | 171°21’59’’ | 1.917 | 1.767 | 1.620 | 95°44’54’’ | -5°44’54’’ | 28.907 | -2.909 | 933.184 | Bed level |
|  | 9’ | 176°46’39’’ | 1.864 | 1.704 | 1.543 | 94°52’32’’ | -4°52’32’’ | 31.868 | -2.718 | 933.438 | Normal level(L) |
|  | 10’ | 184°52’47’’ | 1.740 | 1.549 | 1.356 | 89°50’56’’ | 0°09’04’’ | 38.399 | 0.101 | 936.412 | HFL |
|  | 11’ | 186°28’39’’ | 1.907 | 1.688 | 1.474 | 85°58’36’’ | 4°01’24’’ | 43.087 | 3.031 | 939.203 | Road edge |
|  | 12’ | 189°34’47’’ | 1.764 | 1.513 | 1.261 | 86°36’03’’ | 3°23’57’’ | 50.123 | 2.977 | 939.324 | Road end |

For 100m chainage

| Station | Sighted to | HCR | Top | Middle | Bottom | VCR | Vertical angle(t) | Distance(D)=KScos^2(t) | Vertical(V)=Dtan(t) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | 1’’ | 100°40’43’’ | 1.770 | 1.534 | 1.298 | 88°07’29’’ | 1°52’31’’ | 46.849 | 1.534 | 937.86 | Field end(R) |
|  | 2’’ | 121°02’28’’ | 1.638 | 1.443 | 1.248 | 89°56’53’’ | 0°03’07’’ | 39.199 | 0.035 | 936.452 | Field junction |
|  | 3’’ | 136°49’20’’ | 1.957 | 1.763 | 1.569 | 89°26’49’’ | 0°33’11’’ | 38.796 | 0.374 | 936.472 | Field edge |
|  | 4’’ | 139°11’53’’ | 1.870 | 1.678 | 1.486 | 90°09’09’’ | -0°09’09’’ | 38.399 | -0.102 | 936.080 | HFL(R) |
|  | 5’’ | 142°56’34’’ | 1.859 | 1.670 | 1.481 | 91°36’16’’ | -1°36’16’’ | 37.870 | -1.061 | 934.939 | GW(R) |
|  | 6’’ | 148°49’58’’ | 2.084 | 1.887 | 1.692 | 94°19’28’’ | -4°19’28’’ | 38.977 | -2.947 | 933.026 | Normal level(R) |
|  | 7’’ | 151°46’49’’ | 1.022 | 0.822 | 0.622 | 96°30’16’’ | -6°30’16’’ | 39.388 | -4.490 | 932.548 | Bed level |
|  | 8’’ | 164°23’34’’ | 1.058 | 0.849 | 0.640 | 95°16’47’’ | -5°16’47’’ | 41.446 | -3.830 | 933.181 | Normal level(L) |
|  | 9’’ | 187°29’01’’ | 1.695 | 1.468 | 1.240 | 90°39’28’’ | -0°39’28’’ | 45.494 | -0.522 | 935.870 | HFL(L) |
|  | 10’’ | 188°11’00’’ | 0.980 | 0.745 | 0.510 | 87°27’07’’ | 2°32’53’’ | 46.907 | 2.087 | 939.202 | Road edge |
|  | 11’’ | 194°42’00’’ | 1.330 | 1.065 | 0.800 | 87°11’08’’ | 2°48’52’’ | 52.872 | 2.600 | 939.395 | Road end |
|  | B | 288°24’09’’ | 1.925 | 1.600 | 1.285 | 87°12’32’’ | 2°47’28’’ | 63.848 | 3.113 | 939.373 | B |

TACHEOMETRIC DETAILING SHEET

UPSTREAM OF RIVER

For 20m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(t) | Distance(D)=KScos^2(t) | Vertical(V)=Dtan(t) | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  | 939.883 |  |
|  | 1 | 286°38’32’’ | 0.810 | 0.717 | 0.624 | 90°30’34’’ | -0°30’34’’ | 18.599 | -0.165 | 940.481 | Road End |
|  | 2 | 312°42’48’’ | 0.869 | 0.769 | 0.669 | 90°29’14’’ | -0°29’14’’ | 19.999 | -0.170 | 940.424 | Road Edge |
|  | 3 | 324°15’17’’ | 1.625 | 1.525 | 1.425 | 94°28’20’’ | -4°28’20’’ | 19.878 | -1.555 | 938.283 | HFL(L) |
|  | 4 | 348°34’54’’ | 2.225 | 2.094 | 1.966 | 101°21’34’’ | -11°21’34’’ | 24.895 | -5.001 | 934.268 | Normal Level(L) |
|  | 5 | 352°55’32’’ | 1.861 | 1.715 | 1.565 | 100°54’31’’ | -10°54’31’’ | 28.539 | -5.500 | 934.148 | Bed |
|  | 6 | 0°06’29’’ | 1.136 | 0.960 | 0.785 | 100°19’22’’ | -10°19’22’’ | 33.972 | -6.188 | 934.215 | Normal Level(R) |
|  | 7 | 1°41’24’’ | 2.010 | 1.830 | 1.650 | 92°31’20’’ | -2°31’20’’ | 35.930 | -1.583 | 937.95 | GW |
|  | 8 | 2°25’27’’ | 1.315 | 1.131 | 0.949 | 92°31’19’’ | -2°31’19’’ | 36.529 | -1.608 | 938.624 | HFL(R) |
|  | 9 | 2°01’57’’ | 1.152 | 0.959 | 0.765 | 91°06’43’’ | -1°06’43’’ | 38.685 | -0.751 | 939.653 | Field Junction |
|  | 10 | 4°23’19’’ | 1.645 | 1.430 | 1.215 | 91°06’39’’ | -1°06’39’’ | 42.983 | -0.833 | 939.1 | Field Mid |
|  | 11 | 10°31’25’’ | 2.352 | 2.185 | 2.015 | 92°49’44’’ | -2°49’44’’ | 33.618 | -1.661 | 937.517 | Field End |
|  | B | 28°58’’14’’ | 1.087 | 0.908 | 0.730 | 91°45’01’’ | -1°45’01’’ | 35.666 | -1.089 | 939.366 | Station B |

For 40m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical Angle(ѳ) | Distance(D)=KScos^2(ѳ) | Vertical(V)=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S2 |  |  |  |  |  |  |  |  |  | 940.951 |  |
|  | 1 | 231°39’58’’ | 1.684 | 1.514 | 1.344 | 87°11’8’’ | 2°48’52’’ | 33.918 | 1.667 | 942.544 | Abutment |
|  | 2 | 233°12’04’’ | 0.770 | 0.620 | 0.470 | 87°11’3’’ | 2°48’57’’ | 29.928 | 1.471 | 943.242 | Big stone near suspension bridge |
|  | 3 | 225°27’46’’ | 0.884 | 0.734 | 0.585 | 101°10’42’’ | -11°10’42’’ | 28.776 | -5.687 | 935.97 | HFL(L) |
|  | 4 | 227°00’30’’ | 2.815 | 2.680 | 2.546 | 101°9’51’’ | -11°9’51’’ | 25.891 | -5.109 | 934.626 | Normal(L) |
|  | 5 | 230°00’27’’ | 2.890 | 2.775 | 2.664 | 103°40’38’’ | -13°40’38’’ | 21.336 | -5.192 | 934.424 | BED |
|  | 6 | 232°44’02’’ | 1.826 | 1.735 | 1.642 | 110°47’33’’ | -20°47’33’’ | 16.081 | -6.106 | 934.55 | Normal(R ) |
|  | 7 | 246°35’52’’ | 1.485 | 1.441 | 1.398 | 113°11’42’’ | -23°11’42’’ | 7.350 | -3.149 | 937.801 | GW |
|  | 8 | 239°23’46’’ | 1.684 | 1.645 | 1.610 | 113°11’42’’ | -23°11’42’’ | 7.350 | -3.149 | 937.597 | HFL(R) |
|  | 9 | 217°52’41’’ | 1.115 | 1.100 | 1.085 | 106°7’34’’ | -16°07’34’’ | 2.768 | -0.800 | 940.491 | Wing Wall |
|  | B | 139°56’59’’ | 1.401 | 1.188 | 0.974 | 92°7’59’’ | -2°07’59’’ | 42.640 | -1.588 | 939.615 | Station B |
|  | 11 | 352°24’16’’ | 0.925 | 0.892 | 0.859 | 92°8’04’’ | -2°08’04’’ | 6.590 | -0.246 | 941.253 | Abutment |

For 60m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical Angle(ѳ) | Distance(D)=KScos^2(ѳ) | Vertical(V)=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S2 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 324°34’30’’ | 1.110 | 0.994 | 0.876 | 83°13’18’’ | 6°46’42’’ | 23.074 | -2.743 | 938.654 | Rock(R) |
|  | 2 | 298°03’09’’ | 1.881 | 1.765 | 1.650 | 97°02’31’’ | -7°02’31’’ | 22.753 | -2.810 | 937.816 | HFL |
|  | 3 | 277°42’29’’ | 2.150 | 2.001 | 1.862 | 101°14’27’’ | -11°14’27’’ | 27.706 | -5.506 | 934.884 | Normal (R) |
|  | 4 | 270°57’38’’ | 0.844 | 0.681 | 0.521 | 102°43’08’’ | -12°43’08’’ | 30.734 | -6.937 | 934.773 | BED |
|  | 5 | 266°39’30’’ | 1.045 | 0.861 | 0.680 | 100°35’18’’ | -10°35’18’’ | 35.268 | -6.592 | 934.938 | Normal(L) |
|  | 6 | 264°29’00’’ | 1.715 | 1.500 | 1.286 | 95°54’12’’ | -5°54’12’’ | 42.446 | -4.388 | 936.503 | HFL(L) |
|  | 7 |  |  |  |  |  |  |  |  |  | CLIFF(Jungle) |

For 80m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical Angle(ѳ) | Distance(D)=KScos^2(ѳ) | Vertical(V)=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S2 | 1 | 314°5845 | 1.289 | 1.044 | 0.631 | 91°31’50’’ | -1°31’50’’ | 65.753 | -1.757 | 939.59 | HFL(L) |
|  | 2 | 306°040 | 2.750 | 2.530 | 2.309 | 95°56’48’’ | -5°56’48’’ | 43.626 | -4.544 | 935.317 | Normal level(L) |
|  | 3 | 301°3730 | 2.331 | 2.106 | 1.879 | 96°35’00’’ | -6°35’00’’ | 44.606 | -5.148 | 935.137 | Bed level |
|  | 4 | 298°4215 | 1.742 | 1.515 | 1.289 | 96°59’41’’ | -6°59’41’’ | 44.628 | -5.475 | 935.401 | Normal level(R) |
|  | 5 | 285°150 | 1.985 | 1.710 | 1.430 | 94°06’54’’ | -4°06’54’’ | 55.214 | -3.972 | 936.709 | HFL(R) |

For 100m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(ѳ) | Distance(D)=KScos^2(ѳ) | Vertical(V)=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S3 |  |  |  |  |  |  |  |  |  | 940.085 |  |
|  | S2 | 132°19’45’’ | 1.040 | 0.830 | 0.630 | 89°24’18’’ | 0°35’42’’ | 40.996 | 0.426 | 940.951 | Temporary Station 2 |
|  | 1 | 324°18’51’’ | 1.684 | 1.290 | 0.896 | 94°06’55’’ | -4°06’55’’ | 78.394 | -5.640 | 935.425 | HFL(R) |
|  | 2 | 301°16’13’’ | 2.580 | 2.505 | 2.441 | 105°48’47’’ | -15°48’47’’ | 12.868 | -3.644 | 935.206 | Normal level(R) |
|  | 3 | 282°45’09’’ | 1.331 | 1.258 | 1.179 | 109°19’11’’ | -19°19’11’’ | 13.536 | -4.745 | 935.352 | Bed level |
|  | 4 | 276°17’21’’ | 0.405 | 0.316 | 0.225 | 109°19’10’’ | -19°19’10’’ | 16.030 | -5.620 | 935.419 | Normal level(L) |
|  | 5 | 262°36’21’’ | 0.671 | 0.615 | 0.469 | 97°49’09’’ | -7°49’09’’ | 19.826 | -2.722 | 938.018 | HFL |

For 120m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(ѳ) | Distance D=KScos^2(ѳ) | Vertical H V=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S3 | 1 | 325°05’11’’ | 1.665 | 1.528 | 1.390 | 95°14’04’’ | -5°14’04’’ | 27.271 | -2.498 | 937.192 | HFL(R) |
|  | 2 | 316°10’35’’ | 0.960 | 0.822 | 0.685 | 98°29’11’’ | -8°29’11’’ | 26.901 | -4.014 | 936.381 | Rock |
|  | 3 | 316°17’42’’ | 0.825 | 0.680 | 0.535 | 100°46’48’’ | -10°46’48’’ | 27.985 | -5.328 | 935.202 | Normal level(R) |
|  | 4 | 308°29’39’’ | 0.992 | 0.841 | 0.690 | 101°05’55’’ | -11°05’55’’ | 29.080 | -5.704 | 934.659 | Bed level |
|  | 5 | 303°49’32’’ | 1.760 | 1.602 | 1.449 | 98°10’02’’ | -8°10’02’’ | 30.472 | -4.373 | 935.222 | Normal level(L) |
|  | 6 | 285°22’05’’ | 0.791 | 0.605 | 0.420 | 96°08’44’’ | -6°08’44’’ | 36.674 | -3.949 | 936.615 | HFL(L) |

For 140m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(ѳ) | Distance D=KScos^2(ѳ) | Vertical H V=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S3 | 1 | 324°10’50’’ | 1.040 | 0.790 | 0.550 | 95°43’21’’ | -5°43’21’’ | 48.513 | -4.861 | 935.704 | Normal level(R) |
|  | 2 | 321°50’27’’ | 1.905 | 1.605 | 1.305 | 94°41’20’’ | -4°41’20’’ | 59.599 | -4.888 | 934.862 | Bed level |
|  | 3 | 319°24’43’’ | 1.000 | 0.750 | 0.501 | 95°28’38’’ | -5°28’38’’ | 49.445 | -4.741 | 935.864 | Normal(L) |
|  | 4 | 300°30’11’’ | 1.250 | 0.880 | 0.620 | 93°31’00’’ | -3°31’00’’ | 62.763 | -3.857 | 936.618 | HFL(L) |

For 150m chainage

| Station | Sighted to | HCR | Stadia Reading |  |  | VCR | Vertical angle(ѳ) | Distance D=KScos^2(ѳ) | Vertical H V=Dtanѳ | RL=RL of station+HI+-V-M | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Top | Middle | Bottom |  |  |  |  |  |  |
| S3 | 1 | 316°18’52’’ | 2.170 | 1.850 | 1.510 | 92°46’57’’ | -2°46’57’’ | 65.844 | -3.200 | 936.305 | Bed level |
|  | 2 | 313°13’35’’ | 2.190 | 1.870 | 1.550 | 92°42’01’’ | -2°42’01’’ | 63.858 | -3.012 | 936.473 | Normal level(L) |
|  | 3 | 301°28’’23’’ | 1.030 | 0.700 | 0.735 | 92°41’37’’ | -2°41’37’’ | 29.435 | -1.385 | 939.27 | HFL(L) |